3GPP TSG-WG2 Meeting #165	S2-2410000

# Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1241 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Relative Proximity Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Relative Proximity Analyitcs is used in the UAV triggered and USS triggered Network-assisted DAA in TS 23.256. For the Relative proximity predictions, under the Time To Collision, two sub-predictions can be generated, i.e., Collision shape and Collision direction. Collision shape means predicted shape of the collision area at the collision time. Collision direction means predicted collision direction of the target UE at the collision time. This paper proposes to add the two sub-predictions as outputs under the Time To Collision of the Relative proximity predictions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added sub-predictions under Time To Collision for Collision shape and Collision direction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The outputs of the Relative Proximity Analytics is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 6.19.3	Output analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are used to expose the analytics.

-	Relative proximity statistics information is defined in Table 6.19.3-1.

-	Relative proximity predictions information is defined in Table 6.19.3-2.

Table 6.19.3-1: Relative proximity statistics

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies the UE(s) for which the statistics apply by a list of SUPIs or GPSIs, or a group of UEs by a list of Internal-Group-Ids. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributes | Observed proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in clause 6.19.1 |
| >> relative proximity information | Observed proximity information. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.19.3-2: Relative proximity predictions

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies the UE(s) for which the statistics apply by a list of SUPIs or GPSIs, or a group of UEs by a list of Internal-Group-Ids. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributes | Predicted proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in clause 6.19.1. |
| >> relative proximity information | Predicted proximity information. |
| >> Confidence | Confidence of this prediction. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| Time To Collision (TTC) information (NOTE 1) |  |
| > Time To Collision | Time until a collision with another UE happens. |
| >> Collision shape | Predicted shape of the collision area at the collision time, e.g. ellipsoid point as mentioned in TS 23.032 [34]. |
| >> Collision direction | Predicted collision direction of the target UE at the collision time. |
| > Confidence | Confidence of the prediction for TTC, and collision direction. |
| > Accuracy | Accuracy of TTC, and collision direction (dependent on both the UE location accuracy and confidence of the prediction). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

>>>> End of Changes <<<<