3GPP TSG-WG2 Meeting #165	S2-2410003

14th – 18th October 2024, Hyderabad, India	revision of S2-2407492

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1097 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS and policy assistance information analytic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As it has been agreed in TR 23.700-84 for FS_AIML_CN KI#3, based on PCF request, NWDAF may provide assistance information analytics to PCF to assist the QoS determination and/or modification of QoS parameters. This CR proposes to specify a new QoS and policy assistance information Analytics ID, according to the conclusions in the TR 23.700-84 for KI#3. The PCF may send an analytics request or subscripton for QoS and policy assistance information analytics Id to the NWDAF requesting to provide the QoE and optionally the NW performance and QoS sustainability associated to one or a list of candidate QoS parameter set(s). Then the PCF may take into account the information received for every candidate QoS parameters set to determine the final QoS parameters for an application or service flow, based on operator’s policy. Table 6.x.2-1 is based on the one included in CR 23.288 CR1132r2 (SA2_164) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new QoS and Policy Assistance Analytics Id in NWDAF to assist PCF in the determination and/or modification of QoS parameters based on conclusions agreed in TR 23.700-84. In the output of the new analytic the NWDAF provides the expected QoE, and optionally NW performance and QoS sustainability associated to every candidate QoS parameters set provided by the PCF in the request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusions in TR 23.700-84 for KI#3 are not implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change -all new text-* * * *

## 6.x	QoS and policy assistance information analytic

### 6.x.1	General

This clause specifies how NWDAF can provide QoS and policy assistance analytics, in the form of statistics or predictions, to a service consumer.

The QoS and policy assistance information analytic provides statistics or predictions of the Service Experience for an Application or service flow associated to a set of QoS parameters. I.e. the Service Experience of the Application when the NW applies that set of QoS parameters on the corresponding QoS flows for the application. Additionally, the QoS and policy assistance information analytic may also provide the NW performance and QoS sustainability associated to the set of QoS parameters. Therefore, the QoS and policy assistance information analytic may provide one or more of the following outputs associated to a set of QoS parameters:

-	Service Experience: statistics or predictions of the service experience for the application or service flows;

-	QoS Sustainability: QoS change statistics or the likelihood of a QoS change in the future;

-	NW Performance: statistics or predictions on the NW performance (i.e., gNB status information, gNB resource usage, communication performance and mobility performance).

The service consumer may be e.g. PCF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "QoS and policy assistance";

-	Target of Analytics Reporting: for a UE or group of UEs or any UE in an Application.

-	Analytics Filter Information:

-	Candidate QoS parameter set(s): It may include just a single set of QoS parameters or a list of them. The QoS parameters set may include:

-	QoS parameters (as defined in clause 5.7.2 of TS 23.501) includes: 5QI (standardized or pre-configured) and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR);

-	the QoS Characteristics attributes (as defined in clause 5.7.3 of TS 23.501) including: Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume and their values;

- MBR;

-ARP;

-	Application ID;

-	Optionally Area of interest;

NOTE 1:	A service consumer can use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	Optionally S-NSSAI and NSI ID;

-	Optionally DNN;

-	optionally a list of analytics subsets that are requested, e.g. whether information about NW Performance and QoS sustainability is included (see clause 6.x.3);

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred granularity of location information: TA level or cell level or "longitude and latitude level";

-	Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	in a subscription, the Notification Correlation Id and the Notification Target Address; and

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.x.3).

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.x.3.

### 6.x.2	Input Data

The detailed information collected by the NWDAF is listed in Table 6.x.2.1.

Table 6.x.2-1: Input data from 5GC NF related to used QoS

| Information | Source | Description |
| --- | --- | --- |
| QoS Flows Information (1..max) | SMF | One or more list(s) of QoS flow information |
| > Used QoS parameter set(s) | SMF | The QoS parameter set(s) that have been already applied by SMF. As detailed in clause 6.x.1, the QoS parameter set includes QoS parameters and QoS characteristics attributes (as defined in TS 23.501 [2]). |
| > Used QoS profile | SMF | The QoS profile associated to the used QoS parameter set(s). |
| > Event type | SMF | The type of QFI change, i.e., QoS Flow establishment (i.e., QFI Change) or QoS Flow termination (i.e., QFI deallocation) or Traffic binding of QoS Flow event |
| > QFI | SMF | QoS Flow identifier |
| > Traffic descriptor | SMF | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| > PDU Session ID | SMF | PDU session ID containing the QoS Flow |
| UE identifier | SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Time stamp | SMF, AMF | The time stamp associated to the collected data. |

In addition, the NWDAF needs to collect data associated to the calculation of Application Observed Service Experience analytic as defined in section 6.4.2, QoS sustainability analytic as defined in clause 9.12 and NW performance analytic as defined in clause 6.6.2.

### 6.x.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are used to expose the QoS and policy assistance information analytic.

-	QoS and policy assistance information analytic statistics information is defined in Table 6.x.3-1.

-	QoS and policy assistance information analytic predictions information is defined in Table 6.x.3-3.

Table 6.x.3-1: QoS and policy assistance information statistics

| Information | Description |
| --- | --- |
| QoS Parameters set list (0..max) | Identifies the list of candidate QoS parameters sets for which assistance information is provided. The list of candidate QoS parameters set(s) are provided by the consumer and the information included for every candidate QoS parameters set is described in 6.x.1. Per every candidate QoS parameters set the NWDAF provides statistics about Application observed service experiences, and optionally QoS Sustainability and Network Performance information. |
| >Application observed service experiences (0..max) | List of application observed service experience information statistics per QoS Parameters set as defined in table 6.x.3-2. |
| >List of QoS sustainability Analytics (1..max) | List of QoS sustainability information statistics per QoS Parameters set as defined in table 6.9.3.1. |
| >List of network performance information (1..max) | List of Network Performance Information statistics per QoS Parameter set as defined in table 6.6.3-1. |

Table 6.x.3-2: Application Observed service Experience per QoS Parameter set

| Information | Description |
| --- | --- |
| Application Observed service experiences (0..max) | List of observed service experience information statistics for the Application. It is based on Observed Service Experience analytic when it applies to an Application, as described in Table 6.4.3.-1. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, or location coordinates, etc) when the UE service is delivered. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. |  |

Table 6.x.3-3: QoS and policy assistance information predictions

| Information | Description |
| --- | --- |
| QoS Parameters (0..max) | Identifies the list of candidate QoS parameters sets for which assistance information is provided. The list of candidate QoS parameters set(s) are provided by the consumer and the information included for every candidate QoS parameters set as described in 6.x.1. Per every candidate QoS parameters set the NWDAF provides predictions about Application Observed service experiences, and optionally QoS Sustainability and Network Performance information. |
| >Application observed service experiences (0..max) | List of application observed service experience information predictions per QoS Parameters set As defined in table 6.x.3-4 |
| >List of QoS sustainability Analytics (1..max) | List of QoS sustainability information predictions per QoS Parameters set as defined in table 6.9.3.2 |
| >List of network performance information (1..max) | List of Network Performance Information predictions per QoS Parameter set as defined in table 6.6.3-2 |

Table 6.x.3-4: Observed service Experience per QoS Parameter set predictions

| Information | Description |
| --- | --- |
| Application service experiences (0..max) | List of observed service experience information for each Application. It is based on Observed Service Experience analytic when it applies to an Application, as described in Table 6.4.3.-2 |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, or location coordinates, etc) when the UE service is delivered. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. |  |

### 6.x.4	Procedures to request QoS and policy assistance information for an Application

Figure 6.x.4-1: Procedure for NWDAF providing QoS and policy assistance information for an Application

This procedure allows the consumer to request Analytics ID "QoS and policy assistance" for a single candidate QoS Parameter set or for a list of them and an Application. The consumer e.g. the PCF includes both the candidate QoS Parameters set(s) and the Application ID for which the asistance is requested and indicates that the Target of Analytics Reporting is "any UE". If the Target for Analytics Reporting is either a SUPI or an Internal-Group-Id the procedure in clause 6.x.5 applies. The candidate QoS parameters set(s) is determined by the consumer, based on e.g. configuration or operator policies in the consumer.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = QoS and policy assistance, Target of Analytics Reporting = any UE, Analytics Filter Information including one or a list of candidate QoS parameters set and the associated application, and optionally including one or more of the following as defined in Table 6.x.1-1 (e.g. S-NSSAI, DNN, Area of Interest, Analytics Reporting Information=Analytics target period, a list of analytics subsets) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.x by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID, Event Filter information, Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of trusted AF, NWDAF provides the Area of Interest as a list of TAIs to AF. In the case of untrusted AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into geographic zone identifier(s) that act as event filter for AF.

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

NOTE 1: When requested by the NF consumer (e.g. PCF) the NWDAF may also collect data from OAM as described in 6.6.4 and 6.9.4 for the purpose of derive NW performance and QoS sustainability information associated to the QoS parameters set(s).

2c. With these data, the NWDAF derives the requested analytics.

3.	The NWDAF provides the data analytics to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, including assistance information for every QoS parameter set provided in the request. The assistance information indicates per every candidate QoS parameters set and application the statistics/predictions of the service experience for the application, the information about QoS sustainability and the NW Performance information when the PCF applies the candidate QoS parameter set to the application.

NOTE 2:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

NOTE 3:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

4.	If the consumer NF is a PCF, it may use the analytic response as input to determine the QoS parameters to be applied, as defined in clause 6.1.1.3 and clause 6.2.1.2 of TS 23.503 [4].

5.	NWDAF collects new data and derives new value of the analytics, e.g. obtains new value for service experience for one or more of the candidates QoS parameters.

6.	When relevant according to the Analytics target period and Reporting Thresholds, the NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (Network Performance analytics, Subscription Correlation Id, Confidence) including the new value of the analytics.

7.	If the consumer NF is a PCF it may determine to modify the QoS parameters to be applied and provide the modified QoS towards the SMF based on the operator's policy, as defined in clause 6.1.1.3 and clause 6.2.1.2 of TS 23.503 [4].

* * * End of Changes * * * *