3GPP TSG-WG2 Meeting #165	S2-2410008

14th – 18th October 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1246 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – Update of VFL training and inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solving multiple Editor’s Notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes in 5.4: 1) Added that Vertical Federated learning is a machine learning techinique working without exchanging/sharing of local data set nor ML models 2) Editor's note: Support for AF acting as VFL server selecting multiple NWDAFs is FFS. Is solved by introducing: - An AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s)) to participate in a VFL procedure. 3) Editor's note: Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS. Is solved in procedure procedure for preparation phase Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. are solved in procedure clauses, where NWDAFs being part of inference have earlier been part of training. 4) Adding where the procedures are described Changes in 5.2: Following ENs are solved Editor's note: Details of ML Model handling, supported features, Feature alignment and Sample alignment are FFS. Editor's note: Whether we can use the existing FL capability information IE, or a new VFL capability information IE is introduced, is FFS. Editor's note: Whether additional VFL capability information needs to be registered in NRF is required or new IEs are needed in the VFL capability information, is FFS. By introducing follwing text “An NWDAF containing MTLF supporting VFL either as client or as server, include as part of the NF profile registered in NRF the VFL Interoperability information (e.g. type of supported VFL training method(s)). When the VFL training method is “neural neworks” then optionally the dimensionality of the output is also added, optionally support for ML model convergence reports and optionally supported Feature IDs.” Changes in 6.2F.2: Introducing new parameters for VFL, and stating which parameters are only for HFL. See Discussion paper S2-2410005 for details. Moved 6.2H Registration and Discovery procedure into 6.2G Changes in 6.2G Registration and Discovery procedure Solving: Editor's note: Whether the FL capability will be extended or a new VFL capability will be defined is FFS. By using new VFL capability Editor's note: Whether the VFL server/client needs to register into NRF any interoperability information is FFS. and Editor's note: Whether to use Vendor ID(s) or introduce new identifiers for interoperability among NWDAF and AFs is FFS. by using new VFL Interoperability Information Editor's note: The content of the AF VFL-related capability is FFS. by referring to clause 6.2.2.3 plus the added info in step1 Added Preparation procedure Alignments including Sample and feature alignment is done in this phase Using S2-2409416 as a base for General training procedure Added AF as server All the below Editor’s Notes are solved Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. One procedure is introduced Editor’s Note: It’s FFS to determine the VFL preparation procedure. Preparation phase including alignments is introduced Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. Consumer is introduced Editor’s Note: How the VFL server and client register to the network, how to discover VFL server or VFL client is FFS. Both registration and Discovery is explained Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. Possibility to leave and also termination of VFL procedure are explained Editor's Note: Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and how the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. This is explained in the procedure Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS. Details are explained and no flag different from HFL is added Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment. The preparation phase contains alignments. Possibility to change Sample IDs in general VFL training phase exists as well Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment. Simple alignment for proprietary Feature IDs is introduced in preparation phase Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept. Possibility to lower or increase samples is introduced in VFL training phase. No possibility to change Feature IDs during training has been introduced Editor’s Note: It is FFS whether and how to use ML Model ID or VFL Model Correlation ID or VFL correlation ID in the VFL training process. VFL correlation ID is acceptable and introduced Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS. VFL Interoperabilty information is introduced Editor´s Note: It is FFS whether and how VFL server collects trained ML model from VFL clients after the VFL training process. VFL works without sharing models, so this is not wanted nor introduced Editor´s Note: Parameters to be provided in the request need to be added New parameters are introduced Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process. VFL works without sharing models, so this is not wanted nor introduced Editor’s Note: Whether and how to involve local ML Model training information in the VFL training process is FFS; What parameter contains in local ML model training information is FFS. All needed parameters are defined in 6.2F.2. Editor’s Note: The flow should be updated to show the message exchange for VFL training termination. Introduced termination request from server and how client can ask to leave Editor’s Note: It is FFS whether model storage after each training round is required. No storage is introduced since no model sharing is to be implemented. Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. Introduced termination request from server General Inference procedure Follwing Editor’s Notes are solved: Editor's note Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. One procedure is introduced Editor's note: If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS. NEF performs no assistance apart from e.g. conversion of Sample IDs, in both directions Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. VFL is in control of determine which clients to use for training and re-use in inference Editor's note: Which service used between the consumer and the VFL server is FFS. All services are introduced in procedure Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. VFL correlation ID is acceptable and introduced Editor's note: Which service is used between the VFL server and VFL client is FFS. All services are introduced in procedure Editor's note: Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients. All parameters needed are defined in 6.2F.2 and used in procedure Editor's note: Whether the VFL client needs to retrieval of stored local models is FFS. No storage nor retrival is introduced since no model sharing is to be implemented Changes in 7.10 Changes to service defintion is introduced including making some parameters CONDITIONAL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 5.2; 5.3; 5.4; 6.2C.1; 6.2F.2; 6.2G.1; 6.2G.2; 7.10.1 and 7.10.2, 6.2H (Void) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.2G.2 and underlying sub clauses (changed text in this clause and underlying clauses, new sub clauses and moved sub clauses of 6.2H into this clause). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AI/ML	Artificial Intelligence/Machine Learning

DCAF	Data Collection Application Function

FL	Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

HFL	Horizontal Federated Learning

* * * Next Change * * *

## 5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique working without exchanging/sharing of local data set nor ML models, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AFs.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

-	An AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s)) to participate in a VFL procedure.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL model correlation ID, and it requests to report Client intermediate results.

-	combines intermediate results from VFL client(s) and its own local model and computes Server intermediate results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the Server intermediate results towards VFL clients involved in the joint VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID.

-	It combines inference result from VFL clients and its own local model and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

VFL client:

-	locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	computes the Client intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	performs inference based on the local model and local data and provides inference results to VFL server.

Editor's note:	Accuracy monitoring in VFL is FFS.

Vertical Federated Learning includes the following procedures:

- Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. For an untrusted AF, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes as part of the information about the AF an VFL capability information. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2G.2.1.

- Preparation for VFL including sample alignment to ensure that all the VFL participants has common samples when training ML models. as described in clause 6.2G.2.2.

- VFL training as described in clause 6.2G.2.3.

- Distributed inference as described in clause 6.2G.2.4.

* * * Next Change * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

Editor's note:	How to indicate supporting model training for the LMF-based AI/ML positioning as defined in TS 23.273 [39] by the MTLF to NRF, e.g., using a specific analytics ID or a new indication is FFS.

Editor's note:	For model training for LMF-based AI/ML positioning, whether Positioning case information (e.g. Uplink/downlink, or case 2b, 3b) can be included in NF profile as an optional parameter is FFS.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 5:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 6:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

To discover a VFL server or a VFL client via NRF, an NWDAF:

-	Acting as VFL server shall include its (VFL) capability information including (VFL) capability type (i.e. VFL server), during the registration in NRF.

-	Acting as VFL client shall include its (VFL) capability information including (VFL) capability type (i.e. VFL client), during the registration in NRF.

An NWDAF containing MTLF supporting VFL either as client or as server, include as part of the NF profile registered in NRF the VFL Interoperability information (e.g. type of supported VFL training method(s)). When the VFL training method is “neural neworks” then optionally the dimensionality of the output is also added, optionally support for ML model convergence reports and optionally supported Feature IDs.

NOTE 7:	The Feature IDs are optional, and if not included the Analytics ID, use case context can be used to know what features to use.

Editor's note:	Handling of an untrusted AF acting as VFL client, including Analytics ID translation, needs to be documented in a separate clause, and it's FFS.

* * * Next Change * * *

## 5.3	Federated Learning (FL) among multiple NWDAFs

This clause describes Horizontal Federated Learning.

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from FL client NWDAFs.

-	sends the global ML Model back to FL client NWDAFs to perform an additional training iteration if needed.

FL client NWDAF:

-	locally trains ML Model as tasked by the FL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the FL server NWDAF.

-	receives the global ML Model from FL server NWDAF and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML Model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model. The FL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating FL server to initiate FL procedure on the FL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * Next Change * * *

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Horizontal Federated Learning technique to train an ML Model.

* * * Next Change * * *

### 6.2F.2	Contents of ML Model Training

The consumers of the ML Model training services (i.e. an NWDAF containing MTLF) may provide the input parameters in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	For HFL, ML Model Interoperability Information as defined in clause 6.2A.2.

-	(Only for Nnwdaf_MLModelTraining_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with the subscription.

-	[OPTIONAL] For HFL, ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] For HFL, ML Model file.

NOTE 1:	It is up to NWDAF implementation to determine whether to include ML Model file in input parameters considering ML Model file size, etc.

-	[OPTIONAL] For HFL, ML Model identifier: identifies the provided ML Model.

-	[OPTIONAL] ML Preparation Flag: identifies whether the request is for preparing Federated Learning or executing Federated Learning.

-	[OPTIONAL] ML Model Accuracy Check Flag: identifies that the request is for using the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the NWDAF service consumer acting as the FL Server NWDAF.

-	[OPTIONAL] For HFL, ML Correlation ID: identifies the Federated Learning procedure for training the ML Model. This parameter is included when the service is used for Federated Learning.

-	[OPTIONAL] For HFL, Data Availability requirement. This is the requirement on data availability for the ML Model training. e.g. FL Server NWDAF sends the requirement in preparation request to a FL Client NWDAF for selecting the FL Client NWDAF which can meet the data availability requirement. The following may be included:

-	Event ID list to be collected for local model training.

-	Dataset statistical properties as defined in clause 6.1.3.

-	Time window of the data samples.

-	Minimum number of data samples.

-	[OPTIONAL] FL Availability time requirement. This is the requirement on availability time for the ML Model training, e.g. FL Server NWDAF sends the requirement in preparation request to FL Client NWDAF for selecting the FL Client NWDAF which is available in the required time for training ML Model.

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	[OPTIONAL] Target of Training Reporting: indicates the object(s) for which data for ML Model training is requested, i.e. group of UEs identified by a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Use case context: indicates the context of use of ML Model.

-	[OPTIONAL] Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e. model training results).

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of current ML Model training.

-	[OPTIONAL] Expiry time.

-	[OPTIONAL] Indication of skipping the current FL round.

-	For VFL, VFL model correlation ID: identifies the Vertical Federated Learning procedure for training the ML Model.

-	[OPTIONAL] During VFL preparation phase, VFL Interoperability information containing the VFL training method and the additional information that is needed for interoperability (e.g.,when VFL training method is  neural networks, then the dimensionality of the output may be provided, etc.).

- [OPTIONAL] During VFL preparation phase, additional VFL Interoperability information that depends on the VFL training method. When VFL training is neural networks, the additional VFL Interoperability information, suggested by server includes the dimensionality of the intermediate results which defines the number of samples, number of nodes in the cut layer for the intermediate matrix, the type of content in the intermediate matrix container, exchanged between VFL Clients and VFL Server (e.g., activations, gradients, etc.). the supported model convergence reports.

- [OPTIONAL] During VFL preparation phase, the type of content in the intermediate matrix container, exchanged betweenVFL Clients and VFL Server (e.g., activations, gradients, etc.)

- [OPTIONAL] During VFL preparation phase, the supported model convergence reports.

- [OPTIONAL] For VFL preparation phase, the list of samples suggested by Server.

- During VFL training, Intermediate results that carries information between the VFL participants (server or client). The Intermediate results contain following:

-	Intermediate matrix indicator may be set to e.g. gradients, when the VFL training is "neural networks" and/or other agreed content in the preparation phase.

-	Intermediate matrix container which includes the matrix of e.g., gradients per batch of samples sent in the container.

-	[OPTIONAL] model convergence reports which are used to support the converge of a model during training and may contain one or multiple of the below parameters, The server sends it to the clients that may use it to adapt the internal ML model training settings (Settings are implementation specifics or set by configuration and some examples of possible settings are given below):

-	Expected number of iterations left.

-	Convergence speed such as Fast/slow/almost done converegence. As an example, this may trigger the receiver to e.g., set the learning rate.

-	Under or over fitting. This may trigger the receiver to e.g., expand or reduce the input data to be collected.

-	Not converging at all. This may trigger the receiver to e.g., change the ML Model.

-	Stuck in a local critical point. May trigger the receiver to introduce random noise to its local model.

-	Addition of random noise or similar in Intermediate results. This is sent to inform the receiver, that local monitoring of convergence may diverge from expected.

-	CONDITIONAL Inference indication which indicates that inference is ongoing and only Client Intermediate results without model convergence reports shall be sent from clients.

The NWDAF containing MTLF provides to the consumer of the ML Model training service operations as described in clause 7.10 and clause 7.11, the output information in notification or response as listed below:

-	(Only for Nnwdaf_MLModelTraining_Notify) The Notification Correlation Information.

-	[OPTIONAL] ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] For HFL, ML Model identifier: identifies the provisioned ML Model.

-	[OPTIONAL] Global ML Model Accuracy information: The model metric value of the global ML Model and optionally the used metric, which is calculate by the FL Client NWDAF using the local training data as the testing dataset.

[OPTIONAL] Status report of FL training: Accuracy information of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure.

NOTE 2:	The parameters in Training Input Data Information are up to the implementation.

-	[OPTIONAL] ML Correlation ID. This parameter may be included when the service is used for Federated Learning.

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of ML Model training indicated by the FL Server NWDAF.

-	[OPTIONAL] Delay Event Notification with the following parameters:

-	delay event indication: this parameter indicates that FL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the FL Server NWDAF.

-	[OPTIONAL] cause code (e.g. local ML Model training failure, more time necessary for local ML Model training, etc.).

-	[OPTIONAL] Expected time to complete the training: Indicates to the FL Server NWDAF that expected remaining training time and may be provided with Delay Event Notification.

-	For VFL, VFL model correlation ID: identifies the Vertical Federated Learning procedure for training the ML Model. This is not sent to a consumer outside the VFL participants.

-	[OPTIONAL] For VFL preparation phase, agreeable VFL Interoperability information, including for instance dimensionality of the intermediate results.

-	[OPTIONAL] For VFL preparation phase, the list of samples agreeable by VFL client.

-	[OPTIONAL] For VFL, model convergence reports (during preparation phase the listed parameters indicates which parameters are supported to be received from server. The client shall not send any other parameter than indicated as supported by server in the request).

- [OPTIONAL] For VFL training, Intermediate results that carries information between the VFL participants (server or client). The Intermediate results contain:

-	Intermediate matrix indicator may be set to e.g. activations when the VFL training is "neural networks" and/or other agreed content in the preparation phase.

-	Intermediate matrix container which includes a matrix of e.g., activations per batch of sample sent in the container.

-	[OPTIONAL] model convergence reports which are used to support the converge of a model during training and may contain one or multiple of the parameters mentioned above in the request from the server, The reports are only sent if the client perform convergence monitoring.

NOTE x: Addition of random noise or similar in Intermediate results is only added after server has sent a report that will initiate a setting in client that introduces such action.

* * * Next Change * * *

## 6.2G	Vertical Federated Learning

### 6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL.

### 6.2G.2	Procedures

#### 6.2G.2.1	Registration and Discovery procedure for Vertical Federated

This procedure describes Registration of VFL participants and Discovery of the clients.

Figure 6.2H.2.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1-3.	VFL Server/Client NWDAF and VFL Client AF register to NRF with its NF profile, which includes NF Type (as defined in clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), VFL Interoperability information, Address information of NWDAF/trusted AF, optionally Service Area, Time interval supporting FL as described in clause 5.2.

For an untrusted AF, the NEF registers at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes other information related to the AF capabilities.

Steps 4 to 6 are the NWDAF and AF Discovery procedures.

4-6.	VFL server determines that the ML Model requires VFL based on e.g. operator policy, Analytics ID, that data cannot be obtained directly from data producer NF (e.g. due to privacy reasons) and/or Service Area.

NOTE 1:	Step 4 in Figure 6.2G.2.1-1 may be triggered by VFL server itself or a request from a consumer.

If an NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, Time Period of Interest, and Service Area.

Once the VFL Server is determined, a VFL Server internal to PLMN discovers VFL Clients from NRF by invoking the Nnrf_NFDiscovery_Request service operation and an untrusted AF uses Nnef_NFDiscovery_Request service operation, where NEF forwards the request by using Nnrf_NFDiscovery_Request service operation The following criteria might be used: Analytics ID, and Service Area.

NOTE 2: A deployment scenario that can be agreed between operator and application service provider is when untrusted AF is server and only one NWDAF will be client. In such case the Server can be configured to go directly to preparation procedure.

#### 6.2G.2.2	Preparation procedure for Vertical Federated Learning

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) on what of the VFL Interoperability information shall be used, sample alignment and optionally feature alignment.

NOTE 1: 	Features can be non-specified, privacy protected and therefore feature alignment is a simple alignment using info registered in NRF, e.g. data sources info or proprietary Feature ID).

Figure 6.2G.2.2-1: Registration and Discovery procedure for Vertical Federated Learning

If VFL Server is an AF, it may use UE member selection to select candidate UEs before starting below procedure.

NOTE 2:	Federated Learning preparation procedure (i.e. steps 1-3) can be skipped if the VFL Server can decide that theVFL Client(s) support the FL procedure to be performed, e.g. based on information acquired from previous FL procedures or from the NRF, or based on local configuration.

1.	FL Server sends Federated Learning preparation request to theVFL Client(s), using Nnf_MLModelTraining_Subscribe or Nnf_MLModelTrainingInfo_Request service with the ML Preparation Flag. The Server may add VFL Interoperability information, sample IDs, Analytics ID and optionally Feature ID.

Nnf_MLModelTraining_Subscribe can either be Nnwdaf_ MLModelTraining_Subscribe, Naf_ MLModelTraining_Subscribe or Nnef_ MLModelTraining_Subscribe dependent on whether server is NWDAF or AF and whether clients are NWDAF(s), trusted AF(s) or untrusted AF(s). The NEF is used to forward the Nnef_ MLModelTraining_Subscribe, to NWDAF or untrusted AF respectively. Before forwarding between untrusted AF(s) and NWDAF(s), the NEF converts between internal sample IDs and external sample IDs, respectively.

In the case when untrusted AF is server and no discovery according to 6.2G.2.1 have been done prior to preparation phase, the NEF either Discovers NWDAF using NRF or is configured to determine an NWDAF to forward the request. NEF can use e.g. a parameter in VFL Interoperability information, to select NWDAF.

NOTE 3: The NWDAF that NEF determines to forward the request to, may work like a proxy for VFL requests and may have a very broad service area and may e.g. forward the request to another NWDAF with smaller service area fitting the requested area.

2.	FL Client(s) checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

3.	FL Client(s) invokes Nnf_MLModelTraining_Notify or Nnf_MLModelTraining_Subscribe response or Nnf_MLModelTrainingInfo_Request response service operation to indicate to theVFL Server whether it accepts the requirements, request new requirements or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the FL process.

Nnf_MLModelTraining_Notify can either be Nnwdaf_ MLModelTraining_Notify, Naf_ MLModelTraining_Notify or Nnef_ MLModelTraining_Notify dependent on whether server is NWDAF or AF and whether clients are NWDAF(s), trusted AF(s) or untrusted AF(s). The NEF is used to forward the Nnef_ MLModelTraining_Notify, to NWDAF or untrusted AF respectively.

If the notify or response includes new requested requirements, procedure from step 1 is repeated.

4.	FL Server NWDAF determines theVFL Client(s) to be involved in the FL procedures based on the information received in step 6 in Figure 6.2G.2.1-1 (if performed) and other information received in step 3 (if available).

#### 6.2G.2.3	General training procedure for Vertical Federated Learning

The figure 6.2G.2.3-2 below shows general training procedure for Vertical Federated Learning.

Figure 6.2G.2.3-1: General training procedure for VFL

This below procedure aims for NWDAF based Vertical Federated Learning. The VFL training process may be conducted after VFL preparation procedure.

0.	OPTIONAL If the NWDAF can be the VFL server, then step 0 is skipped.

If the NWDAF does not have a model, and the NWDAF would like to request such model from another NWDAF, it sends a subscription request for a model to NWDAF containing MTLF which in this case also is a VFL server NWDAF using Nnwdaf_MLModelProvision_Subscribe including Analytics ID, optionally Target of Analytics Reporting = e.g. UE IDs.

Or NWDAF containing MTLF does not have a model and realizes it cannot be VFL server, the NWDAF sends a subscription request for a model to VFL server AF using Nnwdaf_MLModelTraining_Subscribe including Analytics ID, Optionally Target of Analytics Reporting = e.g. UE IDs.

Either based on the information received, operator policy or internal configuration, VFL server decides to iniate VFL inference procedure with VFL clients.

If decision is triggered on request from an NWDAF, the VFL server sends in the response that no model will be available to download.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL as described in the clause 6.2G.2.2.

2.	To start VFL training, VFL server sends a request to the selected VFL clients.

When VFL server is NWDAF, it sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request to the selected AF clients(s), which participate in the Federated learning to perform the local model training. The request includes VFL model correlation ID, sample IDs, and may include VFL Interoperability information.

If untrusted AF(s) are involved as VFL Client(s), the message from VFL Server goes to the NEF using Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request. The NEF forwards to untrusted AF using Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request.

Before forwarding to untrusted AFs, the NEF converts internal sample IDs to external sample IDs.

When VFL Server is AF, it sends per client a Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request to NEF. The request includes VFL model correlation ID, sample IDs, and may include VFL Interoperability information.

Before forwarding to NWDAF(s), the NEF converts external sample IDs to internal sample IDs NEF forwards the request to each client using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request. In this case no ML Model Interoperability information is sent.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model based on its own data, and reports Client intermediate result of the local ML model to the VFL server.

An NWDAF client use Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response, while an AF client use Naf_MLModelTraining_Notify or Naf_MLModelTrainingInfo_Request response. The Client Intermediate results includes Intermediate matrix indicator per intermediate Intermediate matrix container and may include model convergence reports.

A client may indicate in message that it will leave the FL.

If untrusted AF(s) are involved as VFL Client(s), the message from VFL Clients goes to the NEF. The NEF forwards to NWDAF using Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response.

Before forwarding, the NEF converts between external sample IDs and internal sample IDs

Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS.

5.	The VFL server may collect local data and generate its own local intermediate result. The VFL Server computes  training information (e.g. gradient information or loss information) based on the client intermediate results received in step 4 and its own intermediate results in combination with labels. The information is used in later step to send Server intermediate result for updating the models of VFL clients. Different Server intermediate results may be computed for different VFL clients, respectively.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of feature is computed by VFL server is FFS.

6.	[Optional] The VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required. If yes, it determines the VFL Training is completed.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: The flow should be updated to show the message exchange for VFL training termination.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to update the ML model metric to the consumer.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

If the VFL server received a request from consumer or decide to stop the Federated Training process, steps 7 is performed with a termination request and 8 is skipped.

7.	When NWDAF is VFL server, and if the VFL procedure continues, VFL Server determines whichVFL Client(s) to continue the VFL with and sends to NWDAF client Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request and to AF client Naf_MLModelTraining_Notify or Naf_MLModelTrainingInfo_Request response, that includes the Server Intermediate result to theVFL Client(s). The Server Intermediate results includes Intermediate matrix indicator per intermediate matrix container and may include model convergence reports, sample IDs (if changed).

If the VFL procedure terminates and to the VFL Clients which is not selected to continue, VFL Server sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes termination request. The VFL Server and each VFL Client, stores the VFL model correlation ID and latest information related to their locally trained Models. Step 8 is not performed.

NOTE: The VFL model correlation ID is later used for inference.

When VFL server is AF, and if the FL procedure continues, VFL Server determines which VFL Client(s) to continue the FL with and sends per client to NEF Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request that includes the Server Intermediate result to theVFL Client(s). The Server Intermediate results includes Intermediate matrix indicator per intermediate matrix container and may include model convergence reports, sample IDs (if changed).

If the FL procedure terminates and to theVFL Clients which is not selected to continue, VFL Server NWDAF sends Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request that includes termination request. The VFL Server and each VFL Client, stores the VFL model correlation ID and latest information related to their locally trained Models. Step 8 is not performed.

If untrusted AF(s) are involved as VFL Client(s), the message from VFL Server goes to the NEF using Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request. The NEF forwards to untrusted AF using Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request.

Before forwarding received message, and sample IDs are present the NEF converts between internal sample IDs and external sample IDs.

8.	Each VFL client computes gradient of its local model and updates its local ML model based on Server intermediate results distributed by the VFL server at step 7.

The steps 3-8 should be repeated until the training termination condition is reached.

#### 6.2G.2.4	General inference procedure for vertical federated learning

The inference process occurs once the training process has been done, the VFL Server knows that there is an ML Model for an AnalyticsID trained and determines which ones from participants in the training that shall participate in the inference. The VFL Server has kept the client NF IDs from training and can from that initiate inference. The server and clients (if NWDAFs) are a combined NWDAF containing both MTLF and AnLF. The differences between the VFL training and inference are that there is no check of the labels, nor any Server Intermediate results are sent and as such only Client Intermediate results are sent to the server.

The figure below shows general inference procedures for Vertical Federated Learning.

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server using Nnwdaf_AnalyticsInfo_Request/Nnwdaf_AnalyticsSubscription_Subscribe or Nnef_AnalyticsInfo_Request/Nnef_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

Before forwarding the message, the NEF do conversions, e.g. between internal UE IDs and external UE IDs.

2a.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients.

VFL Server selects clients(s) using input from earlier training described in 6.2G.2. The server may not select all clients, e.g. depending on their contribution to the training result.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

2b	When VFL server is NWDAF and the VFL client(s) are NWDAF(s), VFL server sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request  to the VFL clients which includes VFL model correlation ID to indicate the VFL client which previously trained VFL local model associated with this ID will be used.

When VFL server is NWDAF and the VFL client(s) are AF(s), VFL server sends Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request  to the VFL clients which includes the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

When VFL server is AF the VFL client(s) are NWDAF(s), VFL server sends Nnef_MLModelTraining_Subscribe or Nnef_MLModelTrainingInfo_Request to the VFL clients which includes the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

Before forwarding the message, the NEF converts between internal sample IDs and external sample IDs

3.	OPTIONAL Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

5.	Each VFL Client sends the Client intermediate  results to the VFL server using Nnwdaf_MLModelTraining_Notify, Naf_MLModelTraining_Notify and Nnef_MLModelTraining_Notify respectively.

Before forwarding, the NEF converts between internal sample IDs and external sample IDs.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The Client intermediate results, which are sent from the VFL Client(s) to the VFL Server during the VFL inference process, contains the information for the VFL Server to generate the VFL inference results.

6.	The VFL server may collects local data and generate intermediate local results. The VFL Server combines all the intermediate results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computiong the VFL inference is FFS.

7.	Depending on service used in step 1, the VFL server sends Nnwdaf_AnalyticsInfo_Response/Nnwdaf_AnalyticsSubscription_Notify or Nnef_AnalyticsInfo_Response/Nnef_AnalyticsSubscription_Notify forward to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

Before forwarding, the NEF converts between internal sample IDs and external sample IDs.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * * Next Change * * *

## 6.2H	 Void

* * * Next Change * * *

## 7.10	Nnwdaf_MLModelTraining Service

### 7.10.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe/notify/modify for ML Model training.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Horizontal Federated Learning, this service enables FL server NWDAF to enable Federated Learning while providing global ML Model information to FL Client NWDAF and getting local ML Model information and status report of FL training as defined in clause 6.2C.2.3 from the FL Client NWDAF.

When used for Vertical Federated Learning, this service enables the VFL server to initiate VFL as defined in clause 6.2G.2.3.

This service may also be used by the consumer (i.e. FL Server NWDAF) to check if the service provider (i.e. FL Client NWDAF) can meet the ML Model training requirement as described in clause 6.2F.1.

This service may also be used by the consumer (i.e. FL Server NWDAF) to request the service provider (i.e. FL Client NWDAF) to calculate and provide Model Accuracy of the global ML Model as described in clause 6.2F.1.

### 7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML Model training with specific parameters.

Inputs, Required:

-	CONDITIONAL Analytics ID as defined in Table 7.1-2, not provided for VFL inference;

-	CONDITIONAL Inference indication, which is replacing Analytics ID during VFL inference;

-	CONDITIONAL applicable for HFL, ML Model Interoperability information;

-	CONDITIONAL applicable for VFL, VFL model correlation ID;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2);

-	ML Model file;

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Use case context;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current FL round.

-	VFL Interoperability information:

-	VFL training method

-	additional VFL Interoperability information.

- Intermediate results that carry information between the VFL participants (server or client) according to 6.2F.2:

-	Intermediate matrix indicator(s)

-	Intermediate matrix container(s)

-	model convergence reports according to 6.2F.2.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: ML Correlation ID (e.g. confirm of the subscription for this FL process).

Editor’s Note:	Further updates to Nnwdaf_MLModelTraining Service and the underlying clauses are FFS.

* * * End of Changes * * *