3GPP TSG-WG SA2 Meeting # 165	S2-2410032

# Hyderabad, IN, 14 - 18 October 2024										(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5699 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB Regulatory services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the UE access control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UE access control description in clause 5.49.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions related to access control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR#5700 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause referred here 5.49.1.x is defined in CR#5700 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.49.8	Support of regulatory service via MWAB

Based on configurations as described in clause 5.49.1.1, regulatory services (e.g. emergency services, priority services) may be supported by the MWAB-gNB within the MWAB Broadcasted PLMN.

Based on existing configuration mechanisms using URSP, S-NSSAI/DNN and also other information may be provided from the MWAB-gNB via implementation based internal communication, the MWAB-UE and BH 5GC may establish appropriate BH PDU session to support the traffic for the regulatory services supported by the MWAB-gNB.

NOTE:	Operator can configure specific S-NSSAI and DNN with proper QoS handling as needed (see clause in 5.49.1.x).

When MWAB is de-authorized as described in clause 5.49.3, the MWAB-gNB triggers the handover procedure for UEs in CM-CONNECTED mode as described in clause 5.49.5, including the case when MWAB-gNB provides regulatory services.

# * * * * End of changes * * * *