3GPP SA WG2 Meeting #165	 S2-2410049

# 14 - 18 October, 2024, Hyderabad, India 									                 (was S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1386 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3_PSI based DSCP marking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusion was agreed in TR 23.700-70 on Key Issue 3 QoS Handling when Traffic Characteristics Change Dynamically and included in SP-240990 which was approved in SA#104 - For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF as described in clause 5.8.2.7 of TS 23.501 [2]. For a QoS Flow configured for PDU Set QoS handling, besides the above information, the SMF may further consider PDU Set Importance values to be used by the QoS Flow, to provide a list of DSCP marking values, each of the DSCP marking values corresponding to one or several PDU Set Importance values. NOTE 1: It is recommended that DSCP markings be used to vary the drop precedence between PDUs in the transport network nodes (e.g. IP routers) on the N3/N9 interfaces. If the Class Selector Codepoint of the DSCP markings varies within a QoS Flow, the packets of the QoS Flow can be reordered by the transport network. NOTE 2: The transport level marking values being provided on per-QoS Flow basis, it is up to operator deployments to enforce consistency of transport level marking in the transport network. - The SMF sends the mapping list of the transport level marking for DL packets (N3/N9 interface) and related PDU set importance value(s) in the FAR to the UPF, and the UPF performs the DSCP value marking based additionally on the PDU Set Importance per PDU Set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Feature description to support the KI#3 conclusions in clause 8.3 of TR 23.700-70. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.27.x	PDU Set Importance based DSCP Marking

The PDU Set Importance based DSCP marking as specified in clause 5.8.2.7 and 5.37.1 of TS 23.501 [2].

NOTE:	It is recommended to keep consistency of PDU Set Importance Marking of SDFs from the same AS, when considering the PDU Set Importance based DSCP Marking.

# * * *  End of changes * * * *