SA WG2 Meeting #165	S2-2410056

14th – 18th October, 2024, Hyderabad, India

Source:	Lenovo, NEC, HONOR

Title:	Conclusions on the common supported functionality of AIoTF

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes conclusion principles for the supported functionality of AIoTF.

1	Discussion

After SA2#164 meeting, initial conclusion principle on architecture support for topology 1 was agreed:

A new core network function is introduced to support Ambient IoT.

Based on the introduction of AIoTF, this paper continues to further discuss the supported functionality of it. An assumption is made that the new network function, i.e., AIoTF, will also be introduced to support Ambient IoT related service in topology 2 when UE is acting as the reader.

Based on this, some common principles on the supported functionality of AIoTF, for both the topology 1 and topology 2 are presented in this paper.

2	Proposal

It is proposed to include the following changes in TR 23.700-13.

* * * * Start of Changes – All New Text * * * *

8.1	Conclusions for Key Issue #1

8.1.X 	Supported common functionality of AIoTF

The following aspects are concluded as principles for normative work:

- A new core network function (e.g., AIoTF) is introduced to support Ambient IoT service for both the topology 1 and topology 2.

- The AIoTF is responsible for both the BS reader and UE reader selection to interact with the AIoT devices for the AIoT service. Optionally, the BS reader and UE reader register at the AIoTF with their supported service information, location, and capability. Optionally, for topology 2, AIoTF may interact with the AMF/RAN for selecting the UE reader.

- The AIoTF performs the AIoT device ID validation by interacting with the AUSF/UDM/AAA server, when necessary.

- The AIoTF terminates the AIoT NAS protocol from the AIoT device.

- AIoTF stores the AIoT device context information after successful device ID validation. Optionally, AIoTF can store and update the device context information into UDM, that includes the device status, last known (binding) reader information of the device, and the device validation result.

- The AIoTF supports the inventory and command operations with the following details:

- AIoTF receives an AIoT service request (e.g., inventory, write, read, enable, disable) from the AF via NEF. The AIoTF may store an AIoT service context/parameters based on the request from the AF.

- AIoTF executes the AIoT service request within the network by creating an AIoT NAS message and sending it to the selected reader(s).

- AIoTF configures the reader with necessary parameters for required AIoT service.

- AIoTF collects the AIoT related service response received from reader, and optionally aggregates the responses, before exposing to the AF via NEF.

- The AIoTF registers itself in the NRF with its NF profile, including the supported service information and its service area (e.g. the service area is the sum of the coverage areas of the readers registered with the AIoTF).

NOTE: The functionalities listed above may be related to the conclusions of other Key Issues.

* * * * End of Changes * * * *