3GPP TSG-WG2 Meeting #165	 S2-2410065

# 14 - 18 October, 2024, Hyderabad, India	                                                  (revision of S2-2409154)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4883 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF influence on N6 delay measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR intends to support the 5GC enables N6 delay measurement via AF influence procedure based on the conclusion of clause 8.2 in TR 23.700-49. To address the following NOTE in the conclusion: “NOTE 1: For the enabling of SMF to measure N6 delay aspect, whether one mechanism between AF influence and EDI is enough will be determined in normative phase.” It is concluded that AF can provide an indication of considering N6 delay, a list of DNAI(s), measurement endpoint, the measurement protocol(s) and optionally the protocol-specific configuration parameters. - For “Indication of considering N6 delay”, it is per application and more suitable to be provided via AF influence procedure. - For “measurement endpoint”, AF may provide different measument endpoints for different applications, instead of providing all the possible measurement endpoints deployed in the Local part of the DN. So this parameter is also per application. For “measurement protocol(s) and protocol-specific configuration parameters”, it is corresponding to specific measurement endpoint. So they are more suitable to be provided via AF influence procedure. - For “a list of DNAI(s)”, the existing parameter Potential Locations of Applications could be reused. Proposal: The AF influence mechanism is more suitable and enough to provide the parameters for enabling SMF to measure N6 delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the descriptions for supporting enhancement of EAS and local PSA UPF (re)selection in procedure of AF influence traffic routing. - Add support of enhancement of EAS and local PSA UPF (re)selection related parameters in Nnef_TrafficInfluence_Create operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new functionality of enhancement of EAS and local PSA UPF (re)selection can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1, 4.3.6.2, 5.2.6.7.2, 5.2.5.3.2, 5.2.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> First change <<<<

4.3.6.1	General

Clause 4.3.6 describes the procedures between an Application Function and the SMF to maintain an efficient user plane path and/or to provide N6-LAN service function chaining for Application Functions that require it.

As described in clauses 5.6.7 and 5.6.16 of TS 23.501 [2], an Application Function may send requests to influence SMF routeing decisions for User Plane traffic of PDU Sessions. The AF requests may influence UPF (re)selection and allow routeing of user traffic to a local access (identified by a DNAI) to a Data Network and/or influence the steering of user traffic to service function chain(s) identified by SFC identifier(s). The AF may also provide in its request subscriptions to SMF events. A V-SMF supporting HR-SBO provides, SUPI, an indication of support for HR-SBO, and HPLMN DNN and S-NSSAI related to the PDU session to the L-PSA UPF at PDU session establishment.The AF may also request N6 delay measurement for the delay sensitive application.

NOTE 1:	NEF uses the SUPI to derive PLMN ID of the UE.

The following cases can be distinguished:

NOTE 2:	Such requests target an on-going PDU Session.

When receiving an AF request on Application Function influence on traffic routing targeting an individual UE IP address the NEF needs to determine whether the target PDU Session is working in HR-SBO mode. If the target PDU Session is NOT working in HR-SBO mode (non roaming or LBO PDU Session) then the NEF contacts the PCF of the PDU Session as further defined in the clause 4.3.6.4. If the target PDU Session is working in HR-SBO mode, the NEF does not contact the PCF of the PDU session but needs to store the AF request in UDR as defined in clause 4.3.6.5.

The NEF determines whether the PDU Session that the AF requests to influence is working in HR-SBO mode or not and in the former case determines the HPLMN of the UE, the DNN and S-NSSAI of the PDU Session as follows:

If the AF has determined based on procedure in clause 4.3.6.5.2 that the PDU session is home routed, the AF provides IP address of the UE (assigned by HPLMN) and HPLMN ID, HPLMN DNN and S-NSSAI, and the NEF determines if the PDU session is subject to HR-SBO using the provided information and proceeds according to clause 4.3.6.5.2.

If the AF has provided an IP address of the UE known to the AF (IP address not assigned by HPLMN), and DNN and S-NSSAI, the NEF determines if the PDU session is subject to HR-SBO based on local configuration. If PDU session is not subject to HR-SBO, the NEF proceeds according to clause 4.3.6.4. If PDU session is subject to HR-SBO the NEF determines the HPLMN and proceeds from according to clause 4.3.6.5.2, excluding step 0a.

NOTE 3:	The DNN can be unique for a PLMN (see TS 23.003 [33]) and can be mapped to a HPLMN, or the IP address can be within a range that can be mapped to a HPLMN.

If the AF has not provided the (H)PLMN ID, DNN and S-NSSAI:

-	If the IP address of the UE in the AF request is a private IP address, the NEF determines if the request relates to HR-SBO or not based on local configuration, or based on interactions with UPF, since the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. If the request relates to HR-SBO, the NEF proceeds according to clause 4.3.6.5.3, otherwise it proceeds according to clause 4.3.6.4.

NOTE 4:	Whether or not NEF interact with UPF to determine if HR-SBO applies is based on configuration. If NEF does not interact with UPF, the 5GC NF (e.g. SMF/UPF, NEF) in PLMN can be configured to have separate private IP address ranges for HR-SBO PDU Sessions. This would allow NEF to distinguish AF request for influence on traffic routing for HR-SBO PDU Sessions from non-roaming and LBO PDU Sessions.

-	If the IP address of the UE in the AF request is a public IP address:

-	If this Public IP address belongs to an IP range not owned by the PLMN of the NEF, then the target PDU Session is working in HR-SBO mode and the NEF proceeds according to clause 4.3.6.5.4.

-	If the UE IP Address in the AF request is an IP address NATed by the PLMN that the NEF belongs to, the NEF invokes steps 3 to 6 of clause 4.15.10 to get the corresponding private UE IP address. If HR-SBO applies, the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. Thus, the NEF can also receive these parameters and determines if the AF request relates to HR-SBO PDU Session, the NEF proceeds according to clause 4.3.6.5.3. Otherwise the NEF proceeds according to clause 4.3.6.4.

NOTE 5:	In this Release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

NOTE 6:	It is assumed that the NEF is configured with the NATed IP range of its own PLMN. It is assumed that the NEF is configured based on HR-SBO roaming agreements for the Public IP address ranges with an HPLMN ID.

NOTE 7:	Whether the AF needs to use the NEF or not is according to local deployment. If the AF request is expected to possibly address PDU Sessions in HR-SBO mode, then the AF sends its requests via the NEF.

When AF requests are routed (by the AF or by the NEF) to an individual PCF this may use the BSF. This case applies to both AF influence on traffic routing as well as AF influence on Service Function Chaining. This is described in clause 4.3.6.4.

-	AF requests described in clause 5.6.7 of TS 23.501 [2] or clause 5.6.16 of TS 23.501 [2] targeting a group of UE(s), or any UE accessing a combination of DNN and S-NSSAI and optionally PLMN ID of the DNN and S-NSSAI, or targeting individual UE(s) by one or more GPSI(s) or targeting UEs with External Subscriber Category(s) which can be combined with External Group ID(s) or any UE as described in table 5.6.7-1. These AF requests may also affect UE(s) with an established PDU session. For such requests the AF shall contact the NEF and the NEF stores the AF request information in the UDR. For non-roaming and LBO cases, the PCF(s) receive a corresponding notification if they had subscribed to the create/update/delete operations of the AF request information corresponding to UDR Data Keys / Data Sub-Keys. This is defined in clause 6.3.7.2 of TS 23.501 [2] and further described in clause 4.3.6.2. For HR-SBO case, V-SMF(s) receive a corresponding notification if it has subscribed to the create/update/delete operation of the AF request information corresponding to UDR Data Keys /Data Sub-Keys, this is described in clause 4.3.6.5. For a single UE, NEF determines if HR-SBO applies for the PDU session based on the local configuration using the received GPSI. For "any UE", both PCF(s) and SMF(s) (via NEF) may need to subscribe to creates/updates/deletes of AF influence on traffic routing data in UDR depending on if a specific PLMN is targeted.

NOTE 8:	Such requests can target on-going or future PDU Sessions.

If the AF request targets any UE the procedure described in clause 4.3.6.5 may also be performed. In that case if HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have been provided (see clause 4.3.6.5.2), if the HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have not been provided by the AF, the NEF determines the HPLMN DNN and HPLMN S-NSSAI as described in clause 4.3.6.5.5.

This Release of the specification does not support AF influence on traffic routing for HR-SBO PDU session, if the AF request targets an external group ID, or UE(s) identified by SUPI.

If the AF interacts with 5GC via the NEF, the NEF performs the following mappings or determinations where needed:

-	Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

-	Map the AF-Service-Identifier into a list of DNAI(s) and Routing Profile ID(s) determined by local configuration.

The NEF can only provide this mapping when the DNAI(s) being used by the applications are statically defined. When the DNAI(s) where applications are instantiated may vary dynamically, the AF should provide the target DNAI(s) in its request together with either Routing Profile ID(s) or with N6 traffic routing information.

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into Internal Group Identifier, according to information received from UDM.

-	Map the External Subscriber Category(s) and any UE, or External Subscriber Category and External Group ID(s) to, Internal Group ID(s) or Internal Group ID(s) and Subscriber Category(s).

-	Map the geographical area in Spatial Validity Condition into areas of validity, determined by local configuration.

-	Determine whether an AF request targeting an UE IP address corresponds to HR-SBO and if yes determine the HPLMN of UE, the DNN/S-NSSAI of the PDU Session based on the description given above.

NOTE 9:	As a user can be associated with multiple Subscriber Category(s), some values of Subscriber Category(s) can correspond to an SLA between an application provider represented by an AF and the 5GC operator. In the NEF API, the combination of application identifier and External Subscriber Category can also be used to refer to this SLA.

>>>> Second change <<<<

4.3.6.2	Processing AF requests to influence traffic routing and/or Service Function Chaining for Sessions not identified by an UE address

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing and/or Service Function Chaining for Sessions not identified by an UE address

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in the case of a PDU Session in LBO mode).

0.	The PCF(s) subscribe to modifications of AF requests (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = S-NSSAI and/or DNN and/or Internal Group Identifier or SUPI) from the UDR.

1.	To create a new request, the AF invokes a Nnef_TrafficInfluence_Create service operation. The content of this service operation (AF request) is defined in clause 5.2.6.7. The request contains also an AF Transaction Id. If it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

To update or remove an existing request, the AF invokes a Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

The Nnef_TrafficInfluence_Create (initiated by target AF) or Nnef_TrafficInfluence_Update (initiated by source AF or target AF) service operation may be used for the case of AF instance change. If Nnef_TrafficInfluence_Update service operation is invoked, the NEF is required to update the subscription resource. The Nnef_TrafficInfluence_Update service operation may include an updated notification target address. The updated subscription resource is used by the target AF.

NOTE 2:	If the source AF transfers the application context to the target AF, then target AF may create new subscription via Nnef_TrafficInfluence_Create operation or update existing subscription via Nnef_TrafficInfluence_Update. However, whether and how the application context transfer is done is out of this specification.

2.	The AF sends its request to the NEF. If the request is sent directly from the AF to the PCF, the AF reaches the PCF selected for the existing PDU Session by configuration or by invoking Nbsf_management_Discovery service.

The NEF ensures the necessary authorization control, including throttling of AF requests and as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_TrafficInfluence_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining, as described in Table 5.2.12.2.1-1.

NOTE 3:	Both the AF Transaction Internal ID and S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI are regarded as Data Key when the AF request information are stored into the UDR, see Table 5.2.12.2.1-1. The Subscriber Category(s) is determined by NEF as described in clause 4.3.6.1.

(in the case of Nnef_TrafficInfluence_delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining.

The NEF responds to the AF.

4.	The PCF(s) that have subscribed to modifications of AF requests receive(s) a Nudr_DM_Notify notification of data change from the UDR. The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

The PCF validates whether the SFC identifier (if available) corresponds to an authorized SFC for the AF based on local configuration. If the validation has succeeded the PCF maps the SFC identifier to the corresponding Traffic Steering Policy identifier (i.e. TSP ID).

The PCF includes the Traffic Steering Policy ID(s) in the AF influence on traffic routing Enforcement Control information and/or N6-LAN Traffic Steering Enforcement Control information of the relevant PCC rule as defined in clause 6.3.1 of TS 23.503 [20]. The PCF also includes the Metadata in the N6-LAN Traffic Steering Enforcement Control information of the PCC rule when Metadata was provided by the AF.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

If the AF request includes an EAS Correlation indication or indication of traffic correlation, PCF includes in the PCC rule(s) an EAS Correlation indication or indication of traffic correlation and a Traffic Correlation ID corresponding to a set of UEs that AF request aims at, also if AF request includes a common EAS IP address or common DNAI for a set of UEs, PCC rule includes the common EAS IP address or common DNAI.

In the case of AF influence on traffic routing, the PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3 of TS 23.288 [50], to provide an updated list of DNAI(s) to the SMF.

The PCF may use the "Subscriber categories" as defined in "PDU Session policy control subscription information" in table 6.2-2 of TS 23.503 [20] to determine whether the PDU Session is impacted by the AF request.

If the AF request includes an N6 delay measurement support indication, the PCF includes parameters, e.g. the N6 delay measurement support indication, and etc, in PCC rule(s) as per clause  6.3.1 of TS 23.503 [20].

Editor’s note: It is FFS whether or not the AF request includes the End-to-End delay requirement, the N6 delay monitoring information, the Measurement endpoint and Measurement protocol.

6.	When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to reconfigure the User plane of the PDU Session.

In the case of AF influence on traffic routing, examples of actions are:

-	The SMF may consider service experience analytics and/or DN Performance analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clauses 6.4.3 and 6.14.3, respectively, of TS 23.288 [50] before taking any actions.

-	Determining a target DNAI.

-	Determining if a common DNAI needs to be used as a target DNAI.

-	Adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF in the target DNAI with AF influence on traffic routing control parameters as described in clause 5.6.7.1 of TS 23.501 [2].

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

-	Determining whether to relocate PSA UPF considering the user plane latency requirements provided by the AF (see clause 6.3.6 of TS 23.548 [74]).

-	Triggers N6 delay measurement, and (re)selects the L-PSA UPF and EAS based on the measured N6 delay and End-to-End delay requirement (if available).

When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to assist the EAS discovery and re-discovery for PDU Session with Session Breakout connectivity model such as:

-	Retrieve the EAS deployment information as defined in clause 6.2.3.4.1 of TS 23.548 [74].

-	Providing DNS message handling rule to forward DNS messages of the UE and/or report when detecting DNS messages as defined in clause 6.2.3.2.2 of TS 23.548 [74].

In the case of AF influence on Service Function Chaining, the SMF may take appropriate actions to enforce the N6-LAN traffic steering control:

-	Provide N6-LAN traffic steering control parameters to UPF as described in clause 5.6.16 of TS 23.501 [2].

7.	The SMF may decide whether it is required to send the target DNAI to the AMF for triggering SMF/I-SMF (re)selection and then inform the target DNAI information for the current PDU session or for the next PDU session to AMF via Nsmf_PDUSession_SMContextStatusNotify service operation.

>>>> Third change <<<<

5.2.6.7.2	Nnef_TrafficInfluence_Create operation

Service operation name: Nnef_TrafficInfluence_Create

Description: Authorize the request and forward the request for traffic influence.

Inputs, Required: AF Transaction Id, AF Identifier.

The AF Transaction Id refers to the request.

Inputs, Optional: The address (IP or Ethernet) of one or more UE(s) if available, one or more GPSIs if available, DNN if available, S-NSSAI if available, External Group Identifier(s) if available, External Application Identifier or traffic filtering information, AF-Service-Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, Indication of traffic correlation, Indication of application relocation possibility, Indication of UE IP address preservation, Early and/or late notifications about UP path management events, Notification Target Address, immediate reporting flag, Temporal validity condition, Spatial validity condition, User Plane Latency Requirements, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation and AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, External Subscriber Category(s), SFC Identifier(s), Metadata, Common EAS IP address, Traffic Correlation ID, FQDN(s) , Indication of considering N6 delay as described in clause 5.6.7 of TS 23.501 [2], HPLMN of the UE. If an IP address of one or more UE(s) is provided, a corresponding port number (e.g. TCP or UPDP) for each IP address.

The IP address of one or more UE(s) may be provided together with the corresponding port number allowing to support the case where the PSA UPF is carrying out NAT on the traffic exchanged with the EAS.

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	In this Release the following cannot be used by AFs in VPLMN influencing traffic on Home Routed PDU sessions:

-	External Group Identifier;

-	External Subscriber Category.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor's note:	Besides Indication of considering N6 delay, whether the E2E delay requirement the additional indication for considering N6 delay are also needed is FFS.

>>>> Fourth change <<<<

5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Port Management Information Container for ports on DS-TT or NW-TT, or User plane node Management Information Container.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: GPSI(s) or SUPI(s) if available, Internal Group Identifier, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, ASP Identifier, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, Common EAS IP address, Traffic Correlation ID, FQDN(s) , indication of considering N6 delay, as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 or 5.37.8.3 of TS 23.501 [2], Data Burst Handing Information as described in clause 6.3.1of TS 23.503 [20], Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event, Multi-Modal Service ID together with Multi-modal Service Requirements information for each data flow as described in clause 6.1.3.27.3 of TS 23.503 [20], QoS duration, QoS inactivity interval as described in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	A dedicated Notification Target Address for PMIC/UMIC UPF event and Correlation ID for PMIC/UMIC UPF event are provided by the event consumer over Npcf_PolicyAuthorization as the corresponding events are reported by the UPF and not by the PCF. Providing such information indicates that the consumer of the Npcf_PolicyAuthorization (TSN AF, TSCTSF) supports PMIC/UMIC via Nupf event reporting.

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clauses 6.1.3.16 and clause 6.1.3.10 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Editor's note:	Besides Indication of considering N6 delay, whether the E2E delay requirement the additional indication for considering N6 delay are also needed is FFS.

>>>> Fifth Changes <<<<

5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation, indication of considering N6 delay as described in clause 5.6.7 of TS 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 or 5.37.8.3 of TS 23.501 [2], Data Burst Handing Information as described in clause 6.3.1of TS 23.503 [20], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event, updated information for Multi-modal Service Requirements as described in clause 6.1.3.27.3 of TS 23.503 [20].

NOTE:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clause 6.1.3.16 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

Editor's note:	Besides Indication of considering N6 delay, whether the E2E delay requirement the additional indication for considering N6 delay are also needed is FFS.

>>>> End of Changes <<<<