3GPP TSG SA WG2 Meeting #165	S2-2410078

Hyderabad, India, 2024‑10-14 - 2024‑10-18	(revision of S2-220xxxx)

Source:	Apple

Title:	VFL: Sample Alignment and Sample Update

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: This discussion paper analyses the key components of sample alignment procedure and makes proposals for the procedure in 5GC.

# 1. Discussion

According to the conclusions for KI#2 in 3GPP TR 23.700-84 v2.0.0 clause 8.2, it was agreed that sample alignment procedure is to be agreed in normative phase. The following principles were agreed regarding sample alignment.

P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the interclause of samples. P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information) , then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training. P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples.

The following Notes are captured for P#2.3 sample alignment for VFL.

NOTE 3: The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase. NOTE 4: How NEF assists sample alignment in P#2.3.3 will be determined in normative phase. NOTE 5: For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase.

This discussion paper analyses the key components of sample alignment procedure and makes proposals for the procedure in 5GC.

### Sample Alignment procedure for Vertical Federated Learning

The purpose of sample alignment is to make sure that VFL server and client are able to perform their own parts of ML training for the same UEs. The sample alignment procedure takes place after client selection and before the start of training.

In the case of VFL with AF as server, An AF may start the sample alignment procedure with a list of potential samples for VFL, but there are also scenarios where an AF may be interested in further constraining the operation to UEs in an Area of Interest. For the case of untrusted AF, NEF may be involved in mapping external UE IDs to internal UE IDs.

The client NWDAF verifies the requested sample UE IDs and responds with the result and if successful, the samples it can serve. A client may reject the sample alignment process due to insufficient availability of samples.

In the mirror case of NWDAF as the VFL server, the NWDAF would provide the initial list of samples. The client AF responds with the result and if success, the samples it can serve for this training operation.

1.2 Sample update procedure during VFL training

However, during the training process, there may be cases of sample misalignment. The UEs may no longer be in the Area of Interest and thus not suitable for the training process. Another aspect that may change is the user consent for participation in the VFL training process.

Sample update for VFL with AF as server

The NWDAF may get a notification triggered by UDM on the change of stored user consent values.

The client NWDAFs can trigger a sample update procedure during the course of training. It is also possible that that training session can continue with possible re-training from a previous checkpoint where the local model information was updated based on the server inputs. The sample update procedure may also include a previous checkpoint where the training should re-start due to change in samples.

The server AF confirms the sample update request and performs the sample alignment again. The server confirms the training re-start point that the client has provided or provides an alternative checkpoint that the client has to use.

There can also be cases where the AF (VFL server) needs to update the samples list. In this case, the VFL server starts with VFL sample alignment request. However, this request indicates a checkpoint from which the ongoing session is to be restarted. The client performs the same steps as initial sample alignment but it continues the training from the requested checkpoint.

### Sample update for VFL with NWDAF as server

The similar sample update may be triggered from NWDAF when acting as VFL server. Similar to AF as server case, the NWDAF performs the sample alignment procedure but with the indication that intention is to start from a specific checkpoint of the training.

If the AF, as VFL client, has to initiate the sample update procedure, it starts the VFL sample update procedure indicating a re-training starting point. NWDAF as the VFL server confirms the sample list and the training re-start point.

1.3 Generic procedure for VFL Sample Alignment and Update

The call flow below shows the VFL sample alignment and sample update procedure initiated by the client. The procedure is applicable for both AF as VFL Server or NWDAF as VFL Server.

8. VFL Sample update confirm (sample UE IDs, Retraining information)

7. VFL Sample update request (removed UE IDs, Re-training information)

6. Trigger condition for Sample update

5. Backward local ML Model information reporting (Checkpoint information)

4. Local model information reporting

3. VFL Training Start

VFL Server          AF

VFL Client

2. VFL Sample Alignment Response

1 VFL Sample Alignment Request

- VFL Client Discovery and selection

The call flow below shows the VFL sample update procedure initiated by the VFL server. The procedure is applicable for both AF as VFL server or NWDAF as VFL Server

6. Trigger condition for Sample update

8. VFL Sample Alignment Response

7 VFL Sample Alignment Request (Retraining Information)

5. Backward local ML Model information reporting (Checkpoint information)

4. Local model information reporting

3. VFL Training Start

VFL Server          AF

VFL Client

2. VFL Sample Alignment Response

1 VFL Sample Alignment Request

- VFL Client Discovery and selection

# 2. Conclusion

A corresponding CR to 23.288 to describe the VFL sample alignment and sample update procedure is presented in S2-2410077