3GPP TSG-WG SA2 Meeting #165	S2-2410083

# 14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5714 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Enhancement for BDT with Energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, TOYOTA MOTOR CORPORATION |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-66 v19.0.0 has documented to make policy decisions based on operator policy for key issue#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. The following information is stored as part of the subscription data in the UDM/UDR: - The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy. This contribution provides to enhance BDT considering the Energy Consumption information, based on the solution #25. Note that the detail of Energy consumption information is described in CR5712. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an Enhancement for BDT considering the Energy Consumption information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for BDT with Energy Consumption information in R19 Energy Efficiency study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new), 5.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5055 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***1st Changes (All new texts)***

5.X	Support for Energy Efficiency and Energy Saving

5.X.3	BDT with Energy Consumption information

5GS supports BDT (background data transfer) policy selection and re-negotiation process by introducing Energy Consumption information.

For negotiation for future BDT with Energy Consumption information, following principles apply:

-	AF may interact with H-PCF via NEF and request or subscribe a NF/Node-level Energy Consumption information as follows:

-	Area of interest, Desired time window and Energy related criteria (e.g. Maximum allowed Energy Consumption for NG-RANs and/or UPFs) as described in clause 5.X.1.

-	The H-PCF may interact with EECF and request or subscribe the required NF/Node-level Energy Consumption information.

-	The EECF generates an NF/Node-level Energy Consumption information report as described in clause 5.X.1 and provides it to the H-PCF. Then the H-PCF may consider the NF/Node-level Energy Consumption information report for determination of BDT policies.

For BDT policy warning notification with Energy Consumption information, following principles apply:

-	The H-PCF is notified with the NF/Node-level Energy Consumption information report in the Area of interest from the EECF when the EECF determines that the requested Energy related criteria go below the threshold.

-	The H-PCF may retrieve all the BDT Policies together with the relevant information received from the AF and identifies the BDT policies affected by the notification received from the EECF.

-	The H-PCF may provide a list of new set of candidate BDT policies considering the NF/Node-level Energy Consumption information report on the new network conditions, from which the AF can choose.

-	The H-PCF may also provide the reasons for triggering such re-negotiation to allow the AF to make a choice regarding the selection of the new BDT policy considering the time schedule and data transfer rate vs the imposed Energy related criteria. Alternatively, the PCF may notify the AF indicating only that the Energy related criteria cannot be met, then the AF can select a different BDT policy.

# *** End of Changes ***