3GPP TSG-WG SA2 Meeting #165	S2-240XXXX

# Hyderabad, India, 14-18 October 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | - | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Emergency SMS over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EmsSMSoNAS |  |  |  |  |  | Date: |  |  | 2023-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support emergency SMS over NAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable routing of emergency SMS over NAS based on UE location and detection of emergency numbers to a suitable PSAP in roaming and non-roaming cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex ZZ (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex ZZ (normative):
Support of emergency SMS over NAS

# ZZ.1	General

This annex describes support for the emergency Short Message Service (SMS) over NAS. Emergency SMS over NAS in this context is SMS over NAS using an emergency number as destination address. A key requirement for this service is that the emergency Short Message (SM) must be routed to a PSAP in the country where the UE initiating the emergency SM is located. This implies that emergency SMS for roaming subscribers cannot be home routed while non-emergency SMS is typically home routed.

This feature is only supported for UE detectable emergency numbers.

# ZZ.2	Service assumptions

The following service assumptions are considered for supporting emergency SMS over NAS:

-	The UE that supports emergency SMS over NAS and detects emergency SMS over NAS based on used emergency numbers. Provisioning of emergency numbers and associated types as specified in clause 4.1 bullets 2a, and 2b) apply.

-	SM messages from the PSAP to the UE are routed like any other SM.

-	Serving network can support emergency SMS over NAS for own subscribers and inbound roamers.

-	SMS Centre in the serving network supports emergency SMS over NAS.

-	Emergency SMS over NAS applies to EPS, and 5GS. For EPS applies only for 3GPP accesses and for 5GS  for 3GPP and non-3GPP accesses.

-	There is no support for unauthenticated UEs and UEs in limited service state in this Release of the specification.

-	There is no indication from the serving PLMN to the UE that Emergency SMS in supported. After the UE sends Emergency SMS over NAS it can get informed about the  success or failure based on the SMS delivery notification using existing procedures.

# ZZ.3	Functional description

## ZZ.3.1	General

This clause lists the additional functionality that is required to support emergency SMS over NAS for roaming and non-roaming users.

High level description and signalling flows are described in TS 23.272 [x] Annex C for EPS and TS 23.502 [x] for 5GS.

## ZZ.3.2	UE

The UE that supports emergency SMS over NAS shall support the functionality listed below:

-	Identify the need to send Emergency SMS over NAS based on the use of emergency numbers.

-	Performs domain selection for Emergency SMS over NAS as described in TS 23.221 [x].

-	Replaces the SC Address that is configured with country code of the UEs serving PLMN and the emergency number e.g.  +32112.

NOTE 1: If UE cannot determine country from serving PLMN, UE can use methods out of scope of the present specification to determine the county.

NOTE 2: After the UE sends Emergency SMS over NAS it can get informed about the  success or failure based on the SMS delivery notification using existing procedures.

# * * * * End of changes * * * *