3GPP SA WG2 Meeting #165	 S2-240xxxx

# Hyderabad, India, 14-18 October 2024									                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.272 | CR | <CR#> | rev | <Rev#> | Current version: | <19.0.0> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Emergency SMS over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EmsSMSoNAS |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support emergency SMS over NAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the procedure to support emergency SMS over NAS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for emergency SMS over NAS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1, C.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

Annex C (normative):
SMS in MME

# C.1	General

SMS in MME is an architecture option primarily intended for networks that do not deploy GERAN or UTRAN. SMS in MME delivers SMS services over EPS NAS signalling for UE's requiring SMS services. SMS in MME architecture option enables support of SMS in the EPS via E-UTRAN without requiring deployment of 3GPP MSCs.

For SMS in MME the SMS service control is performed by the MME. The MME receives SMS subscriber data and the MME verifies whether the subscriber is permitted to obtain SMS services.

The SMS subscriber data received and used by the MME may be PS subscriber data (the MME directly obtains from HSS) or CS subscriber data (the MME obtains via an interworking function as described in clause C.5).

NOTE:	The use of SMS in MME by an operator that also deploys CS service on GERAN or UTRAN may lead to increased signalling traffic to the HSS.

MPS priority treatment of SMS via MME for a UE with an MPS subscription is supported as described TS 23.401 [2].

Support for Emergency SMS via MME is described in subclause C.x.

# >>>>NEXT CHANGE<<<<

# C.X	Support for Emergency SMS in MME

## C.x.1	General

Service assumptions for the support of Emergency SMS over NAS are described in TS 23.167 [x].

## C.x.2	Additional impacts in the UE and network entities to support Emergency SMS in MME

#### C.x.2.1	UE

The UE procedures for Emergency SMS in MME for UE supporting emergcny SMS over NAS are defined in TS 23.167 [x].

#### C.x.2.2	MME

In addition to what is described in clause C.4.2 MME needs to support the following features for Emergency SMS in MME:

-	if the SM contains emergency numbers in the MME acting as Short Message Relay (SM-RL) defined in TS 24.011 [28] sends the SM to a locally configured SMS-GMSC/IWMSC.

# C.X.3	Procedures for Emergency SMS in MME

## C.x.3.1	MO SMS

UE procedures for Emergency SMS in MME are defined in TS 23.167 [x].

An MME supporting emergency SMS in MME needs to be configured with mapping between the emergency numbers used in the country and SMS-GMSC/IWMC address.

The following sequence shows the procedure for UE sending for SMS in MME.

Figure C.8.1-1: MO emergency SMS

1.	If the UE supports emergency SMS over NAS and the UE detects the emergency number as defined in TS 23.167 [x] and includes in the SM the country code and emergency number in place of the SMSC address and as destination address the emergency number.

2.	The MME acting as SMS is configured to detect the SMSC address and emergency number in destination address and acting as SMS Relay follows the procedures procedures in TS 24.011 [28] and sends the SM to local SMS-GMSC/IWMSC .

## C.x.3.2	MT SMS

The MT SMS procedures defined in TS 23.040 [14] apply.

# >>>>END OF CHANGES<<<<