3GPP SA WG2 Meeting #165	 S2-240xxxx

# Hyderabad, India, 14-18 October 2024									                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | <CR#> | rev | <Rev#> | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Emergency SMS over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EmsSMSoNAS |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support emergency SMS over NAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the procedure to support emergency SMS over NAS in 5GS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for emergency SMS over NAS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.x, 5.16.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 4.4.x	Support for emergency SMS over NAS

Emergency SMS over NAS in 5GS is based on the architecture defined in clause 4.4.2. It can be supported over 3GPP and non-3GPP access.

There is no capability negotiation between the UE and AMF.  The UE that supports emergency SMS over NAS shall  replace the SMSC Address with the country code and the detected emergency number and the detected emergency number as destination address as defined in TS 23.167 [18].

For more details see, TS 23.167 [18].

# >>>>NEXT CHANGE<<<<

##### 5.16.3.8.1	UE originating SMS for IMS Capable UEs supporting SMS over IP

To allow for appropriate domain selection for SMS delivery, it should be possible to provision UEs with the following HPLMN operator preferences on how an IMS enabled UE is supposed to handle SMS services:

-	SMS is not to be invoked over IP networks: the UE does not attempt to deliver SMS over IP networks. The UE attempts to deliver SMS over NAS signalling.

-	SMS is preferred to be invoked over IP networks: the UE attempts to deliver SMS over IP networks. If delivery of SMS over IP networks is not available, the UE attempts to deliver SMS over NAS signalling.

In this release of the specification, there is only support for emergency SMS over NAS. If the UE supports emergency SMS over NAS and the UE detects that the SM destination address is an emergency number as defined in TS 23.167 [18], the UE shall attempt to deliver the SMS over NAS signalling.

# >>>>END OF CHANGES<<<<