3GPP SA WG2 Meeting #165	S2-2410102

Hyderabad, India, October 14 - 18, 2024													(was S2-240xxxx)

Source:	Ericsson

Title:	Conclusion on Key Issue #3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes the conclusion on Key Issue #3 for Ambient IoT.

1. Introduction

According to RAN SID (RP-240826), there are type 1 and type 2 devices to be studied in Rel-19:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

[Observation-1] Type 1 devices are very power limited. Compared with type 1 devices, type 2 devices have much higher power.

In TSG RAN#105, there was some discussion regarding harmonized air interface design, the discussion conclusion is included in RP-242360:

Conclusion: RAN reiterates that “study a harmonized framework with minimized differences (where necessary)” includes studying minimized differences (where necessary) between devices, e.g. for frequency synchronization between device 1/2a and device 2b, etc.

[Observation-2] TSG RAN clarifies the harmonized framework allow some differences between devices.

[Proposal-1] Based on [Observation-1] and [Observation-2], in SA2, the solution framework also allow some differences between type 1 and type 2 devices.  Studying actual differences in terms of system architecture and system functions needs cooperation with RAN WGs.

In TR 38.769, RAN2 is studying three use cases:

- Inventory-only;

- Inventory and Command;

- Command-only;

In our view, the three use cases in RAN2 can be mapped to the two procdeures (i.e., inventory, command) in SA2:

- Inventory-only: it is used in inventory procedure for all AIoT devices.

  - For type 1 devices, the devices simply respond the inventory request with their device IDs.

  - It is expected that there are no obstacles for type 2 devices to perform registration with the network, after responding with their device IDs. That is, there can be authentication and security parameter exchanges between the network and the devices, so that the network can allocate a CN Temp ID (similar as 5G-GUTI for UEs) towards the device. The type 2 device stores the security parameters and the CN Temp ID.

- Inventory and Command: according to TR 38.769, this is the baseline for device types 1 and 2, i.e. a concatenation of inventory and subsequent exchange of data between the AIoT device and the network. It is expected that type 1 devices are not able to optimise respective signalling on the AIoT air interface..

- Command-only: it is expected that there are no obstacles for type 2 devices to perform command-only procedures, omitting some steps as compared to “Inventory and Command”, see TR 38.769. The network use the allocated CN Temp ID to page the device and deliver the command to the device directly.

To support Command-only use case in RAN2, the network and the devices need to authenticate each other, exchange security parameters, and the network need to allocate a CN Temp ID towards the device. It is similar as the initial registration for the UEs, but can only be passively triggered by the network, as some type 2 devices (i.e., type 2a devices) are passive devices. Such “passive registration” can ensure the following command delivery to be executed in an efficient and secure way. However, it requires the “registered” devices to be able to write the received information (i.e., security parameters and CN Temp ID) into their NVMs.

[Proposal-2] It is proposed to differentiate the solutions for type 1 and type 2 devices. For type 1 devices, without being registered towards the network, the command delivery can only be achieved in an “inventory and command” manner. For type 2 devices, they can be registered towards the network in inventory, and thus the command delivery can be achieved in a “command-only” manner.

SA2 receives reply LS from RAN1 (R1-2407364), RAN1 answered the question from SA2 about whether Ambient IoT device can update is NVM:

- An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times.

- The power consumption during writing to the NVM is higher than during reading.

- RAN1 thinks that frequent or recurring writing to NVM should be avoided.

- RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

[Observation-3] Based on the answer from RAN1, the ability of writing to a NVM is possible, RAN1 has not studied the cost in terms of necessary energy for such operation.

SA2 also receives a reply LS from SA1 (S1-242527) to further clarify the security and privacy requirements for AIoT devices:

Question 2: Do the requirements listed in clause 5.2.6 of TS 22.369 for privacy protection apply to any Ambient IoT device that need to be supported in Rel-19? If these requirements do not apply to all Ambient IoT devices, does this imply that such devices would be restricted to certain deployment scenarios?

Answer 2: All functional service requirements in chapter 5 in TS 22.369 are valid for all categories.

Question 3: Does command / inventory response signaling, e.g. for sensor data collection or actuator control in TS 22.369 clause 4.3, need to generally be security protected (privacy, integrity and confidentiality protection)?

Answer 3: All functional service requirement in chapter 5 in TS 22.369, including the security and privacy requirements in chapter 5.2.6 are valid for all categories regardless of use case.

[Observation-4] All the security and privacy requirements are valid for all type of devices, regardless the use case and the deployment.

Based on [Proposal-2], [Observation-3] and [Observation-4], in our view:

- For the type 1 devices, the extreme low power consumption restrict their functionalities. They may not be able to write to their NVMs during the procedure. And thus, the “registration” towards the network is unlikely to take place. Given such preliminary feedback from RAN1, SA1 requirements on security and yet to be awaited feedback from SA3, type 1 devices with low energy storage capabilities and high cost restrictions might not be able to fulfill basic requirements for operation within a 5G system.

- For type 2 devices, with much higher power consumption, they can be registered towards the network passively, and store the security parameters and CN Temp ID into their NVMs.

[Proposal-3] It is proposed to capture the dependency with SA3 about the discussion above

Inventory Procedure:

In inventory procedure, after the AIOTF receives inventory request from the AF (optionally via NEF), it needs to distribute the request towards the proper readers. In topology 1, it needs to distribute towards proper A-IoT RAN nodes, and in topology 2, it needs to distribute towards proper NG-RAN nodes and further towards UE readers. The RAN reader and UE reader selection mechanism needs to be decided by SA2 and RAN3 jointly.

[Proposal-4] It is proposed to capture the RAN3 dependency in the EN.

For A-IoT paging for a group of devices, there are two options in TR 23.700-13 for the ID used:

- Option-1: Group ID

- Option-2: Device ID MASK information (including EPC with MASK).

Group ID requires additional information stored in NVM of the devices, besides the individual device ID. Device ID MASK information allows the network to reach a group of devices by masking some bits in the device ID, so that only the Device ID needs to be stored in the NVM of the devices. For type 1 devices, the less storage requirement would be more important. That is, Option-2 is recommended for type 1 devices.

On the other hand, the group ID bring more flexibility than the MASK. Using the MASK, the group size needs to be pre-determined. For example, if the AF uses the last 4 bits for a group of devices in the planning phase. In Device ID MASK information, the last 4 bits are masked. But after running for a period, there need to be the 17th device to be deployed, the MASK adjustment (e.g., needs to mask the last 5 bits) of this group may cause conflicts (e.g., some unintended devices respond the A-IoT paging due to the new MASK). For flexibility consideration, Option-1 is recommended for type 2 devices.

And for both options, it is also up to SA3 to determine whether it will cause security/privacy concerns using Group ID and/or Device ID MASK information.

[Proposal-5] It is proposed to conclude the solutions discussed above, and capture the SA3 dependency in the EN.

In inventory procedure, after the reader triggers A-IoT paging, the devices need to respond their device IDs or truncated device IDs, if they are within the group or match the device MASK information. Following the existing solutions in 5GS, the device ID needs to be concealed when there’s no mutual authentication between the network and the device. Are Ambient IoT devices able to perform the concealment, especially for type 1 devices? If not, is there a secured way to report the device ID towards the network? It is up to SA3 to determine.

[Proposal-6] It is proposed to capture the SA3 dependency in the EN.

Command Procedure:

For command procedure, AF request network to deliver the command to one or multiple devices (in an area).

As discussed above, if the device can be registered to the network, the CN Temp ID can be used to reach the device and have the command delivery in “command-only” manner. There is no specific security/privacy conern.

Otherwise, the device ID needs to be used  to reach the device and have the comment delivery in “inventory-command” manner. Different from inventory, the individual device ID is used directly without MASK information this time. There could be stronger security/privacy conern in this case. It is possible that AIOTF add several-bit MASK intendedly to reduce the security/privacy concern. Anyway, it is up to SA3 to determine.

[Proposal-7] It is proposed to capture the SA3 dependency in the EN.

In clause 5.2.3 of TS 22.369, SA1 put clear requirements on (temporary) disable, enable and permanent disable as a part of the management requirements:

The 5G system shall support a mechanism to:

- 	disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals

- 	enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently disabled to transmit RF signals

Based on operator policy, the 5G system shall provide a suitable mechanism to permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals. Subject to operator policy and regulatory requirements, the 5G system shall support suitable mechanisms for the Ambient IoT device to move between one or more networks and countries.

So far, no solutions in TR 23.700-13 have addressed such aspect very well. There are some solutions discussed the disable from network point of view (i.e., network should not accept the information from the device if the device is disabled). There are no complete solution to clarify the handling for enable/(temporary) disable/permanent disable.

[Proposal-8] It is FFS about the solution of enable/(temporary) disable/permanent disable AIoT devices.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.0.0:

* * * Start of Change (ALL TEXTS ARE NEW) * * * *

## 8.X	Conclusion on Key Issue #3

Editor's note:	For type 1 devices, it is up to SA3 to determine whether they can fulfill security and privacy requirements defined by SA1. SA2 will make alignment with SA3 decisions if needed.

The following aspects and principles for Ambient IoT services are agreed for the conclusion on the support of Ambient IoT Services, which apply to both topology 1 and topology 2:

-	Inventory and Command are supported.

-	For inventory, AIoT devices respond their device IDs towards the network. For type 2 devices, they can be registered towards the network. During registration, CN Temp ID of the device can be allocated by the network.

Editor's note:	It is up to SA3 to determine whether the devices send concealed device IDs. And if not, it is up to SA3 do determine whether and how the privacy concerns can be resolved.

-	For reader selection, it is performed by both CN and RAN (A-IoT RAN and NG-RAN) together.

Editor's note:	It requires colaboration with RAN3 to determine how the readers are selected for both Topology 1 and Topology 2.

-	For A-IoT paging for inventory, to reach a group of AIoT devices, device ID MASK information is used for type 1 devices, and Group ID is used for type 2 devices.

Editor's note:	It is up to SA3 to determine whether there are security/privacy concerns when using the device ID MASK information and Group ID in A-IoT paging. And if there are, it’s up to SA3 to determine how to resolve them.

-	For command, AIoT devices execute the command and respond the results towards the network. For type 1 devices, the command delivery is performed in "Inventory and Command" manner. For type 2 devices, the command delivery can be optimized to "Command-only" manner.

-	For A-IoT paging, to reach one or more specific AIoT devices. For type 1 devices, device IDs are used. For type 2 devices, the allocated CN Temp IDs of the devices are used.

Editor's note:	It is up to SA3 to determine whether there are security/privacy concerns when using the device IDs in A-IoT paging for type 1 devices. And if there are, it’s up to SA3 to determine how to resolve them.

Editor's note:	It is FFS about the solution of enable/(temporary) disable/permanent disable AIoT devices.

* * * End of Change * * * *