3GPP SA WG2 Meeting #165	S2-2410103

Hyderabad, India, October 14 - 18, 2024													(was S2-240xxxx)

Source:	Ericsson

Title:	Conclusion on Key Issue #1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes the conclusion on Key Issue #1 for Ambient IoT.

1. Introduction

In SA2#164, it has been agreed that for topology 1, a new core network function is introduced to support Ambient IoT.

Regarding the name of the new NF, there are different names in different solutions, e.g. Ambient IoT NF, AIOT NF, AIOTF, AIoTF, AIoTMF, AIoTCF, AIoT Controller, etc. In our view, AIOTF (or AIoTF) is more general from naming point of view as a new NF in 5GS. And so far, in TS 23.501, the abbreviation names of the NFs are all capitalized.

[Proposal-1] It is proposed to name this new NF as AIOTF (Ambient IoT Function), and clarify the function of AIOTF is to provide AIoT services (e.g., inventory and command) to the AF.

According to RAN TR 38.769 v1.0.0, it is A-IoT RAN which hosts the functions for AIoT as part of the functional split between RAN and CN; while for topology 2 the AIoT RAN is an AIoT enabled gNB interfacing a UE Reader, the nature of AIoT RAN nodes for topology 1 is still FFS. Also the nature of the XX interface between the AIoT RAN and and assumed AIoT CN is FFS and stated to be dependend on SA2 decisions.

In contrast to RAN TR 38.769, different solutions in SA2 TR 23.700-13 uses different terms for the network entity interfacing the AIoT enabled 5GC: A-IoT RAN, RAN Reader, A-RAN, or BS reader, etc.

For the 5GC to interact with A-IoT RAN, there are some options discussed in the TR 23.700-13 from system architecture perspective:

- Option-1: AMF with integrated AIOTF interact with AIoT RAN

- Option-2: AMF interact with AIoT RAN

- Option-3: AIOTF interact with AIoT RAN

Considering the dimensioning of Ambient IoT is quite different from the dimensioning of other use cases (e.g., MBB, etc.), there could be different scalability needs. And also to be able to separate the traffic for Ambient IoT and the one for other use cases effectively, it is proposed support direct connectivity of the AIOTF with A-IoT RAN (i.e., Option-3 above).

[Proposal-2] It is proposed to support direct connectivity of the AIOTF with A-IoT RAN in topology 1 via XX, while the nature of the XX interface is to be determined by RAN3.

For the existing 5GC functional entities which need to be involved, it is straight forward to have:

- NEF: Provide service exposure function for Ambient IoT services

- NRF: Stores the NF profile for AIOTF and provides service discovery for AIOTF

- CHF: Provide charging function for Ambient IoT services.

- UDM/UDR: Store device subscription information, if required.

For UDM/UDR, AUSF and external AAA server, it depends on the business model discussion in SA, SA1 and SA3. In the LS from SA to SA1 (SP-241419), SA provides more background information about the business model and lifecycle management.

Based on the current stage 1 requirement, it is MNO managed device subscription model.

[Proposal-3] It is proposed to capture the impacts on NEF, NRF, CHF, UDM

In the conclusion, there is an EN left on topology 2:

Editor's note:	Whether the new core network function also applies to topology 2 is FFS.

In topology 2, a UE reader is served by an AIoT enabled gNB. AIOTF is still needed to serve the AIoT service requests from the AF (optionally via NEF). The device NAS layer is still needed between this new NF and AIoT devices. That is, the introduction of the new AIOTF applies to topology 2 as well.

[Proposal-4] It is proposed to resolve the EN, and clarify the introduction of the new AIOTF applies to topology 2 as well.

In topology 2, among multiple solutions, in general there are 2 options on how to deliver the device messsage from AF towards UE reader: via control plane or via user plane. If the device message is delivered over control plane, from CN’s perspective, CN can deliver it in UE NAS message (i.e., UE NAS contained in an RRC message) or request NG-RAN to deliver it via RRC (i.e., device NAS/A-IoT upper layer information contained in an RRC message).

Those 3 options are also discussed in RAN3:

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Between RRC based solution and NAS base solution, system architecture wise, there are no significant differences between them. From CN point of view, the only difference is on device message delivery:

- In NAS based solution, the device message is embeded within AIOT UE NAS message. The protocol stack can be depicted as below:

- In RRC based solution, the device message is delivered over XXAP towards NG-RAN, and then over RRC towards UE. The protocol stack can be depicted as below:

Comparing those two options, RRC based solution brings less overhead from two aspects:

- AMF is not involved in the device message delivery (could be involved when paging of the UE Reader is needed).

- No UE NAS overhead when delivering device message.

Architecture wise, RRC based solution is most likely aligned with topology 1 Most likely alignment with topology 1. That is, CN interacts with AIoT RAN node function to request Common reader function to for inventory and command service requests, according to the system architecture definition of A-IoT RAN.

From SA2 point of view, it might look as if the RRC based solution is more efficient and architecture aligned. However, it also appears that there are additional aspects (which significance SA2 cannot judge) to be studied and concluded in RAN WGs.

For UP based solution, it is a bit different from RRC based solution and NAS based solution, which are CP based solution. The device message is delivered over a UE AIoT Layer between AIOTF and UE, which is over PDU Layer towards UE. The UE AIoT Layer can be further split into R-AP and SBI. The protocol stack can be depicted as below:

For UP based solution, the device message is transparent delivered via NG-RAN and CN. To ensure the radio resource is managed by NG-RAN, some interactions between UE and NG-RAN may be required.

[Proposal-5] It is proposed to cooperate with RAN2/RAN3 to determine whether RRC based solution, NAS based solution, and/or UP based solution shall be adopted.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.0.0:

* * * Start of Change * * * *

## 8.1	Conclusion on Key Issue #1

### 8.1.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function, Ambient IoT Function (AIOTF), is introduced to support Ambient IoT, which provides AIoT services (e.g., inventory, command) to the AF.

-	The AIOTF interacts with A-IoT RAN via XX.

Editor’s note: Cooperation with RAN3 is required to decide the XX interface between A-IoT RAN and AIOTF, while the name and the nature of XX is to be determined by RAN3.

-	The NEF exposes AIoT services to the AF outside trusted domain.

-	The NRF stores the NF profile for AIOTF and provides service discovery for AIOTF.

-	The CHF provides charging function for Ambient IoT services.

-	The UDM/UDR stores device subscription information, if required.

* * * Next of Change * * * *

### 8.1.X	Architecture to Support Topology 2

At least the following principles are agreed for the architecture to support topology 2:

-	The AIOTF applies to topology 2.

-	NG-RAN is responsible for radio resource management towards UE for radio resources between UE reader and AIoT devices.

Editor’s note: Cooperation with RAN2/RAN3 is required to determine whether to adopt RRC based solution, NAS based solution, and/or UP based solution.

* * * End of Change * * * *