3GPP SA WG2 Meeting #165	S2-2410104

Hyderabad, India, October 14 - 18, 2024													(was S2-240xxxx)

Source:	Ericsson

Title:	Conclusion on Key Issue #2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes the conclusion on Key Issue #2 for Ambient IoT.

1. Introduction

According to the definition in TS 22.261:

Ambient IoT device: An ambient power-enabled Internet of Things device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g., using a capacitor).

……

IoT device: a type of UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of UEs.

[Observation-1] An Ambient IoT device is an IoT device, which is a type of UE.

[Proposal-1] As the formats of identifiers in 5GS are quite flexible, it is proposed to align the Ambient IoT device ID definition with the identifiers of UE. The format of the identifiers can be adapted to fit the nature and the usage of Ambient IoT devices. Some identifiers may also not be applicable.

In 5GS, there are the following identifiers which are described in TS 23.501 [4]:

-	Subscription Permanent Identifier (SUPI).

-	Subscription Concealed Identifier (SUCI).

-	Permanent Equipment Identifier (PEI).

-	5G Globally Unique Temporary Identifier (5G-GUTI).

-	Generic Public Subscription Identifier (GPSI).

Internal-Group Identifiers and External-Group Identifiers are used to identify a group of UEs.

The different identifiers in 5GS are used for different purposes. Depending on the use cases and required functionality for supporting AIoT, different AIoT device IDs should be introduced.

In SA2#164, it has been agreed as an interim conclusion that an Ambient IoT Device Identifier can be assigned by an operator or by a third party, with an EN as below

Editor's note:	How it is used is FFS.

Even though it hasn’t been agreed on how it is used, as a permanent device identifier, the Ambient IoT Device Identifier can refer to SUPI, SUCI and GPSI of AIoT device.

Regarding SUPI, 5GS allows different SUPI types to be used, e.g. IMSI, NSI, GCI, GLI. The SUPI can be in different formats depending on the SUPI type, while a NAI based format with username@realm can be used and a more condensed encoding can be beneficial.

[Proposal-2] For Ambient IoT device, a new SUPI type can be introduced, which allows the the AIoT Device ID assigned by operator or by a third party (e.g., EPC used in RFID).

Regarding GPSI,  it is defined as an External Identifier and have the form "<Local Identifier>@<Domain Identifier>". It could be an option to introduce EPC in the <Local Identifier>, while the <Domain Identifier> can be the domain name of the AIoT service provider.

[Proposal-3] For Ambient IoT device, GPSI can be defined to include EPC.

Regarding SUCI, it is a concealed SUPI, which is used to identify an AIoT device without being authenticated. The HNI part of the SUCI can include MCC and MNC for a PLMN and also a NID for an SNPN, and with a Routing Indicator allows to route to the proper AUSF and UDM instances capable of handling authentication.

It is up to SA3 to determine whether the device can respond concealed device ID towards the network. And if not, it is also up to SA3 to handle the security and privacy aspects of the device ID.

[Proposal-4] Add a NOTE to capture the the security and privacy aspects are to be handled by SA3.

[Proposal-5] Based on [Proposal-2] to [Proposal-4], the EN can be resolved, as the usage of the device IDs have been analyzed.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.0.0:

* * * Start of Change * * * *

## 8.2	Interim Conclusion on Key Issue #2

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

An ambient IoT Device Identifier can be assigned by an operator or by a third party.

A new SUPI type can be introduced for Ambient IoT devices, which allows the the AIoT Device ID assigned by operator or by a third party (e.g., EPC used in RFID).

For Ambient IoT devices, GPSI can be defined to include EPC.

NOTE:	The security and privacy aspects will be handled by SA3.

* * * End of Change * * * *