SA WG2 Meeting #S2-165	S2-2410176

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5734 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General description on Identifying non-3gpp device connecting behind of UE or 5G RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_UIA_ARC conclusion for KI#4 and the UIA_ARC WID, this CR proposes to add the new clause on “Identifying non-3gpp device connecting behind of UE or 5G RG” to 23.501 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause, Identifying non-3gpp device connecting behind of UE or 5G RG and new clause for non-3GPP device identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature UIA_ARC is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.X (new), 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.9.X	Non-3GPP Device Identifier

The non-3GPP Device Identifier (N3DID) is an identifier of a Non-3GPP Device connecting behind a UE or 5G-RG in the PLMN. 5GC is able to associate the traffic from or to non-3GPP device identified by a non-3GPP Device Identifier and ensure proper QoS handling to the traffic. The usage of the non-3GPP Device Identifier is specified in the clause 5.X.

The N3DID is in generic string format.

NOTE 1:	The allocation of the Device Identifiers to non-3GPP devices is out of scope of 3GPP.

NOTE 2:	This identifier is not intended to be used as a SUPI for Non-3GPP device.

Next CHANGE

## 5.X	Identifying non-3gpp device connecting behind of UE or 5G RG

### 5.X.1	Overview

This feature enable the 5GC to identify the individual non-3GPP devices connecting behind a UE or 5G-RG and provide QoS differentiation for the traffic associated with individual non-3GPP devices.

For the non-3GPP devices connecting behind a 5G-RG, it is specified in the TS 23.316 [84].

For each non-3GPP device connecting behind a UE that requires QoS differentiation, there may be a corresponding permanent non-3GPP Device Identifier. The non-3GPP Device Identifier is a generic string. The UE binds the non-3GPP Device Identifier to a non-3GPP device connecting behind a UE and authenticates it.

NOTE 1:	The allocation and binding of non-3GPP Device Identifiers to non-3GPP devices is out of scope of 3GPP.

NOTE 2:	The Authentication for non-3GPP device is performed by UE, which is not in the scope of 3GPP. The connection between non-3GPP devices and UE is out of scope of 3GPP.

NOTE 3:	The UE or 5G-RG support the traffic from non-3GPP devices behind of a UE or 5G-RG that are not associated with a non-3GPP Device Identifier.

### 5.X.2	Session management enhancement for identifying non-3gpp device connecting behind of UE

If the non-3GPP device requires QoS differentiation, the UE may send the non-3gpp Device Identifier and user plane address corresponding to the non-3GPP device to the SMF in NAS-SM message (i.e. in the PDU session establishment or modification message) or DHCPv6 message. The SMF forward the non-3gpp Device Identifier and user plane address to the PCF for policy decision.

NOTE:	When a non-3GPP device connects to the UE/5G-RG, the UE/5G-RG can, based on locally configured policies, bind the traffic from the non-3GPP device to a Device Identifier.

When using an IP PDU Session Type, the user plane address corresponding to the non-3GPP Device Identifier is the UE IP Address (es) / Port Number(s) that carry the non-3GPP device's traffic. When using an Ethernet PDU Session Type, the user plane address of the non-3GPP device is the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device's traffic.

### 5.X.3	QoS differentiation for traffic associated with Non-3GPP Device Identifier

QoS differentiation can be performed within a PDU Session (using different QoS Flows) or between PDU Sessions. AF may request QoS requirement as per clause 4.15.6.6 of TS 23.502 [3] for the corresponding non-3GPP device Identifier.

When the SMF receives the non-3gpp Device Identifier and user plane address corresponding to the non-3GPP device, it forward them to the PCF. The PCF takes it into account for policy decisions based on the Device Identifier information retrieved from the UDR or local policy.

The details of QoS differentiation and policy control for non-3gpp Device Identifier is defined in TS 23.503 [45] clause 6.1.3.X

### 5.X.4	Non-3GPP Device information Provisioning

An AF can request provisioning of non-3gpp Device Identifiers and their corresponding QoS/Policies parameters associated with a UE subscription as described in clause 4.15.6 of TS 23.502 [3].

The AF may be in trust or un-trust domain.

END OF CHANGES