SA WG2 Meeting #S2-165	S2-2410184

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5738 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Additional ULI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_VMR2 conclusion (clause 8.5 of 23.700-06.), there are three options for the Additional ULI, which will be determined in normative phase: - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. In the MBSR, the 'Additional ULI' is the ULI of the IAB-UE and is encoded as TAC/Cell ID. To keep the similar principles, option 2 is not appropriate. Other options can be FFS.. This CR proposes to add the Support of Additional ULI to 23.501 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Support of Additional ULI for MWAB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Support of Additional ULI for MWAB is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.49.4	Support of Additional ULI

The TAC and cell ID broadcasted by the MWAB cell(s) are configured and reconfigured as described in clause 5.49.2.

When a UE is served by a MWAB cell, the MWAB-gNB may provide 'Additional ULI' in addition to User Location Information, in N2 messages to AMF.

Editor's note:	Whether the Additional ULI is mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM, or TAC/Cell ID of the cell serving the MWAB-UE in other PLMN is FFS.

The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

When the AMF provides user location information to other NFs (e.g. LMF as specified in clause 5.19 of TS 23.273 [87]) for a UE connected via MWAB cell, the AMF may also send the Additional ULI received via N2 messages.

END OF CHANGES