# 3GPP TSG-SA WG2 Meeting #165	S2-2410210

# 14 - 18 Oct 2024, Hyderabad, India

Title:	[Draft] Reply LS on AIML data collection

Response to:	LS RP-242389 (S2-2409600) LS on AIML data collection

Release:	Rel-19

# Work Item:	     NR_AIML_Air

Source:	WG SA2

To:	TSG RAN

Cc:	TSG SA, WG SA3, WG SA5, WG RAN1, WG RAN2, WG RAN3, WG CT1

Contact Person:	Ulises Olvera-Hernandez

E-mail Address:	ulises.olvera-hernandez@InterDigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank TSG RAN for the RP-242389 / S2-2409600 LS on AIML data collection. SA2 have discussed it, and SA2 would like to provide the following response and feedback:

On the feasibility of solution option 1b in terms of MNO full controllability and full visibility of standardized data without the need of SLA

According to RAN2, option 1b refers to a scenario where “the UE collects training data and it transfers it to a data collection server, for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training, to an OTT server (outside the MNO)”.

With option 1b, the first termination entity may be inside the MNO or outside the MNO domain, and it could be that the entity receiving the collected data does not belong to the MNO, which prevents any degree of controllability on the data collection process and any degree of visibility on the collected data unless an SLA exists between MNO and the owner of the OTT server.

An SLA between the MNO and the owner of the OTT server could provide a limited degree of control and visibility (e.g. via using APIs, Event IDs and Event Filters Supported by Data Collection AF). However, even with an SLA in place, option 1b would not allow dynamic controllability on the data collection process, e.g. when to transmit the data, and adaptability to changing network conditions or changing data collection configurations.

A network may support multiple AI/ML related features and functions simultaneously. In case of solution 1b, the network cannot harmonize these simultaneously ongoing activities as they are triggered and controlled by the external OTT server, and the configuration is Application specific (i.e., per Application ID). The lack of harmonization may result in network performance issues or conflicting configurations. Moreover, in the case of end-to-end encryption or data pe-processing at Data Collection AF outside MNO domain, the MNO has no visibility or limited visibility on the data that is transmitted from the UE to the server.

Option 1b does not allow full controllability on the data collection process nor full visibility on the collected data.

On the feasibility of solution option 2 and 3 in terms of MNO full controllability and full visibility of standardized data without the need of SLA

On the contrary, option 2, which has a first termination entity in the CN, allows visibility of the collected data at least on the CN side. Controllability can also be achieved e.g. via CP signalling between the entity in the CN and the UE (via RAN) or between the entity in the CN and the RAN. The data collection path between the UE and the first termination entity in the CN can be via the user plane or via the control plane.

The exposure of the collected data from the first termination entity can be achieved via NEF APIs available to the OTT server. The authorization of the NEF API invocations would be based on SLA., However the SLA described here would only be an SLA used for exposing the collected data, not for the step of collecting the data.

It should be noted that, with option 3, the data path is UE  gNB  OAM. Given that gNB and OAM are in MNO domain, full visibility of standardized data is also achieved.

Option 2 and option 3 allow full controllability on the data collection process and full visibility on the collected data.

On the feasibility of supporting UP solution/tunnel in option 2 and 3

It is our understanding that relying only on a UP solution or only on a CP solution may not fit all use cases, because while some data collection might require larger data volumes. some data collection may be small enough that they can be provided by CP. It should also be understood that CP solution and a UP solution are not mutually exclusive and may co-exist. Configuration for data collection will be sent over the CP from an entity in the Core Network to RAN, while data collected by the UE can be sent over UP. Note that data can also be sent over CP for small amount of data, so a solution with CP for control of data collection process and collection of small amounts of data, and UP for collection of large amounts of data would also work.

The first termination entity in the Core Network (i.e. a new NF or enhanced existing NF), as per option 2, would need to be decided by SA2, and may depend on the use cases from RAN. Different flavours are possible for the UP approach, depending on the termination point of data, but also depending on how control messages are exchanged (either via CP or UP). SA2 could look into detailed solutions if and when option 2 is selected by RAN.

A UP approach is feasible and can be developed by SA2 should RAN conclude on option 2. The CP approach and UP approaches are not mutually exclusive. The two approaches can coexist for the same use case, with configuration of data collection via CP, and data transfer either via CP or UP depending on the volume of data to be collected.

Note SA2 also does not see any issue with developing a UP solution for option 3 in coordination with SA5 should RAN decide that this is the way to go for UE data collection for UE side model training.

2. Actions:

To TSG RAN

ACTION:  SA2 kindly asks TSG RAN to take the above information into account.

3. Date of Next SA2 Meeting:

SA2#166			November 18th – 22nd, 2024			Orlando, US

SA2#166-AH-e			January 20th – 24th, 2025			Online