3GPP TSG SA WG2 #165	            S2-2410217

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1480 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#8 conclusion in TR 23.700-77, the architecture for avatar communications needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of IMS DC based avatar communication is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS DC based avatar communication cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

AC.X	Support of Avatar Communication

AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) and the associated metadata are provided to the UE, so that the user of the UE selects one Avatar ID among the received Avatar IDs that are allowed to the user. The Avatar ID List is provided from the Base Avatar Repository (BAR) to DCSF, and is transferred to the UE through bootstrap data channel.

-	Avatar ID List is the list of Avatar ID(s) that are identifying the Avatar Representation allowed for the user, and the associated metadata (e.g. the data required for the user to select one Avatar to use in a call such as thumbnails, location, time, country, etc).

Editor's note:	It is FFS whether the Avatar ID List can be stored in UDM+HSS for the operator specific avatars.

After selection of one Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar ID is provided from the BAR to MF, and it is transferred to the local UE and/or to the remote UE according to the rendering mode.

-	Avatar Representation is the data which is generated from the inputs such as captured data from camera and other sensor information, and it is used as a base of Avatar for rendering.

The avatar media is rendered by processing the Avatar Representation with the animation data. The avatar media rendering process is performed by the local UE, the remote UE, and/or the network according to the rendering mode.

-	Rendered Avatar media is generated by processing the Avatar Representation with the animation data (e.g. audio, video, and/or metadata such as action and facial expression data).

The details of enhanced IMS data channel architecture and procedures are described in following sub-clauses.

AC.X.2	Architecture

Figure AC.X.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Represenation and its associated Avatar ID. One ore more Avatar Representation(s) is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR provides Avatar ID List to the DCSF, upon receiving requests from DCSF. The Avatar ID List is the list of Avatar ID(s) and the associated metadata (e.g. the data required for the user to select one Avatar to use in a call) that is allowed to use for the user.

-	The BAR sends Avatar Representation to the MF, upon receiving requests from DCSF. The Avatar Representation is the data which is generated from the inputs such as captured data from camera and other sensor information, and it is used as a base of Avatar for rendering.

-	The BAR is connected to DCSF with DCx interface, and to MF with MDCx interface. Based on deployment, the BAR may be co-located with DC Application Server, and may use DC3 and MDC2 interfaces in this case.

DCSF:

-	The DCSF receives event control notifications from the IMS AS with the indication that it is allowed to access to BAR for this user and the requests of delivery of Avatar ID List and/or Avatar Representation.

-	Upon receving the indication of BAR access allowance and the requests of delivery of Avatar ID List from the IMS AS, the DCSF retrieves the Avatar ID List from the BAR via DCx and sends it to the UE through bootstrap data channel. If the BAR is co-located with DC Application Server, the DCSF may use DC3 for Avatar ID List retrieval.

-	The DCSF requests Avatar Representation(s) to the BAR with the Avatar ID(s) requested from the allowed UE and the MDCx resource information of the MF.

IMS AS:

-	Based on the data channel media description information and the user subscription data, the IMS AS validates whether the user of the UE is allowed to use the data channel based avatar communication and whether it is allowed to access to the BAR for this user of the UE. If allowed, the IMS AS indicates to the DCSF about the BAR access allowance and the requests of delivery of Avatar ID List and/or Avatar Representation.

MF:

-	The MF receives the Avatar ID List from the DCSF via MDC1 and sends it to the UE through bootstrap data channel.

-	The MF receives the Avatar Representation from the BAR via MDCx. If the BAR is co-located with DC Application Server, the MF receive the Avatar Representation via MDC2.

-	The MF supports Avatar media processing. In case of UE based rendering mode, the MF transfers the Avatar Representation to the UE. In case of Network based rendering mode, the MF processes the Avatar Representation with the animation data (e.g. audio, video, and/or metadata such as action and facial expression data) to generate the avatar media. The MF transfers the rendered avatar media to the UE.

DC AS:

-	The DC AS provides the controls of Avatar media processing at MF in case of Network based rendering mode.

AC.X.2.1 BAR

Figure AC.X.2.1-1: Architecture to support Avatar communication

The following reference points are used and enhanced to support Avatar communication service over IMS data channel:

-	DCx: Reference point between the BAR and the DCSF for Avatar ID List transfer.

NOTE 1:	If the BAR is co-located in the DC AS, the service operations of DCx are performed via DC3.

-	MDC1: Reference point between the DCSF and the MF for Avatar ID List transfer.

-	MDCx: Reference point between the BAR and the MF for transmission of Avatar Representation.

NOTE 2:	If the BAR is co-located in DC AS, the data transferred via MDCx is transferred via MDC2

-	DC1: Reference point between the IMS AS and the DCSF for handling of Avatar ID List and Avatar Representation, and for Avatar media resource management.

-	DC2: Reference point between the IMS AS and the MF for Avatar media resource management.

-	N33/DC3: Reference point between the DC AS and the DCSF for handling Network based Avatar media processing in MF.

-	Mb: Reference point used for Avatar ID List, Avatar Representation and Avatar media transmission.

AC.X.3	Procedures

AC.X.3.1	Bootstrap DC setup for Avatar communication

Figure AC.X.3.1-1: Bootstrap DC setup signalling procedure for Avatar communication

Figure AC.X.3.1-1 depicts the Boostrap DC setup signalling procedure to download the Avatar data stored in the BAR to the UE which is allowed to use of data channel based avatar communication and also is allowed to access to the BAR via data channel network. This procedure is based on the Bootstrap Data Channel Setup Signalling in clause AC.7.1 with following additions and differences:

Editor's note:	The service operation between the DCSF and BAR will be defined.

- In SDP offer for bootstrap DC, the UE#1 includes the media description for avatar communication in SDP offer for bootstrap DC. If the UE#1 wants to establish an avatar communication over data channel, the avatar communication request, and UE avatar media processing capability are indicated in the media description for avatar communication.

- Based on the avatar communication request included in the media description in SDP offer, the IMS AS checks the user subscription data and validates whether the user of the UE is allowed to use the data channel based avatar communication and whether the network is allowed to the BAR for this user of the UE. If allowed, the IMS AS determines to notify to the DCSF.

- IMS AS includes indicates in Nimsas_SessionEventControl_Notify message that the user of the UE is allowed to use the data channel based avatar communication and whether the network is allowed to the BAR. When IMS AS derives the media specification of the MediaInfoList from the media description included in SDP offer, the media description for avatar communication e.g. avatar communication request, and UE avatar media processing capability are also included to the media specification.

- DC control policy decision and DC media information creation

- Since the bootstrap data channel is offered and the avatar communication is requested with indication that the user of the UE is allowed to use the data channel based avatar communication and whether the network is allowed, the DCSF, based on its policies, requests the Avatar ID List for the user of the UE#1 by providing IMPU, and requests to reserve originating side MDCx resource and terminating side MDCx resource to the BAR, which are used to receive UE request for Avatar Representation downloading from MF. The DCSF, based on its policies, reserves originating side MDC1 and terminating side MDC1 resource, which are used to receive UE request from Avatar ID List downloading from MF.

- BAR checks the existence of Avatar ID List associated with the received IMPU, and if it exists then the BAR reserves MDCx resource for originating side and terminating side.

- After successful MDCx resource reservation, the BAR responds to the DCSF with the Avatar ID List associated with the IMPU, and the result of MDCx resource reservation which includes the reserved MDCx resources.

- The DCSF stores the Avatar ID List and MDCx resources, and provides to IMS AS the replacement HTTP URL representing the Avatar ID List offered via MDCx interface and the MDCx resources created by BAR in the Nimsas_MediaControl_MediaInstruction.

- The IMS AS, based on local configuration, discovers or selects a MF which supports DC media function and avatar communication.

- The IMS AS instructs MF to allocate the required data channel media resources including the MDCx resources towards BAR, and MDC1 resources towards DCSF while providing the replacement HTTP URL representing the Avatar ID List offered via MDCx, and the MDCx resources reserved by BAR. The MF responses with the MDCx resources and MDC1 resources to the IMS AS.

- The IMS AS responds to the step 8 with the result of MDCx and MDC1 resource reservation which include the reserved resources.

- The DCSF provides the MDCx resources from MF to the BAR with the IMPU. The BAR stores the MDCx resources from MF, so the data path between the BAR via MDCx is set up.

- The DCSF stores the MDCx and MDC1 resources from MF and responds to the step 3.

  - Whether to use the avatar communication services over data channel, UE avatar media processing capability, and downloading of Avatar Representation from BAR through originating network are additionally negotiated with the terminating network and UE#2.

- Based on the received responses, the UE#1 is informed that the bootstrap data channel has been established and the avatar communication over data channel becomes available.

  - The UE#1 sends the request messages to download the Avatar ID List that is allowed to the UE#1 to the MF via bootstrap data channel. The MF replaces the root URL with the replacement URL received in step 10 and forwards the request messages to the DCSF. The DCSF provides the Avatar ID List to the UE#1 through MF.

- Subsequent procedure is performed.

  - The user of the UE#1 selects an Avatar to use in the avatar call with the UE#2, based on the metadata associated with each Avatar ID. The selection of an Avatar to use is conducted before and/or after application data channel setup for an application. When it is selected before application data channel setup, the UE#1 requests the Avatar Representation of the selected Avatar to the MF with the Avatar ID and the destination of the Avatar Representation downloading (i.e. local UE, remote UE, or both) via bootstrap data channel. The MF replaces the root URL with the replacement URL received in step 10 and forwards the request messages to the BAR. The BAR provides the Avatar Representation to the UE#1 and/or to the UE#2 based on the destination indicated in the requests.

The Avatar to be used in the avatar call with the UE#2 may be selected or re-selected after an application data channel setup for an avatar communication application, e.g. based on each avatar communication application specific configurations such as the limited rendering modes, the limited Avatars, the limited usage in certain location/country/time. In this case, the UE#1 sends the requests for downloading Avatar Representation through the application data channel.

# * * * * End of changes * * * *