3GPP TSG-WG SA2 Meeting #165	S2-2410239

Hyderabad, Oct 14 – 18, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#3 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#3.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.0.0.

FIRST CHANGE

## 8.3	Interim Conclusion on Key Issue #3

### 8.3.1	Information transfer and exposure

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices;

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

The following principles are agreed for both topologies:

-	Based on the AF request, the new core network function requests the AIoT Services operations (i.e., Inventory, Command) to the AIoT Reader.

-	New NEF services (e.g., Nnef_AIoTInventory, Nnef_AIoTCommand) will be defined.

-	New events (i.e., AIoT reader overload, reader-device interaction latency, availability of AIoT UE readers per location) to be exposed by NEF to external AFs for supporting AIoT services will be defined.

The following principles are agreed for specifically topology 1:

-	The AF requests one of the AIoT Service operations to the new core network function via NEF.

-	When an AF requests an AIoT service request, it may include location info, which can be used by the 5GC to select appropriate NFs and AIoT readers.

-	The new core network function requests one of the AIoT Service operations to the BS Reader via AMF.

-	The BS reader responds to the new core network function with the service operation result.

-	The new core network function responds to the AF via NEF with the service operation result.

The following principles are agreed for specifically topology 2:

-	When an AF requests an AIoT service request, it may include either location info or a UE reader ID, which can be used by the 5GC to select appropriate NFs and AIoT readers or to validate the requested UE reader ID.

-	The new core network function requests one of the AIoT Service operations to the UE Reader via AMF and gNB.

-	The UE reader(s) may be selected by the new core network function based on, e.g. the location information provided by the AF, UE authorization information, UE capability, UE related analytics, UE expected behaviours, Device capability.

NOTE : 	Whether the gNB selects the UE reader(s) indicated by the CN as they are, or selects some or none of them based on the radio conditions, and whether the gNB can select additional UE reader(s) will be further decided by RAN WG3.

-	The new core network function creates and maintains a binding table that indicates the association information between the AIoT devices and their UE reader(s) in order to avoid performing the UE selection procedure each time the 5GC receives the AIoT request for the same AIoT devices.

-	The UE reader(s) forwards the data read from the AIoT devices to the new core network function, over either CP or UP path. The UL data forwarding method is determined by the new core network function, considering the UE’s capability and the application requirements (e.g., aggregated data volume, latency, communication frequency)

-	The data read from the AIoT devices can be aggregated by the UE reader(s), the gNB and/or the new core network function before responding to the consumer AF.

-	The selected UE(s) is configured with the necessary information for communicating with the AIoT devices and the CN, including the UE role, UL data forwarding method, aggregation policy, target reporting destination.

Editor's note:	Additional conclusions are FFS.

END OF CHANGES