3GPP TSG-SA2 Meeting #165	S2-2410294

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024                               revision of S2-2407720

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0234 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Architecture and Local offloading management authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For KI1, the following principles are included in the conclusion for Session Breakout model: - I-SMF based approach shall be considered to reduce impact on central 5GC NFs - I-SMF selection shall be performed by AMF. I-SMF selection mechanism shall be enhanced to support I-SMF for local offloading management, e.g. I-SMF selection for local offloading management is based on the subscription data. - EASDF configuration and DNS message handling is performed by I-SMF (instead of SMF). - EASDF discovery and selection is performed by I-SMF (instead of SMF). When the I-SMF approach is used the SMF does not discover and select an EASDF. To address the above, this CR proposes a reference architecture for Local Offloading Management and procedures for local I-SMF insertion, which include Local Offloading Management authorization, local I-SMF selection, etc during PDU Session Establishment. Additionally, it is concluded that the existing mechanism shall be used for Distrubuted Anchor Point and Multiple PDU Session connectivity models. To address this, some clarifications are proposed. Update from the previous version is text for reference architecture in yellow highlight. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference architecture for Local Offloading Management scenario with referencing clause 5.32.2.2 Add procedures for PDU Session Establishment and EAS discovery for Local Offloading Management scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local Offloading Management scenario is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.x (new), 6.x.1 (new),6.x.2 (new), 6.x.2.1 (new), 6.x.2.2 (new), 6.x.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

The reference architecture for Local Offloading Management scenario is based on the non-roaming architecture with I-SMF insertion to the PDU Session with UL-CL/BP specified in clause 5.34.2.2 with an additional reference point between I-SMF and EASDF.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

>>>> Next Changes (all new text) <<<<

6.X	Support of the Local Offloading Management with reducing impact on centrally deployed SMF

6.X.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF, 5GS may support the Local Offloading Management, which enables to utilize a locally deployed SMF or local I-SMF for session management of the edge computing traffic toward the local part of DN.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode may be used to a locally deployed SMF to support Local Offloading Management.

In Session Breakout connectivity model (see clause 4.3), the UE establishes a PDU Session that supports Local Offloading Management with the insertion of a local I-SMF as depicted in Figure 4.2-x in clause 4.2. After establishing the PDU Session supporting Local offloading management, the UE can access EAS deployed in EHE in the local part of DN via the local PSA UPF managed by the local I-SMF. The local I-SMF supports the following functionalities for a given PDU Session that enables Local Offloading Management:

-	EASDF configuration and DNS message handling instead of SMF.

-	EASDF discovery and selection is performed by the local I-SMF instead of SMF.

-	Management and retrieval of EDI from NEF to configure EASDF.

NOTE 1:	The local I-SMF can be also preconfigured with the EDI.

-	Retrieval of the Local Offloading Management policy, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s), from PCF via SMF.

-	Management of AF provided traffic influence information provided from SMF.

Local I-SMF selection is performed by AMF, which considers the SMF selection subscription data, i.e., the Local Offloading Control allowed indication and the local offloading control service area, the area where the local offloading management is supported.

NOTE 2:	When the local I-SMF is inserted to support local offloading management, the SM NAS message is still terminated to the SMF.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

6.X.2	Procedure

6.X.2.1	General

This clause describes the authorization procedure of the Local Offloading Management with the insertion of local I-SMF and EAS discovery procedure supporting Local Offloading Management.

6.X.2.2	PDU Session establishment for supporting Local Offloading Management

Figure 6.X.2.2-1: Procedure for PDU Session establishment supporting HR-SBO in VPLMN

1.	During the Registration procedure, the AMF receives the Local Offloading Management allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	The PDU Session Establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3] is used with the following differences and additionals:

-	The V-SMF and V-UPF are replaced by local I-SMF and I-UPF, respectively

-	The H-SMF and H-UPF are replaced by SMF and UPF(PSA), respectively.

-	Only the S-NSSAI with the value defined by the serving PLMN is sent to the SMF.

-	The local I-SMF provides the supported DNAI list to the SMF, and then the SMF provides the DNAI(s) of interest for this PDU Session to the local I-SMF based on the DNAI list information received from the local I-SMF as defined in Figure 4.23.9.1-1 step 1.

-	If the AMF received in SMF Selection Subscription data from UDM the Local Offloading Management allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a local I-SMF supporting Local Offloading Management and sends a Local Offloading Management allowed indication to the SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

If the local I-SMF receives the Local Offloading Management allowed indication from AMF, the local I-SMF may:

-	select UL CL/BP UPF and L-PSA UPF based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL CL/BP UPF and L-PSA UPF can be co-located.

-	select an EASDF;

-	may obtain EASDF DNS security information based on local configuration or via interaction with EASDF; and

-	send the request for the establishment of the PDU Session supporting Local Offloading Management and EASDF information to the SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

SMF authorizes the request for Local Offloading Management based on SM subscription data (i.e. Local Offloading Management authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the Local Offloading Management is authorized, the SMF requests and retrieves the Local Offloading Management Policy from PCF by SM Policy Association Establishment/Modification with the Local Offloading Management support indication as indicated in clause 5.2.5.4.2 of TS 23.502 [3].

If Local Offloading Management is authorized for the PDU session, the SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following additional information:

-	optionally, Local Offloading Management Policy that may include FQDN range or IP range for the local part of DN via the local PSA UPF managed by the local I-SMF.

3.	The local I-SMF configures the EASDF with the DNS handling rules by using the Local Offloading Management Policy received from the SMF.

The local I-SMF configures the UL CL UPF and local PSA UPF selected in the step 2 to forward DNS messages EASDF.

4. 	EAS Discovery procedure with EASDF is performed as described in clause 6.X.2.3.

6.X.2.3	EAS discovery with EASDF for supporting Local Offloading Management

Figure 6.x.2.3-1: Procedure for EAS Discovery with EASDF for Local Offloading management scenario

1.	The DNS query sent by the UE reaches the UL CL UPF controlled by the local I-SMF selected in step 2 of clause 6.x.2.2. The UL CL UPF forwards it to Local PSA UPF then EASDF as described below.

a)	Based on local I-SMF instruction in step 3 of clause 6.7.2.2, the EASDF proceeds to step 12 of clause 6.2.3.2.2. The DNS query is sent to the DNS server address according to the DNS message handling rules provided by the local I-SMF or to the default DNS server configured in the EASDF. Upon receiving the DNS response, the procedure proceeds immediately to step 5.

Editor’s Note:	It is FFS how to deal with the DNS query that is not matched with the target traffic for the Local 			Offloading Management.

The rest of the procedure assumes the target FQDN of the DNS query is matched with the target traffic for the Local Offloading Management.

2.	The steps 8 to 15 of the procedure in the clause 6.2.3.2.2 by replacing SMF with local I-SMF.

3.	The local I-SMF selects UL CL/BP and local PSA based on the EASDF notification, EAS Deployment Information, Local Offloading Management policy information and UE location. The V-SMF may perform insertion or change of UL CL/BP and local PSA.

The local I -SMF configures the UL CL/BP and local PSA for the traffic to be offloaded to the local part of DN based on the Local Offloading Management policy information received from SMF.

The local I-SMF sets up user plane between the selected UL CL/BP in this step and RAN.

NOTE:	If the selected UL CL/BP and local PSA in this step is the UL CL/BP and PSA selected by local I-SMF in the step 2 of clause 6.7.2.2, the insertion of the UL CL/BP and local PSA will not be performed in this step.

4.	The steps 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF with local I-SMF.

5.	The EASDF sends the DNS Response to the UE.

>>>> End of Changes <<<<