SA WG2 Meeting #165	S2-2410300

14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1258 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN on Vertical Federated Learning procedures when NWDAF is VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84 and CR S2-2409416, which was Noted on SA2#164 but considered to be baseline of the discussion, this CR aims to specify the general VFL model training when NWDAF is VFL server. This CR aims to remove some Ens on Clause 6.2G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2G.2.1 This CR resolves following EN and specifies the detailed procedure between untrusted AF and NWDAF. Editor's note Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. Two procedures for VFL when AF is as Server and VFL when NWDAF is as Server are required as so far. If we have stable description on both two procedures in the future, we can decide whether to merge them into one procedure or not. Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. VFL model correlation ID is preferred because this ID combines distributed ML model. Editor's note: Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients. Sample information should be shared among VFL Server and Client. The corresponding description is added and the EN is removed. Clause 6.2G.2.X This part is based on S2-2409416. The CR resolves the following EN. Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. Since VFL Server determines to perform VFL training based on the operator’s policy as denoted in Note, no more interaction between consumer and VFL server is needed. Editor’s Note: How the VFL server and client register to the network, how to discover VFL server or VFL client is FFS. This is described in Clause 6.2H Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable. Two procedures are needed as described above. Editor's Note: Further extensions are needed to show when any of the VFL participants are untrusted AF(s). In this case the procedure below will contain a NEF, and how the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. NEF related procedure is added to Step 2. Editor’s Note: A possible need for a process of sample exchange between VFL server and VFL client is FFS. The process may be determined after agreement on sample and/or feature alignment. The samples to be used are determined on preparation procedure. After preparation procedure, sample alignment is not needed. Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS. VFL model training process is not Editor’s Note: Whether weight of feature is computed by VFL server is FFS. The weight of feature belongs to the VFL server internal logic. Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. After the VFL training process is terminated, the VFL Server may collect well-trained ML model from VFL Clients, or VFL Server and Client(s) locally store the well-trained model with the VFL model correlation ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL functionality when AF is VFL server is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 6.2G	Vertical Federated Learning

### 6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference.

### 6.2G.2	Procedures

#### 6.2G.2.1	General inference procedure for vertical federated learning

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

Editor's note:	Which service used between the consumer and the VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients.

Editor's note:	How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL client is the NWDAF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and sample information (e.g., UE ID).

If the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and sample information (e.g., UE ID). If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.
Before NEF forwards, NEF translate the Analytics ID, VFL correlation ID and sample information to external IDs.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

5.	Each VFL Client sends the intermediate local inference results to the VFL server. This message includes sample information. If untrusted AF is involved as the VFL Client, the AF send the message to NEF, and NEF forward the message to NWDAF. 
Before forwarding, NEF translate the external ID to internal ID.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computiong the VFL inference is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# >>>>Next CHANGES<<<<

### 6.2G.2.X General training procedure for Vertical Federated Learning when NWDAF is VFL Server

The figure 6.2G.2.X-1 below shows general training procedure for Vertical Federated Learning.

Figure 6.2G.2.X-1: General training procedure for Vertical Federated Learning when NWDAF is VFL Server

This below procedure aims for NWDAF based Vertical Federated Learning. The VFL training process may be conducted after VFL preparation procedure.

Editor’s Note: It’s FFS to determine the VFL preparation procedure.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: Whether and how to exchange feature between VFL server and VFL client is for FFS. The process may be determined after agreement on sample and/or feature alignment.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training round or there is a session concept.

Editor´s Note: It is FFS whether and how VFL server collects trained ML model from VFL clients after the VFL training process.

0.  The NWDAF acting as VFL Server and the respective NWDAFs or AFs acting as VFL Clients completed the VFL Training Preparation procedure.

1.	The NWDAF acting as VFL server selects the VFL clients that participate in VFL as described in the clause 6.2H.

NOTE: VFL Server determines to perform VFL training based on the operator’s policy.

2.	To start VFL training, VFL server sends a request to the selected VFL clients. The request includes VFL model correlation ID and sample information that will be used in the VFL training procedure. If untrusted AF is involved, the request goes to NEF, NEF forwards to the AF. Before NEF forwards, NEF translate the external and internal IDs.

Editor´s Note: Parameters to be provided in the request need to be added

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model based on its own data and reports intermediate training result of the local ML model to the VFL server.

Editor’s Note: Whether and how to involve local ML Model training information in the VFL training process is FFS; What parameter contains in local ML model training information is FFS.

Editor’s Note: The details on whether and how to support that VFL clients provide intermediate results to other VFL clients is FFS.

5.	The VFL server may collect the local data and generate its own local the intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the intermediate training result received in step 4 and label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process. The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: The flow should be updated to show the message exchange for VFL training termination.

The VFL training termination decision may be also made as follows:

- Based on the consumer request, the VFL server sends VFL status report to update the ML model metric to the consumer.

- The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

- Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

If the VFL server received a request from consumer or decide to stop the Federated Training process, steps 7 and 8 are skipped. If the VFL procedure continues, VFL server sends the backward local ML model training information (e.g. gradient information, loss information) to the VFL clients for next round of VFL training.

8.	Each VFL client computes gradient of its local model and updates its local ML model based on backward local ML model training information distributed by the VFL server at step 7. The NWDAF acting as VFL Server and each NWDAF or AF acting as VFL Client, stores the latest information about their locally trained Models.

Editor’s Note: It is FFS whether model storage after each training round is required.

The steps 4-8 should be repeated until the training termination condition is reached.

After the VFL training process is terminated, the VFL Server may collect well-trained ML model from VFL Clients, or VFL Server and Client(s) locally store the well-trained model with the VFL model correlation ID.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# >>>>END OF CHANGES<<<<