SA WG2 Meeting #S2-165	S2-2410304

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5755 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2, adding Policy control based on the Subscription and Energy Consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the KI#2 conclusion of 23.700-66 and EnergySys WID, this CR proposes to add new clause on Policy control based on the Subscription and Energy Consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Policy control based on the Subscription and Energy Consumption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KI#2 conclusion of EnergySys feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.4	Policy control  based on the Subscription and Energy Consumption

The UE may have an Energy saving subscription information (e.g. Energy credit). If the Energy consumption of the UE exceeds the Energy credit, the 5GC performs the energy saving strategies for the UE according to operator’s policy.

-	The UE has an Energy saving subscription information in the UDM/UDR. The Energy saving subscription may comprise Energy saving indication and Energy credit for the UE.

-	During PDU session establishment for the UE, the PCF subscribes with the EECF for the Energy Consumption status of the UE.

-	The EECF collects the Energy consumption information and calculates the Energy consumption for the UE as described in the clause 5.X.2.

-	When the Energy consumption of the UE exceeds the Energy credit in the UE Energy saving subscription data, the EECF notify the PCF the event.

Editor's note:	Whether the remained Energy credit is stored in the UDM/UDR is FFS.

-	When the PCF receives Energy consumption status notification, it may make policy decisions as defined in the TS 23.503 [45] based on operator policy.

END OF CHANGES