SA WG2 Meeting #S2-165	S2-2410306

14 - 18 October, 2024, Hyderabad, India	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5757 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3, UP path adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-9-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the KI#3 conclusion of 23.700-66 and EnergySys WID, the UP path of PDU session may be adjusted according to energy related information. This CR proposes to add a new clause on UP path adjustment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding UP path adjustment based on the Energy information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KI#2 Conclusion on NF discovery and selection of EnergySys feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.6	UP path adjustment

The SMF may select UPF (including PSA-UPF and I-UPF) with high energy efficiency in the User plane path for energy saving. The SMF may adjust the User plane path in the following case:

-	During PDU session establishment, SMF may discover and select UPF according to the Energy status information in the NF profile as described in clause 5.X.5.

-	When the I-UPF needs to be inserted, the SMF/I-SMF may discover and select I-UPF according to the Energy status information in the NF profile as described in clause 5.X.5.

-	According to the I-UPF Energy status information, the SMF can proactively replace the I-UPF with a more energy-efficient I-UPF.

Editor's note:	Additional case is FFS.

END OF CHANGES