SA WG2 Meeting #165	S2-2410307

14 - 18 October, 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1259 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General preparation procedure for Vertical Federated Learning. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the VFL preparation procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Clause 6.2G.2.X is added. VFL preparation procedure aims to algin samples between VFL Server and VFL Client(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL preparation functionality is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2G	Vertical Federated Learning

6.2G.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference and VFL preparation.

# >>>>Next CHANGES<<<<

6.2G.2.X	General preparation procedure of Vertical Federated Learning

Before VFL model training or VFL model inference procedure, VFL Preparation procedure is performed aiming to determines the samples that used for VFL model training and/or inference procedure.

Figure 6.2G.2.X-1 General preparation procedure for VFL

1. After the VFL Server discovers the VFL Client(s) as clause 6.2H, the VFL Server sends a request(s) to the VFL Client(s) to query the availability of samples. The request includes the candidate sample IDs (e.g., list of DNAI, NSSAI, SUPI, GPSI) and VFL model correlation IDs. 
If VFL Server is untrusted AF, the requests go to NEF, and NEF forwards the requests to the NWDAFs, based on the instruction by the AF. Before NEF forwards, NEF translates from external ID to internal ID (e.g., from GPSI to SUPI).
If VFL Server is NWDAF and VFL Client is untrusted AF, the requests go to NEF, and NEF forwards the requests to AF. Before NEF forwards, NEF translates from internal ID to external ID (e.g., from SUPI to GPSI).

2. Each VFL Client checks the availability of the data.

3. Each VFL Client responds to the availability of the data. If untrusted AF is involved, the NEF translates as Step 1.

4. The VFL Server demines the final list of data samples that will be used for the VFL process, taking the responses on step 3 into account.

# >>>>END OF CHANGES<<<<