SA WG2 Meeting #165	S2-2410339

Hyderabad, India, October 14 – 18, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1486 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Base Avatar Repository in IMS DC Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The paper is proposed to add Avatar Communication architecture based on Data Channel and Base Avatar Repository. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Avatar Communication architecture based on Data Channel and Base Avatar Repository. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Avatar Communication architecture is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

XX.1	Architecture

Figure XX.1-1 shows the Avatar communication archiecture based on IMS Data Channel and Base Avatar Repository (BAR).

Figure XX.1-1: Architecture for Avatar Communication Based on Data Channel and BAR

XX.2	Functional entities

XX.2.1	Base Avatar Repository (BAR)

BAR stores base Avatar Representation and its associated Avatar ID.

BAR is used by MF to retrieve the Avatar Representations used in Avatar Communication.

XX.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DCx:	Reference point between the DCSF and BAR.

The following reference points are defined for data channel media handling:

-	MDCx:	Reference point for transport of base Avatar representation between MF and BAR.

END OF CHANGES