3GPP TSG-SA WG2 Meeting #165	S2-2410360

# Hyderabad, India, October 14 - 18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5765 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support local services in 5G Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As RAN3 WG’s recommendation, for local services in 5G femto cell we can reuse LADN mechanisms. Accordingly the associated LADN information and S-NSSAI location availability information for local services should be provisioned to UE, which re-uses the UCU mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provisioning local services at Femto cell is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local services in 5G Femto cell are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.1, 5.50.2, 5.50.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.50	Support for 5G Femto

5.50.1	Overview

This clause provides an overview of 5GS functionalities and architecture to support 5G Femto deployments.

The existing CAG concept defined in the clause 5.30.3 for PNI-NPN is used for 5G Femto access control. The Femto cell may broadcast one or multiple CAG Identifiers as specified in the clause 5.30.3 if access control is needed.

For local services at 5G Femto cell(s), the existing LADN concept defined in the clause 5.6.5 and 5.6.5a is used and additionally in order to support cell granularity S-NSSAI location availability information concept defined in the clause 5.15.18 is used together. And, the existing Edge Computing concept defined in the clause 5.13 can be used also.

The enhancements enable the CAG owner or an authorized administrator to control/provision subscribers to allow accessing their 5G Femto cell(s) and local services at the Femto cell(s).

5.50.2	5G Femto architecture

For local services UPF can be collocated with 5G Femto cells(s) and should be managed by a dedicated SMF(s) and/or registered to NRF deployed in macro system. The dedicated SMF can be selected when the PDU Session is established at the 5G Femto cell(s).

5.50.3	5G Femto Information Provisioning

In 5G Femto, a CAG owner or an authorized administrator can control/provision subscribers to allow accessing 5G Femto cell(s) and local services at the Femto cell(s).

The provisioning of 5G Femto Information is defined in clause 4.15.6.2 and 4.15.6.3h of TS 23.502 [3].

For access control to the 5G Femto cells(s), the allowed CAG list is provisioned and for local services at the 5G Femto cells(s) LADN DNN list may be provisioned from AF. If the LADN DNN is configured as associated with CAG ID(s) in the NEF or UDM (i.e. the associated LADN information such as LADN DNN and LADN service area per DNN and S-NSSAI), AF does not need provision the LADN DNN list together. Moreover, to support cell granularity for local service, the associated NSSAI location availability information is configured by AMF or OAM configuration.

Based on the provisioned 5G Femto Information, CAG information and its associated LADN information are updated to UDM/UDR. And, based on notification from UDM, the AMF initiates the UCU procedure to update the UE(s) with the CAG information and LADN information (i.e. LADN DNN and LADN Service Area Information per DNN and S-NSSAI) and S-NSSAI location availability information.

********** End of Changes **********