3GPP TSG SA WG2#165	            S2-2410366

Hyderabad, India, October 14-18 2024 	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1260 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of a new function dedicated to energy analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following the agreement that a new functionality which should support the energy consumption estimates, this CR defines a new function ECEF (Energy Consumption Estimation Function) which reuses the NWDAF framework and some NWDAF services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses added to describe a new NF which is dedicated to the handling the analytic for energy consumption estimation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Energysys collection and exposure of energy related information is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x(new), 6.x (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.X	Energy Consumption Estimation Function

The Energy Consumption Estimation Function (ECEF) is a NF which is dedicated to the support of only energy consumption estimation at UE, PDU session and QoS flow granularities. The ECEF reuses the NWDAF framework documented in this specification and can support NWDAF service operations and is limited to the sole purpose of handling energy consumption analytics. The energy consumption analytics are defined in clause 6.x.

The following Service operations mappings are defined:

Table 4.x-1: ECEF services to NWDAF services

| Necef service | Nnwdaf service |  |
| --- | --- | --- |
| Necef_AnalyticsSubscription | Nnwdaf_AnalyticsSubscription |  |
| Necef_AnalyticsInfo | Nnwdaf_AnalyticsInfo |
| Necef_DataManagement | Nnwdaf_DataManagement |

more CHANGES

## 6.X	3GPP User Plane energy consumption analytics supported by the ECEF

### 6.x.1	General

This clause specifies the procedure for an ECEF to provide statistics on user plane energy consumption of UEs identified by their UE ID (GPSI or SUPI), PDU session identified by a GPSI or SUPI and DNN/S-NSSAI and a QoS flow identified by a SUPI+ DNN/S-NSSAI or IP-5-tuple or Application ID, according to the information provided by the service consumer.

The ECEF collects traffic data usage information related to the requested granularity over the specific time periods that are related to the collection of energy consumption in a PLMN, and then apportions the total usage  of energy consumption over the time period by the affected UPFs and NG-RAN nodes based on the ration of data volume over the overall data volume these UP nodes have handled in the time period.

The ECEF derives the used NG-RAN nodes by a UE in a time period T based on MDT data or the ULI. The ECEF then retrieves the total data volume and total energy consumption in these NG-RAN network nodes to then derive the statistics.  Similarly, the UPFs use are determined based on the DNN

The consumer of these analytics includes in the request:

-	Analytics ID = "UP energy consumption".

-	Target of Analytics Reporting as defined in clause 6.1.3 for a single UE (SUPI, GPSI) or group of UEs (a list of Internal-Group-Ids)

-	Analytics Filter Information containing:

-	Optionally: Area of Interest (i.e. the location of UE(s));

-	Optionally S-NSSAI and DNN

-	Optionally; IP address 5-tuple

-	Optionally, preferred level of accuracy of the analytics.

-	An Analytics target period indicates the time period over which the analytics are requested.

Depending on local regulations, the ECEF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the ECEF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the ECEF skips those SUPIs that do not grant user consent.

### 6.x.2	Input Data

The ECEF shall be able to collect UE related energy consumption at UE, Session level (S-NSSAI+DNN) and QoS flow level (IP address 5-tupledispersion information from NF(s) and AFs. The information collected by the ECEF is based on data from 5GC NFs and OAM:

Table 6.x.2-1 provides the list of inputs received from 5GC NFs at the ECEF.

Table 6.X.2-1: 5GC NF Input data for UP energy consumption analytics

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | SMF | UE IP address. |
| UE ID | SMF | SUPI |
| S-NSSAI +DNN | SMF | Slice and DNN applicable to a PDU session |
| IP 5-Tuple | SMF | IP-5-tuple |
| Timestamp | SMF | Time stamp of the collected information. |
| Data Volume UL/DL per PSA UPF | SMF | Sum of UE per UE, S-NSSAI DNN, or 5-T-uple depending of the finest granularity present. |
| I-UPF ID(s) | SMF | Identifier of any I-UPF(s) used by a PSA UPF |
| ULI (with timestamp) | AMF, SMF | User location information used to derive the NG-RAN nodes used at the designated time. |
| NOTE: The Data volume can be reported either as total volume of the PDU session or periodically. It refers to the Data volume exchanged between the start and stop of the PDU session. When reported periodically, the period can be specified in the requested analytic target period or configured as a default value in the SMF. |  |  |

There are two modes of data volume collection:

-	Non periodical: A mode where the data volume is requested and consequently provided for the total volume of a PDU session from establishment to release.

-	Periodical: A mode where data volume is provided periodically between the establishment and the release of a PDU session. The period, for the purpose of these analytics, is configured PLMN-wide by the operator across all NG-RAN nodes and UPFs.

For both modes of data collection, if there are multiple PDU sessions for a UE or multiple QoS flows for the same UE for the same application, the ECEF aggregates (i.e. sums up) the data volume across all the PDU sessions for the UE to obtain per UE information and across all the QoS flows for the same application to obtain the per application information for the UE.

Table 6.x.2-2 defines the information the ECEF retrieves from OAM.

Table 6.x.2-2: OAM Input data for UP energy consumption analytics

| Information | Source | Description |
| --- | --- | --- |
| UE MDT data | OAM | To derive the Cell ID of the cells visited by a UE and the related gNB at a given time. This can be alternative to obtaining ULI. |
| gNB energy consumption | OAM | The Energy consumed by a gNB (ECgNB) over the configured time period. This is based on clause 6.7.3.4.2 of TS 28.554[10] |
| gNB data volume | OAM | The UL/DL data volume handled by a gNB over the configured time period. This is based on clause 6.7.1.1 of TS 28.554[10] |
| UPF energy consumption | OAM | The Energy consumed by a UPF over the configure time period. This is based on clause 6.7.3.1 of TS 28.554[10] |
| UPF data volume | OAM | Data volume consumed at a UPF. For instance, In case of a UPF with N3 interface(s), the data volume of the UPF is obtained by summing up, for all N3 interface(s), the number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF (see TS 28.552 [8] clause 5.4.1.3) and the number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN (see TS 28.552 [8] clause 5.4.1.4) |

### 6.x.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following Statistics from a ECEF:

-	UE energy consumption as defined in Table 6.x.3-1.

-	Energy consumption per UE and S-NSSAI, DNN as defined in Table 6.x.3-2.

-	Energy consumption for a QoS flow as defined in Table 6.x.3-3.

Table 6.x.3-1: Statistics of Energy consumption per UE

| Information | Description |
| --- | --- |
| UE ID | Identification of the UE via SUPI or GPSI |
| ECUE | Estimated Energy Consumption for a UE |
| Time interval | Time interval for the estimate specified by start and end times. |

Table 6.x.3-1: Statistics of Energy consumption a PDU sessions (identified by UE ID + S-NSSAI+ DNN)

| Information | Description |
| --- | --- |
| UE ID | Identification of the UE via SUPI or GPSI |
| ECUE, (S-NSSAI+DNN) | Estimated Energy Consumption for a UE PDU session identified by (S-NSSAI, DNN) pair |
| Time interval | Time interval for the estimate specified by start and end times. |

Table 6.x.3-1: Statistics of Energy consumption for QoS flow (identified by UE ID + IP-5-tuple)

| Information | Description |
| --- | --- |
| UE ID | Identification of the UE via SUPI or GPSI |
| ECUE, QoS flow | Estimated Energy Consumption for a UE QoS flow identified by the IP-5-tuple |
| Time interval | Time interval for the estimate specified by start and end times. |

### 6.x.4	Procedures

Figure 6.x.4-1: UP energy consumption analytics by ECEF

Figure 6.x.4-1 shows the procedure for ECEF to derive UP energy consumption analytics. The steps are described as follows:

1.	A consumer NF subscribes to/requests a NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Necef_AnalyticsInfo_Request service operation (Analytics ID = UP energy consumption and a set of Analytics Filters defined in clause 6.x.1).

2.	The ECEF derives the SMF and optionally AMF (if the ULI is received via AMF) that are used for the UE. The SMF registrations query can include the S-NSSAI, DNN if the requested granularity is per (S-NSSAI+DNN)

3.	[CONDITIONAL] The ECEF may subscribe to the AMF(s) event exposure service to collect data on the ULI of the UEs if the location of the UE is derived directly from ULI received at AMF.

4.	The ECEF subscribe to the SMF(s) event exposure service to collect input data as in Table 6.X.2-1. ECEF can then use such collected data to determine the ULI of the UE (if not provided by the AMF or OAM) and usage data per UE or session of the UE determined by S-NSSAI, DNN or QoS flow as requested by the NF.

5.	The ECEF subscribes to input data in 6.x.2-2 from the OAM according to the data collection principles from the OAM described in clause 6.2.3. the MDT data is used if ULI from step 3 or 4 are not used to determine the gNB used by the UE at any time in the required time interval for analysis.

7.	The ECEF works out the estimations of energy consumption as per request at step 1.

8.	The ECEF derives slice load analytics.

9.	The ECEF delivers analytics to the consumer NF by invoking Necef_AnalyticsSubscription_Notify or Necef_AnalyticsInfo_Request response service operations.

more CHANGES

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |

Table 7.1-3: Analytics information provided by ECEF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| UP Energy Consumption | Analytics ID: UP energy consumption | Statistics of energy consumption for the User Plane for a UE, PDU session or QoS flow |