3GPP TSG-WG SA2 Meeting #165	S2-2410382

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5770 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-SAT-UE communication via coordination between 5GC and IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support of UE-SAT-UE communication, coordination between 5GC and IMS is needed: - At initial IMS call setup, the 5GC should report ID of the satellite serving UE, which can be used by P-CSCF to know that the UE is using regenerative-based satellite and determine whether UE-SAT-UE can be enabled. The satellite ID can be carried in Access Network Information report information. - Upon receiving a Npcf_SMPolicyControl_UpdateNotify message from PCF that contains an IP address of the IMS-AGW and DNAI associated with the satellite serving the UE, the SMF inserts an UL CL/BP and local PSA UPF on the satellite associated to the DNAI and installs the corresponding N4 rules to direct the IMS media traffic to the IMS-AGW on satellite. The P-CSCF as an AF can re-use existing procedures of Application Function influence on traffic routing. - During satellite change, SMF may decide relocate the onboard UPF to a target satellite or remove the onboard UPF to use the ground PSA UPF. SMF should report the user plane event (i.e., ground DNAI or target DNAI and associated target satellite identifier) to IMS to help IMS select the IMS-AGW on the target satellite or ground. It is assumed that there is a time period the old IMS-AGW serve the UE before SMF receving IMS-AGW relocation decision from IMS. (It is worthing noting that the handover procedure should not be coupled with IMS layer, i.e., whether the SMF relocates UPF to target satellite or removes the UPF from satellite during handover should be decided by SMF itself, e.g., if source and target satellite is not connected, the onboard UPF will be removed during the handover procedure). If SMF reports satellite identifier and a DNAI associated with the target satellite, the IMS can further use the satellite id to derive the satellite connectivity between the target satellite and peer side satellite serving the peer side UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the coordination between 5GC and IMS to support UE-SAT-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how 5GC coordinates with IMS to support UE-SAT-UE communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.x.2	Support of UE-Satellite-UE communication with IMS user plane components on-board satellites

##### 5.4.x.2.1	General

This clause applies to the case where UE-Satellite-UE communication relies on the presence of IMS user plane components (i.e., IMS-AGW) on-board satellite to optimize media routing. The architecture is described in TS 23.228 [15].

##### 5.4.x.2.2	Initial setup of UE-Satellite-UE communication

The AMF reports the satellite identification to the SMF as part of the PDU Session establishment procedure as described in clause 4.3.2 of TS 23.502 [3].

NOTE 1:	Based on local configuration, the AMF knowes the identification of the satellite hosting the gNB node serving the UE.

At initial IMS call setup, after receiving a request from the P-CSCF for access network information report, the PCF serving the UE subscribes to Access Network Information report from the SMF for the IMS PDU Session, as described in clause 6.1.3 of TS 23.503 [45]. The Access Network Information report contains the identifier of the satellite serving the UE as reported by the AMF.

NOTE 2:	How the serving satellite identity is used by the IMS to determine whether the UE-SAT-UE communication can be activated is specified in TS 23.228 [15].

Based on Application Function influence on traffic routing described in clause 5.6.7 and procedures described in clause 4.3.6 of TS 23.502 [3], upon receiving a Npcf_SMPolicyControl_UpdateNotify message from the PCF that contains an IP address of the IMS user plane components allocated for the UE in traffic filter and DNAI associated with the IMS user plane components, the SMF determines and inserts an UL CL/BP and local PSA UPF on the satellite associated to the DNAI and installs the corresponding N4 rules to direct the IMS media traffic to the IMS user plane components on satellite. An N6 tunnel may be used between the L-PSA and IMS user plane component, if N6 traffic routing information is received, the SMF should direct the UE-Satellite-UE media traffic to the N6 tunnel indicated by the N6 traffic routing information.

NOTE 3: It is assumed that each satellite is assigned dedicated DNAI(s).

##### 5.4.x.2.3	Reporting of user plane management event at change of satellite

When the gNB onboard the satellite serving one of the two UEs involved in a UE-Satellite-UE communication changes (e.g. due to the satellite movement), the UL CL/BP and L-PSA may be relocated to a target satellite or removed from satellite.

The AMF should report the target satellite identification to SMF as part of the handover procedure in clause 4.9.1 of TS 23.502 [3]. The SMF should determine to relocate the source onboard UL CL/BP and L-PSA to the target satellite or remove the onboard UL CL/BP and L-PSA based on satellites connectivity of the source satellite and target satellite. The SMF may derive whether the two satellites are connected using the satellite identifications received from AMF.

NOTE 4:	How SMF uses the satellite identifications to derive whether the two satellites are connected is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information (e.g., Remote GTP-U peer information) reported by the UPF deployed on the source satellite and target satellite which presents that the two satellite are connected or not.

The SMF should report user plane management event with early notification (i.e., change of satellite identifer and potiential change of DNAI associated with UPF onboard the target satellite) and late notification (i.e., DNAI already changed to the one assoicated with UPF on ground) to P-CSCF based on the procedure of notification of user plane management event described in clause 4.3.6.3 of TS 23.502 [3]. If the DNAI reported to IMS is associated with UPF on the ground, it means the UE-Satellite-UE communication cannot continue as the UE has to use the IMS user plane components on ground for service continuity. If the DNAI reported to IMS associated with UPF on target satellite, it is up to the IMS to determine whether the UE-Satellite-UE communication can continue with the peer side UE using the target satellite or not and relocate the IMS user plane components to the target satellite or to the ground accordingly.

NOTE 5: How the satellite identifier of the target satellite is used by IMS to detetermine whether the UE-Satellite-UE can continue is specified in TS 23.228 [15].

To coordinate with IMS for IMS user plane components relocation, the following action is performed by 5GC:

-	For the UE that undergoes satellite change, the following coordination is performed for IMS user plane component relocation:

-	To coordinate the IMS user plane component relocation with the UL CL/BP and L-PSA relocation to a target satellite, the simultaneous change of UL CL/BP, L-PSA procedure in clause 4.3.5.6 or 4.3.5.7 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

-	To coordinate the IMS user plane component relocation with the UL CL/BP and L-PSA removal, the procedure of UL CL/BP and L-PSA removal in clause 4.3.5.5 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

-	For the other party UE which does not change serving satellite, the following coordination is performed if IMS user plane component is relocated to ground:

-	To coordinate the IMS user plane component relocation to ground with the UL CL/BP and L-PSA removal, based on Application Function influence on traffic routing described in clause 5.6.7 and procedures described in clause 4.3.6 of TS 23.502 [3], upon receiving a Npcf_SMPolicyControl_UpdateNotify message from the PCF that contains a DNAI associated with IMS user plane components on ground network, the SMF determines the DNAI is associated to the PSA UPF on the ground and removes the UL CL/BP and local PSA UPF from satellite.

# * * * * End of changes * * * *