3GPP TSG-SA WG2 Meeting #165	S2-2410383

# Hyderabad, IN, 14th Oct – 18th Oct, 2024								(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1492 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication in IMS with IMS-AGW on board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, the functionality and procedure of UE-satellite-UE communication via IMS-AGW deployed on satellite should be captured in TS 23.228. For call setup: When the originating P-CSCF receives the initial INVITE request from the originating UE using satellite access, it is not able to determine whether the UE-Satellite-UE communication can be activated before negotiating with the terminating side. It is possible to have different options in this case for media negotiation and IMS-AGW selection: - Option#1: The originating P-CSCF selects an IMS-AGW on ground for the originating UE by default after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side. However, if the terminating side determines that it is possible to activate UE-Satellite-UE communication, then, a re-selection/re-negotiation procedure is needed for the originating P-CSCF to re-select the IMS-AGW on satellite for the originating UE to enable the UE-Satellite-UE communication. - Option#2: The originating P-CSCF selects both the IMS-AGW on ground and the IMS-AGW on satellite after receiving INVITE message and triggers media negotiation by sending SDP offer to the terminate side, which contains both address of the IMS-AGW on satellite and ground. If the terminating P-CSCF decides to activate UE-Satellite-UE communication, the only the IMS-AGW on satellite is selected to be used and the one on the ground is released. If on the terminating P-CSCF decides not to activate UE-Satellite-UE communications only the IMS-AGW on the ground is selected. - Option#3: As the originating P-CSCF can forward the INVITE message to the terminating side without carrying an SDP offer, which let the terminating side to trigger the media negotiation after UE-Satellite-UE communication determination. This means the pattern 4 defined in IETF RFC 6337 is used for media negotiation in this case. Compared with option#1, if UE-Satellite-UE is activated in the end: - the option#2 can avoid an additional interaction between terminating side P-CSCF and IMS-AGW on satellite (as no need to send the address of originating side satellite IMS-AGW address). - the option#3 can avoid additional interaction between P-CSCF and ground IMS-AGW at originating side (as no ground IMS-AGW slection and release performed) and avoid additional interaction between P-CSCF and satellite IMS-AGW at terminating side (as no need to send the address of originating side satellite IMS-AGW address). If UE-SAT-UE is not activated in the end: - the option#2 has additional interaction between P-CSCF and IMS-AGW on satellite at the originating side (as there is additional satellite IMS-AGW selection and release). - the option#3 has same number of interactions between P-CSCF and IMS-AGW at both originating and terminating side (as no IMS-AGW re-selection performed). Considering that use of UE-Satellite-UE communication may be infrequent in an initial period of deployment, option#1 should be adopted as a basic way. Option#2 and option#3 can be specified as optional ways. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of support of UE-Satellite-UE communication in IMS, including: - Functional description of P-CSCF for UE-SAT-UE communication. - Call setup procedures of UE-SAT-UE communication which provide three different ways. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implementation notes: The referenced clause AX.3.2 is added by TS23.228 CR1493. This CR should be implemented before TS23.228 CR1493. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (All following text are new) ---

Annex AX (normative):
Support of UE-Satellite-UE communication in IMS

# AX.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., a communication service where the media is routed utilising only IMS user plane on-board the satellites that provide access to two UEs.

When a call setup is requested, the originating P-CSCF determines whether UE-Satellite-UE communication can be activated on the originating side. If UE-Satellite-UE communication is possible on the originating side, the originating P-CSCF requests the terminating P-CSCF to determine if UE-satellite-UE communication is also possible on the terminating side. Based on both originating side and terminating side support for UE-Satellite-UE communications, the terminating P-CSCF informs the originating P-CSCF and the call setup is progressed accordingly as described in clause AX.3.1.

After the call setup of UE-Satellite-UE communication, ground fall-back routing procedure may be performed as due to a change of satellite or other circumstances, it may not be possible to continue to use UE-Satellite-UE communication. The procedure of ground fall-back routing at change of satellite(s) is described in clause AX.3.2.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and not roaming. The P-CSCFs should take these restrictions into consideration for determining whether to activate UE-Satellite-UE communication or not.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites, using this IP-CAN and with the media not transiting through any core network elements on the ground. The ground fall-back routing refers to the case with the media routing includes ground segment.

# AX.2	Architecture and functional entities

## AX.2.1	Architecture

Editor's note: 	Architecture is FFS.

## AX.2.2	Functional entities

### AX.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionality:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in Annex AX.3.

-	The P-CSCF selects an IMS-AGW on board satellite based on the determined satellite ID. It is assumed the IMS-AGW has provided its satellite ID to the P-CSCF during IMS-AGW registration procedures.

# AX.3	Optimized media routing activation

## AX.3.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using satellite access, it is not able to determine whether the UE-Satellite-UE communication can be activated as it has no knowledge of whether the terminating UE is also using satellite access and a media path between the UEs using IMS media nodes onboard the satellite exists. Therefore, it is the terminating P-CSCF that decides whether the UE-Satellite-UE communication can be activated. Three different ways are specified for media negotiation of the UE-Satellite-UE communication call setup:

-	Option#1: the originating P-CSCF selects the IMS-AGW on ground as default after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side. If the terminate side decides to activate UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. The procedure is described in clause AX.3.1.1.

-	Option#2: the originating P-CSCF selects both the IMS-AGW on ground and the IMS-AGW on satellite after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side, which contains both transport address of the IMS-AGW on satellite and on the ground. If the terminating side decides to activate UE-satellite-UE communication, the originating P-CSCF releases the IMS-AGW on ground. The procedure is described in clause AX.3.1.2.

-	Option#3: the originating P-CSCF forwards the INVITE message to the terminating side without carrying an SDP offer, and it is the terminating P-CSCF to trigger the media negotiation after determination of whether UE-Satellite-UE communication can be activated. The procedure is described in clause AX.3.1.3.

Option#2 and Option#3 have the advantage of avoiding additional interaction between P-CSCF and IMS-AGW on satellite during the call setup for UE-satellite-UE communication.

Editor's note: 	Whether all options should be supported is FFS.

Editor's note: 	Whether and how to take UE subscription aspect into account for UE-satellite-UE communication activation is FFS.

### AX.3.1.1 Originating P-CSCF selects ground IMS-AGW as default

Figure AX.3.1-1:	Session establishment procedure with activation of optimized media routing

1.	UE A sends the SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The P-Access-Network-Info in the SIP INVITE contains ‘3GPP-NR-SAT’ as the 'access-class' as defined in TS 24.229 [10a].

2.	After receiving the initial INVITE request which contains '3GPP-NR-SAT' as the 'access-class', P-CSCF A retrieves Access Network Information from 5GC based on the mechanism defined in TS 23.503 [95]. P-CSCF A determines whether the UE A is using regenerative satellite access or not based on whether a serving satellite identification is obtained as part of the Access Network Information or not. P-CSCF A shall use this information together with other information e.g., the originating UE is in the non-roaming scenario and belongs to the same PLMN as the terminating UE, to determine whether it is possible to activate UE-Satellite-UE communication.

3.	The P-CSCF allocates an IMS-AGW A' on the ground as default. P-CSCF A interacts with IMS-AGW A' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible. Otherwise, normal SIP message routing procedures with IMS-AGW on ground should be performed after P-CSCF A selecting a ground IMS-AGW for normal usage (details have been omitted from the diagram).

4.	P-CSCF A updates the INVITE message inserting the identification of the satellite serving UE A and updates SDP offer with the selected IMS-AGW transport addresses. Then, P-CSCF A forwards the updated INVITE request to the terminating side via IMS core (details have been omitted from the diagram). The satellite identification sent to terminating side conveys the information that UE A is using regenerative satellite access and the terminating P-CSCF should take this into consideration for UE-Satellite-UE communication.

5.	After receiving the INVITE request which contains a satellite identification, P-CSCF B retrieves Access Network Information of UE B from 5GC network based on the mechanism defined in TS 23.503 [95]. If the UE is also using the regenerative satellite access, the serving satellite identification is contained in the Access Network Information.

6.	Based on the two satellite identifications received in step 4 and step 5 which contain the access information of UE A and UE B, P-CSCF B determines that the UE-Satellite-UE communication is possible. To decide whether or not to activate the UE-Satellite-communication, P-CSCF B should also consider whether UE B is in the non-roaming scenario or not and may also derive whether the two satellites are connected using received satellite identifiers if they are not the same.

If UE-Satellite-UE communications is not activated, e.g., due to the UE B is not using regenerative satellite access, or the two serving satellites are not connected to each other or not the same, P-CSCF B selects an IMS-AGW B' on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW B on satellite for UE B.

NOTE:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites which presents that the two satellite are connected or not.

6a-6b. Based on the decision in step 6, P-CSCF B interacts with IMS-AGW B or IMS-AGW B' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, normal SIP message routing procedures with IMS-AGW on the ground should be performed after P-CSCF B selecting a ground IMS-AGW B' for usage as normal (details have been omitted from the diagram).

7-9. P-CSCF B forwards the updated INVITE request to UE B. UE B returns a 18X response with SDP Answer.

9a-9b. After receiving the SDP Answer, P-CSCF B interacts with the selected IMS-AGW B to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media. The IP address of the IMS-AGW B and DNAI associated with the IMS-AGW B on satellite should also be carried in the request sent to PCF to indicate 5GC to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication.
P-CSCF B should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

11.	P-CSCF B updates the 18X response to carry the result indication that UE-Satellite-UE communication is activated and the identifier of the satellite serving UE B. Then, P-CSCF B returns the updated 18X response to P-CSCF A via IMS core (details have been omitted from the diagram).

12.	Based on the UE-Satellite-UE communication indication in step 11, P-CSCF A Determines to perform UE-satellite-UE communication and select an IMS-AGW A on satellite.

12a-12c. P-CSCF A interacts with IMS-AGW A on satellite to allocate transport resources. P-CSCF A also releases the IMS-AGW A' on ground.

13. P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media. The IP address of the IMS-AGW A and DNAI associated with the IMS-AGW A on satellite should also be carried in the request sent to PCF to indicate 5GC to insert UL CL/BP and local PSA UPF for IMS PDU session to enable UE-Satellite-UE communication.

P-CSCF A should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

14. P-CSCF A updates the 18X response and forwards the updated 18X response to UE A.

15. UE A acknowledges the Response and sends the Response Confirmation to P-CSCF A.

16-17. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes the modified SDP offer which contains the IMS-AGW transport address. Upon receiving the modified SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B.

18. P-CSCF B sends the Response Confirmation to UE B.

19. Other sequent procedure continues to setup the call.

### AX.3.1.2	Originating P-CSCF selects both satellite and ground IMS-AGW as candidates

This procedure describes the option where originating P-CSCF selects both the IMS-AGW on ground and the IMS-AGW on satellite after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side. If the originating P-CSCF determines that UE-Satellite-UE communication is possible per the originating side information (i.e., the originating UE is using regenerative satellite access) this information is passed to the terminating side. If the terminating side decides to activate UE-satellite-UE communication, the originating P-CSCF releases the IMS-AGW on ground.

.

Figure AX.3.1.2-1: Media negotiation triggered by the originating P-CSCF for UE-satellite-UE communication during call setup

1.	UE A sends the INVITE request, containing an initial SDP, to P-CSCF A. The P-Access-Network-Info in the INVITE contains '3GPP-NR-SAT' as the 'access-class' as defined in TS 24.229 [10a].

2. '	After receiving the initial INVITE request which contains '3GPP-NR-SAT' as the 'access-class', P-CSCF A retrieves Access Network Information from 5GC based on the mechanism defined in TS 23.503 [95]. P-CSCF A determines whether the UE A is using regenarative satellite access or not based on whether a serving satellite identification is obtained as part of the Access Network Information or not. P-CSCF A shall use this information together with other information e.g., the originating UE is in the non-roaming scenario and belongs to the same PLMN as the terminating UE, to determine whether it is possible to activate UE-Satellite-UE communication.

3.	If P-CSCF A determines it is possible to activate UE-Satellite-UE communications, it selects an IMS-AGW A' on ground and also an IMS-AGW A on satellite for optimized media routing. P-CSCF A interacts with IMS-AGW A and IMS-AGW A' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-19 are performed if the UE-Satellite-UE communication is possible. Otherwise, normal SIP message routing procedures with IMS-AGW on ground is performed after P-CSCF A selects the ground IMS-AGW as normal (details have been omitted from the diagram).

4.	P-CSCF A updates the INVITE message with inserting the identification of the satellite serving UE A, and updates SDP offer with two IMS-AGW transport addresses of which the first one is the address of IMS-AGW A' on ground and the second one is the address of IMS-AGW A on satellite. The second address is carried by an attribute ("a=") line. Then, P-CSCF A forwards the updated INVITE request to the terminating side via IMS core (details have been omitted from the diagram). The satellite identification sent to terminating side indicates that UE A is using regenerative satellite access and the terminating P-CSCF should take this into consideration for UE-Satellite-UE communication.

5.	After receiving the INVITE request which contains a satellite identification, P-CSCF B retrieves Access Network Information of UE B from 5GC network based on the mechanism defined in TS 23.503 [95]. If the UE is also using the regenerative satellite access, the serving satellite identification is contained in the Access Network Information.

6.	Based on the two satellite identifications received in step 4 and step 5 which presents the access information of UE A and UE B, P-CSCF B determines that the UE-Satellite-UE communication is possible. To decide whether or not to activate the UE-Satellite-communication, P-CSCF B should also consider whether UE B is in roaming and may also derive whether the two satellites are connected using received satellite identifiers if they are not the same.

If UE-Satellite-UE communications is not activated, e.g., due to the UE B is not using regenerative satellite access, or the two serving satellites are not connected to each other or not the same, P-CSCF B selects an IMS-AGW B' on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW B on satellite for UE B.

NOTE:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites which presents that the two satellite are connected or not.

6a-6b. Based on the decision in step 6, P-CSCF B interacts with IMS-AGW B or IMS-AGW B' to allocate/configure transport resources based on the mechanism defined in TS 23.334 [74]. Based on whether UE-satellite-UE communication is activated, the P-CSCF B selects the address of IMS-AGW A' or A received in step 4.

7-9. P-CSCF B forwards the updated INVITE request to UE B. UE B returns a 18X response with SDP Answer. After receiving the SDP Answer, P-CSCF B interacts with the selected IMS-AGW B or IMS-AGW B' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media. The IP address of the IMS-AGW B and DNAI associated with the IMS-AGW B on satellite should also be carried in the request sent to PCF to indicate 5GC to insert UL CL/BP and local PSAUPF for IMS PDU session to enable UE-Satellite-UE communication.
P-CSCF B should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

11.	P-CSCF B updates the 18X response to carry the indication if UE-Satellite-UE communication is activated and identifier of the satellite serving UE B. Then, P-CSCF B returns the updated 18X response to P-CSCF A via IMS core (details have been omitted from the diagram). After receiving 18X response from the terminating side, if there is a UE-Satellite-UE communication indication in the received 18X response, P-CSCF A interacts with IMS-AGW A on satellite to allocate transport resources, otherwise P-CSCF A interacts with IMS-AGW A' on ground to allocate transport resources.

12.	P-CSCF A releases the IMS-AGW A' on ground if there is a UE-Satellite-UE communication indication in the received 18X response. Otherwise, P-CSCF A releases the IMS-AGW A on satellite.

13.	P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media. If UE-satellite-UE communication is activated, the IP address of the IMS-AGW A and DNAI associated with the IMS-AGW A on satellite should also be carried in the request sent to PCF to indicate 5GC to insert UL CL/BP and local PSA UPF for IMS PDU session to enable UE-Satellite-UE communication.
P-CSCF A should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

14. P-CSCF A updates the 18X response and forwards the updated 18X response to UE A.

15-18. UE A returns a PRACK to confirm that it has received the 18X response and UE B returns a 200 OK response.

19. Other sequent procedure continues to setup the call.

### AX.3.1.3	Media negotiation triggered by terminating P-CSCF

This procedure describes how media negotiation is triggered by the terminating P-CSCF based on information provided from originating P-CSCF during the call setup. If the originating P-CSCF determines that UE-Satellite-UE communication is possible per the originating side information (i.e., the originating UE is using regenerative satellite access) it triggers the terminating P-CSCF to configure the media to support UE-Satellite-UE communications by means of sending an INVITE without SDP Offer.

Figure AX.3.1.3-1: Media negotiation triggered by the terminating P-CSCF for UE-satellite-UE communication during call setup

1.	UE A sends the INVITE request, containing an initial SDP, to P-CSCF A. The P-Access-Network-Info in the INVITE contains '3GPP-NR-SAT' as the 'access-class' as defined in TS 24.229 [10a].

2.	After receiving the initial INVITE request which contains '3GPP-NR-SAT' as the 'access-class', P-CSCF A retrieves Access Network Information from 5GC based on the mechanism defined in TS 23.503 [95]. P-CSCF A determines whether UE A is using regenerative satellite access or not based on whether a serving satellite identification is obtained as part of the Access Network Information. P-CSCF A shall use this information together with other information e.g., the originating UE is in the non-roaming scenario and belongs to the same PLMN as the terminating UE, to determine whether is possible to activate UE-Satellite-UE communication.

Steps 3-18 are performed if the UE-Satellite-UE communication is possible. Otherwise, normal SIP message routing procedures with IMS-AGW on ground should be performed after P-CSCF A selecting a ground IMS-AGW for normal usage (details have been omitted from the diagram).

3.	If P-CSCF A determines it is possible to activate UE-Satellite-UE communications, i.e., UE A is using regenerative satellite access, P-CSCF A stores the SDP information of the initial INVITE and generates a new INVITE request without SDP offer.
P-CSCF A forwards the new INVITE request to terminating side via IMS core (details have been omitted from the diagram), which contains the identification of the satellite serving UE A. The satellite identification presents that the UE A is using regenerative satellite access and the terminating P-CSCF should take this into consideration for UE-Satellite-UE communication.

4.	After receiving the INVITE request, P-CSCF B retrieves Access Network Information of UE B from 5GC network based on the mechanism defined in TS 23.503 [95]. If the UE is also using the regenerative satellite access, the serving satellite identification is contained in the Access Network Information.

5-7. P-CSCF B forwards the updated INVITE request to UE B. UE B returns a 18X response with the SDP Offer according to the normal SIP message routing procedures.

8.	Based on the two satellite identifications received in step 3 and step 4 which convey the access information of UE A and UE B, P-CSCF B determines that the UE-Satellite-UE communication is possible. To decide whether or not to activate the UE-Satellite-communication, P-CSCF B should also consider whether UE B is roaming and may also derive whether the two satellites are connected using received satellite identifiers if they are not the same.
If P-CSCF B determines that UE-Satellite-UE communication can be activated, e.g., the serving satellite are the same or the two serving satellites are connected to each other, it selects IMS-AGW B on satellite which serves the UE B and allocates the media resource on the IMS-AGW B. Otherwise, it selects IMS-AGW B' on ground for UE B and allocate the media resource on the IMS-AGW B'.

NOTE:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites which presents that the two satellite are connected or not.

8a-8b. Based on the decision on step 8, P-CSCF B interacts with IMS-AGW B or IMS-AGW B' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

9. If the UE-Satellite-UE communication is activated, P-CSCF B updates the 18X response to carry an UE-satellite-communication indication to indicate that the current session uses UE-Satellite-UE communication and identifer of the satellite serving UE B. Then, P-CSCF B returns the updated 18X response to P-CSCF A via IMS core (details have been omitted in the diagram).

10.	When receiving the 18X response with an UE-Satellite-UE communication indication, P-CSCF A selects IMS-AGW A on the satellite which serves the UE A and allocates the media resource on the IMS-AGW A.
If there is no UE-Satellite-UE communication indication in the 18X response, P-CSCF A selects an IMS-AGW A' on ground and allocates the media resource on IMS-AGW A'.

10a-10b. Based on the decision on step 10, P-CSCF A interacts with IMS-AGW A or IMS-AGW A' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

11.	P-CSCF A instructs the PCF to authorize the resources necessary to establish QoS flow for media. If UE-satellite-UE communication is activated, the IP address of the IMS-AGW A and DNAI associated with the satellite serving the UE should also be carried in the request to PCF for IMS PDU session to enable UE-Satellite-UE communication. 
P-CSCF A should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

12-13.	P-CSCF A updates the 18X response to contain SDP Answer and forwards the updated 18X response to UE A according to the normal SIP message routing procedures. Then UE A returns a PRACK request to confirm that it has received the 18X response.

14. P-CSCF A updates the PRACK request based on the SDP information stored when receiving the initial INVITE request in step 1 and adds a SDP Answer. Then P-CSCF A forwards the update PRACK request to P-CSCF B via the IMS core (details have been omitted in the diagram).

14a-14b. Based on the decision on step 8, P-CSCF B interacts with IMS-AGW B or IMS-AGW B' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

15.	P-CSCF B instructs the PCF to authorize the resources necessary to establish QoS flow for media. If UE-satellite-UE communication is activated, the IP address of the IMS-AGW A and DNAI associated with the satellite serving the UE should also be carried in the request to PCF for IMS PDU session to enable UE-Satellite-UE communication. 
P-CSCF B should also subscribe to SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early and Late notifications type based on the clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94]. This can be an implicit subscription by the PCF as part of setting PCC rule(s).

16. P-CSCF B updates the PRACK request to contain the SDP Answer and forwards the updated PRACK request to UE B according to the normal SIP message routing procedures.

17-18. UE B returns a 200 OK response and the subsequent procedure continues.

## ---End of the Change---