3GPP TSG-SA WG2#165	S2-2410388

Hyderabad, IN, October 14 – 18, 2024	(Revision of S2-240xxxx)

Source:	Sony, NTT DOCOMO

Title:	Interim Conclusion Privacy

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion on Privacy protection support in 5G system.

# 1.	Discussion

SA2 asked SA1 for clarification on specific requirements in S2-2407231 as SA2 could not agree whether certain requirements need to fulfilled under all circumstances. SA2 asked SA1 on privacy protection in Question 2 and SA1 replied as below:

Chapter 5 in TS 22.369 list all functional Service requirements, in particular clause 5.2.6 “Security and privacy” states:

The answer from SA1 clarifies that the 5G system shall provide Privacy protection to all Ambient IoT devices even if such devices are used in certain deployment scenarios.

SA3 has also identified this requirement and included a specific key issue in TR 33.713.

Prior this meeting, SA3 has not captured any proposed solutions that address this key issue for Ambient IoT.

SA2 also included a question to RAN1 on Non-Volatile Memory (NVM) in the same LS out S2-2407231. RAN1 response to S2-2407231 is found in S2-2409592 (R1-2407364). Note that the answer from RAN1 is to question 5 and not question 2 (to SA1), but the answers may somewhat relate to each other.

and

RAN1 confirmed that an Ambient IoT device have an NVM. In addition, RAN1 also included further comments or advice highlighted in yellow to the usage of the NVM. This comment may stand in conflict with the SA1 confirmation that Privacy protection shall be supported by the 5G system.

Privacy protection in 5G System is achieved by using temporary ID over the radio interface. Additionally, the Temporary ID shall always be changed if the UE is paged with the Temporary ID.

As for now the only option to provide privacy protection is to use temporary ID. Therefore, we propose to capture this as an interim conclusion with an editor’s note that this conclusion is pending SA3 conclusion.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

## 8.2	Interim Conclusion on Key Issue #2

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

A permanent ambient IoT Device Identifier can be assigned by an operator or by a third party.

Editor's note:	How it is used is FFS.

An Ambient IoT device utilizes a temporary identifier to prevent unauthorized tracking of the Ambient IoT device.

Editor's note:	The above conclusion is pending SA3 confirmation.

# >>>>END OF CHANGES<<<<