3GPP TSG-SA WG2 Meeting #165	S2-2410404

# Hyderabad, India, October 14-18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1494 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-Satellite-UE communication in IMS - Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Support of UE-Satellite-UE communication in IMS focusing on procedures. This CR is written based on the following principle as well as the above-mentioned conclusion. - At call setup, SDP offer in SIP INVITE arriving to the terminating network contains the IP address information of the ground IMS AGW only and does not contain the one of the satellite IMS AGW. Practically, SDP answer and the next SDP offer are used by P-CSCF for negotiation. No new SDP attribute is introduced. (A new SIP header is introduced for conveying satellite ID.) - At change of satellite, either P-CSCF or IMS AS (requested by P-CSCF via SIP MESSAGE) sends SIP re-INVITE during the procedure to complete IMS AGW relocation. I.e., two options are described. No new SDP attribute is introduced. - No impact related to Subscription data. - No UE impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Support of UE-Satellite-UE communication in IMS with regards to procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

# AX.5	Procedures

## AX.5.1	Session establishment procedure with activation of optimized media routing

Figure AX.5.1-1 depicts a signalling flow diagram for establishing a session of optimized media routing. The flow starts with UE under a coverage of a satellite constellation making a call to another UE, not knowing whether the terminating UE is under the same coverage or not.

NOTE 1:	IMS entities not relevant for the procedure are omitted below for brevity of the description.

Figure AX.5.1-1:	Session establishment procedure with activation of optimized media routing

The steps in the call flow are as follows:

1.	UE sends SIP INVITE to P-CSCF as per clause 5.6.2. The SIP INVITE contains information indicating that the UE is connected via satellite (e.g., access-type=3GPP-NR-SAT) in the P-Access-Network-Info header.

2.	When receiving SIP INVITE indicating that the UE is connected via satellite, P-CSCF retrieves Access Network Information containing the satellite ID from 5GC by using the retrieval of network provided location information procedure described in clause Y.0 and as defined in TS 23.503 [95].

3.	Since P-CSCF does not have information on the terminating UE, P-CSCF assumes that the terminating UE is connected to the terrestrial network and determines to establish an IMS AGW on ground. P-CSCF starts setting up IMS-AGW on ground according to steps 1 to 4 of clause 6.2.1 of TS 23.334 [74].

4.	When P-CSCF recognizes that the originating network is capable of optimized media routing, P-CSCF sets the satellite ID into a SIP header for conveying satellite information, P-Satellite-Info header. P-CSCF sends SIP INVITE containing this SIP header to the terminating network as per clause 5.6.2. Either P-CSCF or IMS AS (omitted from the figure) in the originating network acts as a SIP B2B UA.

5.	Either P-CSCF or IMS AS (omitted from the figure) in the terminating network acts as a SIP B2B UA. When receiving SIP INVITE containing P-Satellite-Info header, P-CSCF retrieves Access Network Information containing the satellite ID from 5GC by using the retrieval of network provided location information procedure described in clause Y.0 and as defined in TS 23.503 [95].

6.	P-CSCF determines that optimized media routing is possible based on the satellite ID for the originating UE and the satellite ID for the terminating UE. P-CSCF determines to activate optimized media routing.

NOTE 2:	How the P-CSCF uses the satellite IDs to derive whether the two satellites are connected and whether optimized media routing is possible are not specified.

If P-CSCF determines that optimized media routing is not possible, P-CSCF follows the normal IMS session establishment procedure for subsequent steps.

7.	P-CSCF does not start setting up IMS AGW, which is different from clause 6.2.1 of TS 23.334 [74]. P-CSCF sends SIP INVITE to UE as per clause 5.7.2.

8.	UE sends SIP 183 Session Progress to P-CSCF as per step 5 of clause 5.7.2.

9.	P-CSCF starts setting up IMS AGW on satellite according to steps 1 to 4 of clause 6.2.1 of TS 23.334 [74], where SDP offer is replaced with SDP answer.

10.	P-CSCF does not authorize QoS resources, which is different from clause 5.7.2. When P-CSCF started setting up IMS AGW on satellite in step 9, P-CSCF sets the satellite ID into P-Satellite-Info header. P-CSCF sends SIP 183 Session Progress containing this SIP header to the originating network as per step 7 of clause 5.7.2.

11.	P-CSCF determines that optimized media routing is possible based on the satellite ID for the originating UE and the satellite ID for the terminating UE. P-CSCF determines to activate optimized media routing.

If P-CSCF determines that optimized media routing is not possible, P-CSCF follows the normal IMS session establishment procedure for subsequent steps.

12.	P-CSCF releases IMS AGW on ground according to clause 8.5 of TS 23.334 [74].

13. P-CSCF starts setting up IMS AGW on satellite according to steps 1 to 4 of clause 6.2.1 of TS 23.334 [74], where SDP offer is replaced with SDP answer.

14. P-CSCF does not authorize QoS resources, which is different from clause 5.6.2. P-CSCF sends SIP 183 Session Progress to UE as per step 8 of clause 5.6.2.

15.	UE sends PRACK to P-CSCF as per step 9 of clause 5.6.2.

16.	P-CSCF completes setting up IMS AGW on satellite according to steps 7 to 13 of clause 6.2.1 of TS 23.334 [74], where SDP answer is replaced with SDP offer.

17.	P-CSCF instructs PCF to authorize the necessary resources, i.e., to establish ULCL and L-PSA and a QoS flow for media traffic, as per step 7 of clause 5.6.2. In the request, P-CSCF contains the IP address allocated in IMS AGW on satellite to be used by UE as the destination of media traffic, and DNAI associated with the satellite serving the UE.

18. P-CSCF subscribes via PCF from SMF for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as is defined in TS 23.502 [94].

19.	P-CSCF sends PRACK to the terminating network as per step 11 of clause 5.6.2.

20.	P-CSCF completes setting up IMS AGW on satellite according to steps 7 to 13 of clause 6.2.1 of TS 23.334 [74], where SDP answer is replaced with SDP offer.

21.	P-CSCF instructs PCF to authorize the necessary resources, i.e., to establish ULCL and L-PSA and a QoS flow for media traffic, as per step 6 of clause 5.7.2. In the request, P-CSCF contains the IP address allocated in IMS AGW on satellite to be used by UE as the destination of media traffic, and DNAI associated with the satellite serving the UE.

22.	P-CSCF subscribes via PCF from SMF for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as is defined in TS 23.502 [94].

23.	P-CSCF sends PRACK to UE as per step 11 of clause 5.7.2.

24.	The rest of the procedure is as usual as per clauses 5.6.2 and 5.7.2.

## AX.5.2	IMS AGW relocation and media routing path change due to change of satellites

### AX.5.2.1 Continued optimized media routing procedure

Figure AX.5.2.1-1 depicts a signalling flow diagram for continuation of optimized media routing after change of satellites that serve a UE. The figure is written in such a way that change of satellite occurs in the originating network for the purpose of the explanation, while change of satellite can occur in the terminating network as well.

NOTE 1:	IMS entities not relevant for the procedure are omitted below for brevity of the description.

Based on the procedure described in clause AX.5.1, P-CSCF is expected to subscribe from 5GC for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic by using a scheme defined in TS 23.501 [93] and TS 23.502 [94].

Figure AX.5.2.1-1:	Continued optimized media routing procedure

The steps in the call flow are as follows:

1.	P-CSCF receives the early notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as is defined in TS 23.502 [94]. This early notification contains satellite ID of a target satellite that has gNB to which the UE gets connected and indicates that 5GC is prepared to change the user plane paths for optimized media routing to the one through this target satellite.

2.	P-CSCF determines that optimized media routing continues to be possible based on the satellite ID received in step 1 for the originating network and the satellite ID stored for the terminating network. P-CSCF determines to continue activating optimized media routing.

NOTE 2:	How the P-CSCF uses the satellite IDs to derive whether the two satellites are connected and whether optimized media routing is possible are not specified.

If P-CSCF determines that optimized media routing cannot continue, P-CSCF follows the ground fallback procedure as defined in clause AX.5.2.2 for subsequent steps.

3.	P-CSCF starts setting up IMS AGW on the target satellite according to steps 2 to 4 of clause 6.2.1 of TS 23.334 [74].

4.	P-CSCF completes setting up IMS AGW on the target satellite according to steps 8 to 13 of clause 6.2.1 of TS 23.334 [74] by using the IP address allocated in UE and the IP address allocated in IMS AGW in the terminating network, which P-CSCF has stored, to be used by any IMS AGW in the originating network as the destination of media traffic.

5.	P-CSCF replies as is defined in TS 23.502 [94] to the early notification received in step 1 by indicating that the change of the user plane paths for optimized media routing to the one through the target satellite should be performed. This reply also contains the IP address allocated in IMS AGW on the target satellite to be used by UE as the destination of media traffic. SMF in 5GC establishes ULCL and L-PSA on the target satellite according to clause 4.3.5.7 of TS 23.502 [94] and updates the UE filter for media traffic according to clause 4.3.3.2 of TS 23.502 [94].

6.	P-CSCF receives the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic as is defined in TS 23.502 [94]. This late notification indicates that 5GC has prepared the user plane path for optimized media routing through the target satellite while implying the user plane path for optimized media routing through the source satellite is still maintained as long as active traffic exists.

NOTE 3:	The following steps 7 to 18 are used only for the purpose of updating the setting in UE in the originating network and in IMS AGW in the terminating network with regards to the destination of media traffic towards IMS AGW on the originating network.

7.	[Conditional] If P-CSCF does not act as SIP B2B UA, P-CSCF sends SIP MESSAGE to IMS AS to request it to send SIP re-INVITE to the UE.

8.	[Conditional] If P-CSCF does not act as SIP B2B UA, IMS AS sends SIP re-INVITE to P-CSCF.

9.	P-CSCF sends SIP re-INVITE to UE. This SIP re-INVITE contains the IP address allocated in IMS AGW on the target satellite to be used by the UE as the destination of media traffic.

10.	UE sends SIP 200 OK to P-CSCF.

11. [Conditional] If P-CSCF does not act as SIP B2B UA, P-CSCF sends SIP 200 OK to IMS AS.

12.	[Conditional] If P-CSCF does not act as SIP B2B UA, P-CSCF sends SIP MESSAGE to IMS AS to request it to send SIP re-INVITE to the terminating network. This SIP MESSAGE contains the IP address allocated in IMS AGW on the target satellite to be used by IMS AGW in the terminating network as the destination of media traffic towards the originating network.

13. P-CSCF, or IMS AS if P-CSCF does not act as SIP B2B UA, sends SIP re-INVITE to the terminating network. This SIP re-INVITE contains the IP address allocated in IMS AGW on the target satellite to be used by IMS AGW in the terminating network as the destination of media traffic towards the originating network.

14.	P-CSCF modifies according to clause 8.4 of TS 23.334 [74] the destination of media traffic towards the originating network configured in IMS AGW to be the one received in step 13.

15.	P-CSCF sends SIP re-INVITE to UE.

16. UE sends SIP 200 OK to P-CSCF.

17.	P-CSCF sends SIP 200 OK to the originating network.

18. [Conditional] If P-CSCF does not act as SIP B2B UA, IMS AS sends SIP 200 OK to P-CSCF.

19. SMF in 5GC releases ULCL and L-PSA on the source satellite according to NOTEs 7 and 8 and steps 11 and 12 in clause 4.3.5.7 of TS 23.502 [94].

20. After receiving SIP 200 OK both in step 10 from the UE and in step 17 or step 18 from the terminating network, P-CSCF releases IMS AGW on the source satellite according to clause 8.5 of TS 23.334 [74].

### AX.5.2.2 Ground fallback

Editor's note:	FFS.

## ---End of the Change---