3GPP TSG-WG SA2 Meeting #165	S2-2410410

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion on key Issue 1 for Topology 1 Architecture

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusion principles are proposed on key issue 1 for topology 1 architecture.

1. Introduction/Discussion

  - Overview

This pCR is a summary of Topology 1 architecture proposals in the TR, considering aspects outlined in the following clauses and provides conclusions for topology 1.

  - Interaction between RAN reader and core network

It was agreed an interim conclusion principle in the SA2#164 meeting that “A new core network function is introduced to support Ambient IoT”.

The end-to end Ambient IoT service operation include the interaction between the BS reader and the new network function, AIoTF (NF name is subject to change). From the GSMA case study[1], a light-weight architecture, of which the core network support only the essential Ambient IoT features is preferable for the network deployment. Considering this is the initial stage of this new IoT technology is most likely for localized deployment, a light-weight network will be beneficial for the third party enterprise to deploy its own Ambient IoT network in the campus site, to achieve low-cost and simple maintenance or an operator to deploy isolated from their existing PLMN network.

News reports on two deployed Ambient IoT networks for Asian sports event [2] and digital production material management solution [3], proves that light weight architecture is suitable for both wide-area and localized deployment, including warehouse logistics, intelligent transportation, intelligent manufacturing, and asset management.

Two options are proposed in the SA2 TR solutions regarding how the BS reader interacts with the AIoTF:

Option 1: Interface between BS Reader and AIoTF

Option 2: BS Reader communicates with an AIoTF via an AMF

Figure 1.2-1 Two options for topology1 architecture

In case of option 1, the AIoTF supports the AIoT related features. The AIoTF can be deployed at the same site as the BS Readers (e.g. in factory or campus), for the enterprise or deployed away the from the BS Readers site if multiple enterprises intend to share the AIoTF (e.g. in a data centre).

Option 2 includes the AMF, but its functionalities is unclear. It is observed from the TR solutions that the AMF does not supportany AIoT specific features, which means the only function of the AMF is to route the signalling between the BS Readers and the AIoTF. If AMF is added to the architecture, the interface “Nx” between the AMF and the AIoTF needs to be defend, along with what we might be relaying/proxying functionality for the AMF. However, there is no TR solution describing this interface, how it works or the functionality of the AMF in this case.

Based on the above analysis, option 1 has several advantages, compared to option 2, including less core network functionality and new interfaces to define. Overall, option 1 supports  the light-weight and low-cost deployment, while maintaining flexibility.

With a localised deployment, like a campus or factory, the readers are important for the requested services, for example operations targeting a specific area. When a deployment is localised this can also mean that the information about the AIoT Devices (e.g. from inventory) or the data exchanged can all be maintained at the site.

When new technology is pushing to market, local deployment is usually a good choice, and can be scaled to wide area use case in the future.

Proposal 1: For topology 1 architecture, AIoTF supports AIoT features and core network does not include AMF.

  - Common network functions for topology 1 and topology 2

There is a deployment consideration that an Ambient IoT network can support both topology 1 and topology 2. In such case, one outstanding issue is which network function are needed for both topologies.

Topology 2 assumes a UE Reader accesses a macro-cell and is under network control. TR solutions have a common aspect that the UE Reader is served by an AMF of the public network. The AMF provides registration management, mobility management and connection management for the Reader UE. In addition, the AMF is responsible for authorizing and revoking the reader functionality of the UE.

However, it is questionable whether the AMF used in topology 2 can serve topology 1 as well. The AMF functionalities needed for topology 2, i.e. UE Reader functionality authorization and revocation, are not needed in topology 1. Moreover, it is described in several TR solutions that topology 1 architecture can work without AMF involvement.

In summary, AMF is not the common network function for topology 1 and topology 2.

AIoTF is needed for both topology 1 and topology 2. It is expected that AIoTF functionalities to support topology 1 and topology 2 may have some common aspects. However, it is also foreseen the difference of AIoTF functionalities for topology 1 and topology 2. Depending on the data transmission path between UE reader and AIoTF, topology 2 requires specific AIoTF functionality, as an example.

Proposal 2: AIoTF is the common network function for topology 1 and topology 2.

  - Isolation of Ambient IoT network deployment

SA2#164 meeting agreed a new architecture assumption that “A dedicated Ambient IoT core network can be deployed by the operator to handle the traffic produced by ambient IoT devices.”

Based on the summary of TR solutions, an Ambient IoT core network may include AMF, AIoTF, UDM and NEF.

AIoTF is an Ambient specific network function, so it is part of the dedicated core network by nature.

AMF, as analysed above, is essentially the public network AMF supporting eMBB service when used in topology 2, so that AMF is not suitable to be part of the dedicated Ambient IoT core network.

UDM and NEF can be deployed dedicated for Ambient IoT service. For example, an Ambient IoT UDM can be deployed to manage subscription-like information for only Ambient IoT Devices, but not for normal UE subscription data management.

Proposal 3: AIoTF, UDM and NEF can be the network functions if the dedicated Ambient IoT core network is deployed.

  - Other necessary core network functions excluding AIoTF

Several TR solutions proposed to support subscription-like information for Ambient IoT Devices, and the UDM is proposed as the network function to support this feature. In addition, the UDM may also be responsible to store the information derived by Ambient IoT Service operations, including last (known) reader and the device status.

The last reader can provide a coarse location for the device and will be updated each time the AIoT Device is inventoried. It can be used for providing the AF with a coarse device location or used by the network to help optimise which Readers to use when sending commands to the AIoT Device (i.e. similar in principle to a network paging strategy for UEs).

Device status is to indicate whether the Ambient IoT Device is active or has been disabled. A disabled device should not respond to operations and not be used. This allows an operator to prevent access to a device and/or control access from an AF to a device.

Proposal 4: NEF and UDM are necessary core network functions to support Ambient IoT, in addition to AIoTF.

  - Interaction between AIoT Device and core network

Some companies indicate a NAS protocol is supported between the AIoT Device and the new NF. A more detailed discussion is provided in S2-2410417 on the uses of a new NAS protocol between the AIoT Device and the network.

Although not included in many proposals, exposure or handling of data to/from the AIoT is proposed be handled by the CN/MNO. For many that did not mention this aspect directly, this seems to be a basis on which their proposals are made.

There seems to be a basis in some previous conclusion proposals for the CN to manage and hold information about the AIoT Devices.

Other aspects mentioned in a small number of proposals include reader identification, provisioning of information to the CN, handling (i.e. service related data, subscription-like information) and details of the routing (e.g. CP vs UP, shared PDU Sessions, etc) are included, but there does not look to be enough considerations or support to include them at this time.

Proposal 5: NAS protocol is supported between the AIoT Device and the AIoTF

2. References

- 5G-Advanced to Support Self-Powered Sensors, https://www.gsma.com/5ghub/iot

- CMCC & Huawei Complete the World's First 5G Advanced A-IoT Application in Asian Games Venue, http://en.c114.com.cn/577/a1242304.html

- Haier Smart Home and China Mobile Win the GTI ‘Innovative Mobile Service and Application Award’, https://www.mobileworldlive.com/haier-smart-home-and-china-mobile-win-the-gti-innovative-mobile-service-and-application-award/

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13, v1.0.0.

# * * * * First change * * * *

### 8.1.1	Architecture to Support Topology 1

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

At least the following principles are agreed for the architecture to support topology 1:

-	A new core network function, AIoTF, is introduced to support Ambient IoT features, including:

- Manage the BS Reader information, e.g. based on BS reader reporting or via OA&M configuration

- Receive and handle the AF request (via NEF) for Ambient IoT Services i.e. Inventory and Command (Read, Write or Disable)

- Generate Task ID for the AF requested Ambient IoT Service, and maintain the association (AF ID, Task ID, Mask etc) within the 5GC

- Based on AF request, the stored BS Reader information and the last known BS Reader information (if available), to select the BS Reader

- Interact with the BS Reader for DL and UL Ambient IoT signaling transfer

- Interact with Ambient IoT Device for UL and DL AIoT information transfer, via a light-weight AIoT-NAS protocol.

- Validate the Ambient IoT Device ID

- Aggregate the reported information from the multiple Ambient IoT Devices

- Respond to the AF request with the received information reported by Ambient IoT Devices

-	The AIoTF communicates with a BS Reader via a direct interface.

NOTE X1:	The protocol stack used between the AIoTF and the RAN Reader will be concluded by RAN3.

NOTE X2:	The Ambient IoT feature may be deployed as a standalone AIoT network or being integrated into an existing network.

-	The CN (e.g. UDM, etc) is responsible for management of Ambient IoT Device subscription-like information.

-	NEF is responsible for Ambient IoT service exposure to third-party AF, including data transfer.

Editor's note:	Whether the new core network function also applies to topology 2 is FFS.

# * * * * End of changes * * * *