3GPP TSG-WG SA2 Meeting #165	S2-2410417

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-2408552)

Source:	Huawei, HiSilicon

Title:	Ambient IoT: The importance of NAS in AIoT

Document for:	Discussion

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Discussion on the use and need for NAS in AIoT.

# 1. Introduction

The NAS protocol is an important layer in the 3GPP system, allowing the CN to maintain communication with a UE, while the AS remains agnostic to the messages and the parameters being transported. It also provides communication endpoints directly between the UE and the CN, allowing important information about the UE, for example long term keys, subscription data etc, to be maintained only in the CN, reducing risk.

Many of these important aspects remain valid for AIoT and are important for a good system design with expandability in the future.

# 2. Discussion

## 2.1 	Protocol Stack

Figure 2.1-1: AIoT NAS Protocol Stack

The AIoT NAS runs between the AIoT Device and the CN, and the Reader role (whether a UE Reader or a RAN Reader) is to forward the messages. This is similar to the handling of NAS today in 5GS and EPS by the gNB/eNB, i.e. the gNB/eNB does not construct, deconstruct or interpret the contents of the NAS messages, but just forwards them between a UE and the AMF/MME. AIoT NAS follows the same basic principle.

## 2.2	AIoT Functionality

Key Issues #3 includes the study on how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices. According to the requirements in TS 22.369, the 5G network shall play a key role in the transfer of AIoT data between a 3rd party and an Ambient IoT device, including 1) receiving 3rd party instruction to trigger a group of Ambient IoT Devices, 2) trigger a group of Ambient IoT Devices in a specific area and which action the Ambient IoT Devices perform when triggered, 3) obtain data from Ambient IoT Devices and provide the AIoT data to a 3rd party. AIoT is expected to support the following types of information exchange between 5G network and passive devices:

1. 	Reporting identity

2. 	Data transmission (uplink and downlink)

This list is based on core functionality described by multiple solutions within TR 23.700-13. While the list is not exhaustive, it covers the most common aspects of multiple solutions. We note that some solutions propose or assume additional AIoT NAS-like messages.

RAN2 has assumed (see the LS in S2-2407447) "RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input. ", then according to SA1 TS22.369 clause 5.2.6, "5G shall provide security protection and protect the privacy of information exchanged during communication between an Ambient IoT device and the 5G network", Thus if security is needed for Ambient IoT as concluded by SA3, and then security data is added to the list above.

The AIoT NAS message formats need to be different from the NAS messages formats defined today for 5GS (as defined by stage-3). Fundamentally the NAS messages need to transport information as required by a procedure (whether that is application data, commands, inventory information, or security related parameters etc) and in this discussion paper we make no assumptions about the size, parameters or contents of the information required, only consider the difference that an AIoT NAS would make on top the required transferred information.

## 2.3	Future Growth

In a future release (beyond Rel-19) it can be anticipated that AIoT will support features such as DOA, which may lead to additional signalling between the AIoT device and the CN. Looking beyond that, if AIoT follows the trajectory of other technologies in 3GPP, additional capabilities and features may be added, which can include both RAN and CN based enhancements.

Introducing NAS in Rel-19 allows the behaviour required in Rel-19 to be supported and provides a possibility to add functionality in the future. Having a NAS protocol enables AIoT Device/CN only features to be specified without impacts on the RAN.

There is a counter argument to say either a NAS protocol can be added in the future, when required, or simply that the AS protocols can pass whatever parameters/information is needed in a hop-by-hop way.

Both of these counter arguments would mean either significant re-architecture work in the future to support NAS-like signalling, or each and every future enhancement having RAN changes, even if the feature does not need RAN enhancements to operate, or potential backwards combability issues as at least 2 different ways to interact with AIoT devices become possible (and therefore having to make a choice during operations about which way to use).

As the specification may evolve over time, we also need to be careful about compatibility between the Readers in a deployment. For a deployment with, for example, RAN Readers only it could overall be a simpler task to control updates/redeployments to the Readers such that they all remain in sync. If there is a mixed deployment with very flexible sourcing of Readers, management of the reader capabilities and managing how to use those readers and their capabilities becomes a more complex problem.

Therefore, to ensure simplicity for deployments and to allow the CN to evolve at different pace than the RAN, UE Reader and RAN Readers, having NAS messages is highly desirable from the very first Release where AIoT is specified.

## 2.4	Security End Points

In the 5GS the UE has 2 security endpoints, in the AS terminating in RAN and in NAS terminating in the CN. RAN2 has already determined not to support AS security (subject to SA3). If we consider NB-IoT, which can operate without AS security, then all confidentially and integrity protection is handled between the NB-IoT UE and the MME/AMF.

Keeping the end points of security within the CN and the AIoT Device means that the Reader does not need to handle key material. Having visibility / security end point in the AS means that the Reader can see the data/information being transferred to/from the AIoT Device.

For a RAN Reader, similar to EPS/5GS, it could be suitable to handle some short-term key material (i.e. similar in concept to an RRC Connection), however there could be questions about how trusted a UE Reader could be (and RAN2 to support AS security). To avoid this aspect (which was heavily discussed in ProSe for example with L2 relay vs L3 relay in the past) keeping key material out of RAN (which, as mentioned is not planning to support security) can reduce risks.

Additionally keeping security endpoints, and the signalling for them, between the AIoT Device and the CN using NAS signalling allows a mixed deployment of device capabilities without RAN needing to understand the security requirements.

This principle also allows the extension or evolution of security procedures without RAN impact.

## 2.5 	NAS Overhead

As described in clause 2.2, we assume that same information needs to be transferred for a procedure (whether that is application data, commands, inventory information, or security related parameters etc) and here we examine the overhead beyond that baseline information that a dedicated AIoT NAS brings.

Figure 2.5-1: NAS Additions

Figure 2.4-1 shows some example App Data (e.g. data read from a device) and it either being sent ‘hop-by-hop’, or encased in a NAS message. Also shown is potential security related content, if security related content is required, then it will remain constant for the hop-by-hop example and NAS example, it is either sent together with the App Data in hop-by-hop example or sent with it as part of a NAS message.

We don’t consider the exact content or makeup of the parameters, only they are present, regardless of the way they are transported.

In the hop-by-hop example, the AIoT AS Layers will add some header information, indicated by ‘AIoT AS Layers’ in the figure, followed by the App Data & potential security related content. If the AS Layer has its own control messages/signalling (e.g. from an AIoT MAC Layer, if determined by RAN), then there would need to be some indication in the AIoT AS Layers whether a transmission contains AS Layer signalling or application data. If, in addition, there is NAS in the future, such indication would need be expandable to also include NAS, as well as AS layer signalling and application data.

In the NAS example the same App Data & potential security related content is transferred, along with AIoT AS Layer information, with the difference that the AS would only indicate NAS vs AS signalling (and not AS vs NAS vs App Data & potential security related content in the hop-by-hop case). There is the addition of a NAS Header (in green) that provides the NAS message identifier/command type to provide the disambiguation between the NAS messages.

It is not expected to reuse the existing 5GS NAS message formatting.

The overall length of the message can be determined when constructing it for transmission and on reception by the Reader / AIoT Device (either from the transmission length or from the known message contents), therefore avoiding the need for a length within the NAS message itself, this is a similar approach taken by RFID for example. The length of the message identifier/command, as seen in many other specifications/standards, is typically around a single octet.

There is a small NAS overhead with the introduction of the NAS Header, compared with hop-by-hop example. This could be a single octet (or less if 8-bit alignment is not required). This small NAS overhead brings limited impacts, considering many benefits it brings.

## 2.6	Example NAS Messages

In TR 23.700-13 we can see suggestions for different messages which could be handled as NAS. Examples of the NAS messages are:

-	MASK: RAN2 expects SA2 to define the MASK format. This could be a NAS message, which may also include other selection information (as appropriate).

-	Device ID Report: Sent by the AIoT Device in response to an inventory.

-	Read Command: A request to read information from the AIoT Device, including application defined parameters.

-	Read Response: Response to the Read Command.

-	Write Command: A request to read information from the AIoT Device, including application defined parameters.

-	Write Response: Response to the Write Command.

-	Disable Request: A request to disable the device

-	Disable Response: Response to the Disable Request.

-	Security Commands/Responses: Depending on SA3, there may be a need to additional security commands/responses. Additional parameters required can be combined into the above NAS messages, so no further dedicated messages are required.

Additionally, as seen from the list of potential commands and responses they are expected to only be sent in one direction (e.g. download to the AIoT Device for commands, and uplink from the AIoT Device for responses). This separation can be used to reduce the size of the NAS Header (assuming no 8-bit alignment), as UL and DL NAS commands/responses don’t need to share the same number space.

# 3. Conclusion and proposal(s)

NAS is an essential part for the existing 3GPP systems, including EPS and 5GS where it brings many benefits.

AIoT, while needing to be simple and compact, can benefit from NAS as well. It aids both deployment topologies and allows for expandability in devices themselves in the future.

The processing/interpretation of NAS messages in an AIoT Device to determine the action being requested is simple and, as can be seen from the NAS Header discussion, is not and does not need to be complex. Also, this strategy is proven to work as it is adopted by RFID.

There is a small overhead in NAS, but this can be a single octet or less, which is an acceptable trade-off for the enhancements it brings.

NAS forms an essential part of the 3GPP system allowing for future growth, enhancements and updates, while minimising the impacts of those to the overall system, including RAN.

It is important to provide a strong basis to build from for this and future AIoT releases.