3GPP TSG-WG SA2 Meeting #165	S2-2410425

Hyderabad, IN, 14th Oct – 18th Oct, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the comparison of centralized and distributed solution for energy evaluation

Document for:	Discussion

Agenda Item:	19.4

Work Item / Release:	EnergySys / Rel-19

Abstract: This paper discusses the evaluating functionalities should be implemented within EECF solely, or the SMF can also assume the evaluating for PDU Session or QoS Flow granularities.

# 1. Introduction

The conclusion of KI#1 was made in SA2#164 meeting and the working agreement was confirmed in the TSG SA #105 meeting. It was documented in the approved WID (SP-241388) that:

"1)	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator’s policy:", and

"NOTE 1:	The new network functionality can be deployed standalone or co-located with other NFs under SA2 responsibility."

Therefore, it is assumed in the upcoming SA2 meeting, the role of new network functionalities should be determined.

In the residual clauses of this document, the analyses are provided and we suggest to let SMF assume the responsibility of evaluating the Energy consumption (EnC) of the specific PDU Session or the specific QoS Flow.

# 2. Discussion

## 2.1	Background

The study of TR 23.700-66 is trying to answer the following questions:

-	on how to collect, evaluate and expose the energy related information, especially for the energy consumption of the specific UE, PDU Session or QoS Flow;

-	on how to support subscription and policy control for energy saving and energy efficiency;

-	on how to enhance the 5GS for network energy saving and efficiency.

In which the key aspect during the discussion is the collection, calculation and exposure for energy related information. According to the conclusion of KI#1, A new network functionality is defined to collect, calculate the energy related information and expose to the authorized consumer subject to operator’s policy, and it is under SA2 responsibility to determine whether such new network functionality can be new or co-located with other NF.

## 2.2	Options under discussion

### 2.2.1	Option#1: Hierarchical realization of the new network functionality based on a new NF (EECF) and enhancements to the SMF

This option assumes that the SMF is enhanced to handle the finer granularity, i.e., PDU Session and QoS Flow, for the calculation of the energy consumption. SA2 new Network Function EECF is in charge of UE granularities and to interact with OAM. All necessary information comes from UPF and OAM.

Figure 2.2.1-1: EECF aggregates the energy consumption result

Step 1: the AF and/or the 5GC NF may request to EECF to provide the Energy related information (e.g. EnCUE) and SMF the EnC and DV of UPF and RAN which are in the service area managed by SMF (EnCUPF, EnCRAN, Data VolumeUPF, Data VolumeRAN).

Step 2:The EECF EECF may ask the information based on the request received from SMF, 5GC NG and AF and based on its own configuration . The request may be per events and/or request a periodically update of the information for the measurement period T. The measurement period may be determined taken into account the evaluation periods requested by SMF,5GC NF, AF and internal requirements.The EECF may gets the EnC and DV of UPF and RAN from OAM (possibly all UPFs and RANs), and forward the EnC and DV of related UPFs and RANs (which are possibly all within the service area of SMF) to SMF in the periodical manner.

The interaction between OAM and EECF could be for all UPFs and RAN nodes in the whole network or for the UPFs and RANs nodes related to the request received from several SMFs, i.e., the OAM provides the EnC and DV info for every active UPF and gNB once per time slot, and the EECF can then distribute/replicate the info to the various SMFs according to their service areas.

Step 3. SMF gets the energy consumption (EnC) and data volume (DV) of the certain RAN node(s) and UPF(s) of the target (i.e., the specific PDU Session, or specific QoS Flow).

The triggers for the SMF sending the request can be e.g., from EECF asking for the target PDU Session(s) for calculating the UE energy consumption.

The UPF(s) and RAN nodes sent by the EECF for the EnC/DV information reporting from EECF are all UPFs and RAN nodes in the SMF service area, to potentially save the interactions.

SMF collects DV of the PDU Session from UPF (Data VolumeUE,PDU Session, UPF)

SMF collects the data volume of the PDU Session or QoS flow from UPF from the N4 message. This functionality is already supported by SMF and UPF for example for providing the information to CHF in case of per PDU session or per QoS flow charging.

Step 4: SMF calculates the EnC of the PDU Session

SMF estimates the EnC of the target.

Where Target can be specific PDU Session or specific QoS Flow of a certain UE.

Step 5: SMF send the calculated  EnC of the PDU Session to EECF

Step 6: EECF calculates the EnC of target

EECF can estimates the EnC of the UE by aggregating all the EnC of the PDU Session of the UE (or expose EnC of PDU Session or QoS Flow received from SMF).

Step 7: EECF exposure to the 5GC NF or external AF.

### 2.2.2	Option#2: Centralized realization of the new network functionality based on a new NF (EECF)

This option assumes the EECF acts as a centralized function that calculates all different granularities. Similar as option#1, the calculation is executed at SA2-defined function, and the EnC/DV information are all from OAM/UPF.

Figure 2.2.2-1: EECF calculates the energy consumption result

Step 1: the AF and/or the 5GC NF may request to EECF to provide the Energy related information and SMF the EnC and DV of UPF and RAN which are in the service area managed by SMF, for example the AF may ask to retrieve the Energy Consumption for a Group of UEs or the SMF to retrieve the DV and EC per NF information. These steps are optional. The results of the estimation in step 6 are provided

These steps are similar as step 2 of option#1.

Step 2: EECF gets the EnC and DV of UPF and RAN from OAM (EnCUPF, EnCRAN, Data VolumeUPF, Data VolumeRAN). The EECF may ask the information based on the request received from SMF, 5GC NG and AF and based on its own configuration. The request may be per events and/or request a periodically update of the information for the measurement period T. The measurement period may be determined taken into account the evaluation periods requested by SMF,5GC NF, AF and internal requirements.

This stepis similar as step 2 of option#1.

Step 3: EECF collects DV of the PDU Session of UPFs served by SMF from SMF (RAN ID,UPF ID, Data VolumeUE,PDU Session, UPF, Data VolumeUE,PDU Session,QoS Flow, UPF,)

The EECF requests to SMF to provide the DV for specific PDU Session(s) and/or QoS Flow(s) per a specific UE. The SMF determines the serving UPFs (and I-UPF if presents), collects the data volume of the PDU Session(s)/QoS flow(s) from UPF from the N4 message and forwards to EECF.

Step 4: EECF calculates the EC of the PDU Session

Similar as option #1.

Step 5: EECF exposure of the estimation results to the 5GC NF or external AF.

## 2.3	Analyses

### 2.3.1	Requirements and general considerations

The use cases for activation of EnC information collection can be differentiated into a permanent and a situational category. For the permanent category it should be primarily the end user’s subscription/contract which requires EnC for non-operator services (e.g., green subscription) together with one or more actions (e.g., forwarding to Charging System for documentation in monthly bill, enforcement of energy credit, enforcement of instantaneous energy consumption). In the situational category, there are several different use cases possible. The AF (but also CN internal NF like the NWDAF) could request for exposure of EnC information in different granularities (e.g., for a UE, group of UEs, an application). The operator can also have EnC related decision logic which are focusing on the aggregated EnC in its network, with the typical scope of an area or a certain application, for energy saving actions. So, there is a variety of use cases and possibilities for using energy consumption information in the network and even though some of them may not be achievable in Rel-19, they should still be considered for the design of the architecture.

Figure 2.3.1-1: Use cases for activation of EC information collection

A lot of the use cases require the collection of the data volume that is transferred for a UE, a UE’s PDU Session, for a specific QoS Flow in a UE’s PDU Session or even for a specific Service Data Flow/application flow in a QoS Flow. In all these cases, a tight integration in the PCC rule framework and the corresponding SMF and UPF functionality for traffic detection and counting is required in order to perform this data volume counting properly and efficiently, especially since we can assume that energy consumption information is of interest for large amount of the UEs/subscribers and often on a permanent basis.

In addition, it is essential for the calculation of the energy consumption information to keep track of the specific NFs that are serving the UE during a measurement cycle. Especially for the serving RAN node, it is rather likely that changes occur, e.g. a normal handover due to UE mobility or a handover when the UE is at the cell edge, and the SMF (together with the UPF) need to keep track of the data volume that has been transferred by each of the involved RAN nodes (so that their likely difference in energy consumption can be reflected properly).

There are also a couple of more complicated user plane scenarios/configurations (e.g. Edge Computing/DNAI, I-SMF, I-UPF, roaming, offload, ATSSS, redundant user plane, N6LAN traffic steering) for which the details require further discussion, including the question of whether to perform the data volume counting or not. At this point in time it is however already clear that it would be beneficial to leave and handle those details at the SMF instead of indicating such details to the EECF.

Considering the above aspects together, it is very unlikely that a centralized calculation of the energy consumption information at the EECF would be scalable. Instead of transferring a lot of detailed information about the counted data volume and the situation in the user plane (like which RAN node and which UPF has handled which amount of data volume), it should be much easier to perform the calculation of the energy consumption for the smallest granularities (i.e. PDU Session, QoS Flow and specific Service Data Flow/application flow) in the SMF.

And while the usage of energy consumption information for charging purposes may not be achievable in Rel-19, it can be expected that this will come in Rel-20 and the SMF would then need this information anyway in order to forward it to the CHF or to apply it for energy credit enforcement or maximum energy limit enforcement.

So, to summarize, there are several good reasons for involving the SMF in the energy consumption information calculation process and to give the SMF the responsibility for the calculation of the smallest granularities (i.e. PDU Session, QoS Flow and specific Service Data Flow/application flow). Also, from a future proofness and expandability perspective, the responsibility of the SMF will likely increase in the future and it would be short-sighted to ignore this in the current architecture discussion.

### 2.3.2	Comparison of Hierarchical and distributed solution for energy evaluation

In this clause, we analyse the new functionalities introduced by SA2 study, and also taken into account the future proofness. Table 2.3.2-1 demonstrates the suggested implementation and the associating rationales. Note that this table only touches the EnC aspect of energy related information, while on the renewable energy it should be similar.

According to the key issues, the new functionalities are assumed to support the followings:

1.	Collect the energy related information for a specific UE, PDU Session or QoS Flow;

1.1:	Obtain the of EnC and Data volume for the NFs (including 5GC NF and RAN nodes);

1.2:	Calculate the EnC across PDU Sessions, QoS Flows or services;

1.3:	Traffic volume measurement for PDU Session, QoS Flow or service

2.	Decision making for EnC collection and energy saving actions, potentially based on energy related information;

3.	Storage of specific EnC info, e.g., EnC, Data Volume of the NFs;

4.	Support the subscription based EnC for a UE to assist the network to perform energy saving strategies e.g., energy credit control and instantaneous EnC control;

Note that there is also the on-going discussion for energy in SA1 (i.e., FS_EnergyServ_Ph2), therefore the followings are also to be considered as the potential future extensions, as per the SID description in SP-240494.

5.	Charging aspects;

6.	Potential dynamic adjustments of the delivered communication service;

Table 2.3.2-1: Support of the new functionalities in 5GC.

| Scenario | Suggested implementation | Rationale |
| --- | --- | --- |
| Obtain the of EnC and Data volume for the NFs (including 5GC NF and RAN nodes) | Suggest to use new 5GC NF, i.e., EECF to interact and obtain the EnC and Data Volume. | Utilizing the centralized NF to obtain this information is better by considering the signalling efficiency (e.g., the information is re-usable for different requests). |
| Calculate the EnC across PDU Sessions, QoS Flows or services | Suggest to divide the responsibilities: 1) EECF evaluates the UE granularity of EnC, by aggregating the EnC of PDU Session from SMF; 2) SMF evaluates the PDU Session or QoS Flow granularity of EnC. | It’s better to distribute the calculation, since the effort of measurement is not neglectable, especially when a UE may in average have 2-3 handovers during a measurement cycle. Having this calculated at EECF may have performance issue. |
| Traffic volume measurement for PDU Session, QoS Flow or service | Suggest to use SMF. | Already supported in existing N4 interface. |
| Decision making for EnC collection and energy saving actions, and Support the subscription based EnC for a UE to assist the network to perform energy saving strategies | On Energy saving actions: 1) EECF determine the energy saving action (or energy saving strategies); 2) the energy saving action will be extra input for original NFs (e.g., PCF). | Avoid racing condition for the energy saving actions. And also concluded in 8.2 of TR 23.700-66. |
| Storage of specific EnC info (subscription on energy saving for a UE) | Use UDM/UDR. | Already concluded in 8.2 of TR 23.700-66. |
| Future proof: 1) Charging aspect; and 2) Potential dynamic adjustments of the delivered communication service | 1) SMF interacts with CHF for energy related charging; 2) Potential dynamic adjustments of the delivered communication service: similar as Energy saving actions, should be a case-by-case discussion. | The interaction between SMF and CHF are already existing interface/function for the charging. Note that such interaction also justifies the SMF-calculation aspects. |

### 2.3.3	On the NWDAF-based option

There is also one other possibility to combine the energy related new functionality to the existing NWDAF, and one may think that the data collection functionality of NWDAF can be re-used.

Technically speaking, the NWDAF is designed mainly for the analytic purpose instead of the energy related functionalities. And as mentioned in clause 2.3.1, using NWDAF cannot guarantee the future proofness and expandability perspective. Moreover, energy related execution is different from the data collection approach: for analytics on may want to have as much data as possible, but for EnC the purpose is to collect as few data as necessary for the result (in order not to waste more energy for data collection than what can be potentially saved by any subsequent action).

As such, NWDAF is not as good as the previous option in energy related processing.

### 2.3.4	Regarding the time synchronization issue

Time synchronization is the issue that the information from OAM on the DV and EnC of RAN and NF is for a time interval, while the requested time span of the EnC may not be perfectly aligned with the previous collection interval. Furthermore, the network may handover the UE to a new RAN node that worsen the issue.

It can be assumed that the EnC and DV of the RAN/NF is evenly distributed within the time interval. SMF can base on this information to evaluate the total EnC and DV of the RAN/NF for the requested time span. SMF can also evaluate the source and target RAN nodes related information in case of handover.

### 2.3.5	Summary of the merits of option#1

In summary, the advantages of option#1 are as follows:

-	Achieves scalability for the smaller granularities, i.e., per QoS Flow / per application flow, especially when the energy consumption information collection is active for many UEs;

-	SMF can easily handle/hide all complex user plane scenarios: Edge Computing/DNAI, I-SMF, I-UPF, roaming, offload, ATSSS, redundant user plane, N6LAN traffic steering;

-	Enables re-use of calculation functionality (which is necessary for exposure) when adding further energy related functionality to SMF, like adding energy related info to Charging System CDR, energy credit, (short term) energy limit;

-	SMF/UPF enables reaction/behaviour depending on situation at RAN in the current measurement cycle (based on indication of cell edge/high RB ratio in UL GTP-U), e.g. different counting of traffic volume, enforcement actions based on conditions, event reporting/monitoring

# 3. Conclusion and proposal(s)

Based on the analyses in clause 2, it can be seen that the distributed alternative (i.e., SMF calculates/evaluates the EnC or other energy related information) shows more flexibility and future-proofness, and therefore it is suggested to go with this option in the normative work.