3GPP TSG SA WG2 Meeting #165	S2-2410444

Hyderabad, India, 2024‑10-14 - 2024‑10-18	(revision of S2-220xxxx)

Source:	Apple

Title:	UE Data Collection in 5GC for RAN AIML operation

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: This discussion paper analyses the user data privacy and user consent aspects for UE data collection in 5GC.

# 1. Discussion

The LS S2-2409600 from RAN asks SA2 to provide views on Solution 1b, Solution 2 and Solution 3.

In the LS, RAN has highlighted their agreements:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

This discussion paper analyses the definition of standardized data and system architecture aspects from user consent and privacy considerations and defines architecture principles.

## 1.1 Definition of standardized data for UE side model training

The primary requirement for defining “full visibility for standardized data” is to define what constitutes standardized data.

The data relevant for a UE side model training for RAN AIML operation vary depending on UE implementation and the machine learning models. The data varies according to the machine learning operation also but broadly they may be of two types. The primary type of data are the measurement results UE obtains as part of executing some measurements on radio interface. This measurement results could be from measurement configured by the network or based on internal design.

The other type of data may be related to UE internal states and parameters. This type of data may not be relevant for a third party other than the UE generating the data, and the entity that manages the training. There is no need to have a standardized definition or visibility for this type of data.

Standardized solution should address the visibility of measurement results collected by the UE according to a measurement configuration provided by the network.

Principle 1: The standardized data refers to the results of measurements performed by the UE in response to a measurement configuration provided by the network.

This measurement configuration may need to be influenced by the entity (e.g., in application layer) responsible for UE side model training. At least for option 1b and option 2, the 5GC need to provide interface for an external AF to provide requirements for measurement configuration per UE/ per group of UEs. For solution 3, it is up to SA5 to decide how external entities can influence measurement configurations.

Principle 2: 5GC provides an interface to AF to provide requirements on measurement configurations for AIML operations.

## 1.2 User privacy and user consent aspects for exposure of collected data

As per Option 2 (to some extent, also for Option 1b), an entity in the 5GC is responsible for collection of data and exposure of data outside of MNO domain. Contrary to Option 1a, MNO is now responsible of ensuring user privacy and user consent aspects for this data exposure.

In option 1a, the entity (OTT Server) that receives the data from the UE is known to the UE/user. It can be assumed that user is aware of the data collection for model training by willingly installing an application client and/or acceptance of user agreement. Also, by design, the collected data is only provided to the entity privileged to receive this information.

In option 1b, the data collection makes use of server inside MNO domain before exposing to OTT server outside the MNO domain. The application layer data collection framework defines the intermediary Application Function to collect data from the UE. Data Collection AF (DCAF) could also be deployed by the MNO inside trusted domain. Such a DCAF could be tasked with aggregating all external AF requirements and provisioning UE with measurement and reporting configuration. DCAF is also responsible for exposure of collected data to other third party AF (trusted or untrusted).

In option 2, the core network NF responsible for data collection could be different depending on the machine learning operation. LMF could be one such NF for AI/ML based positioning. Similarly, NWDAF could be another NF responsible for configuring and collecting measurements from the UE. It can also be a new NF with the sole responsibility of configuring and collecting measurements.

In option 1b and option 2, multiple AFs may request 5GC to configure and collect data from the UE.

For this analysis, we treat Option 1b and Option 2 as similar and derive the following observations on the functionality of the core network NF responsible for collecting data from UEs and exposing them to external AFs. We see the need to agree on the following principles to realize an architecture that ensures data integrity and user privacy.

Observation 1: 5GC NF is responsible for authorizing Measurement configuration request from AF(s) and should clearly identify the purpose of measurement and the entity (e.g., AF) that is requesting the measurement to the UE.

Principle 3: The purpose of measurement configuration (e.g., UE side model training) and the identity of requesting AF should be informed to the UE.

Observation 2: The 5GC NF is responsible for checking whether user has provided consent for data exposure to the requesting AF before configuring measurements

Principle 4: 5GC NF responsible for data collection enforces user consent check for collection and exposure of data outside of MNO domain.

In option 3, the entity responsible for data collection and exposure is in the OAM domain. However, one could extend the similar design in that case also.

The data collection could apply many privacy preserving techniques before exposure outside the MNO domain. Some AFs may only require anonymised data (measurement results from UEs of a same vendor from a location), some require only aggregated data, or some may only need temporary proxied identifiers to be reported. The privacy protections methods need to be applied in the reported data before exposure to the AF.

Observation 3: 5GC NF is responsible for obtaining privacy requirements for data collection from the AF and enforcing them prior to exposure.

Principle 5: 5GC NF responsible for data collection applies privacy protecting methods as required before data exposure.

The following figure shows the architectural diagram for data collection using a 5GC NF

Figure 1-1 System architecture for UE data collection using 5GC NF

# 2. Conclusions

It is proposed to agree the following architecture principles for user consent and privacy management for data collection and exposure through 5GC using either option 1b or option 2.

Architecture Principle 1: The standardized data refers to the results of measurements performed by the UE in response to a measurement configuration provided by the network.

Architecture Principle 2: 5GC provides an interface to AFs to provide requirements on measurement configurations for AIML operations.

Architecture Principle 3: The purpose of measurement configuration (e.g., UE side model training) and the identity of the requesting AF should be informed to the UE.

Architecture Principle 4: 5GC NF (responsible for data collection) enforces user consent check for collection and exposure of data outside of MNO domain.

Architecture Principle 5: 5GC NF (responsible for data collection) applies privacy protecting methods as required before data exposure.

# 3. References

1. S2-2409600 - LS from TSG RAN: LS on AIML data collection