3GPP TSG-WG SA2 Meeting #165	S2-2410455

# Hyderabad, India, October 14 - 18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1498 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCSF instructing the IMS AS to terminate the session at the IMS AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In "Originating Standalone Bootstrap DC Setup" in clause AC.10.2.1, the DCSF needs to instruct the IMS AS to terminate the IMS session establishment at the IMS AS, i.e. IMS AS acts as SIP UAS and generates the final response to the INVITE. It is not specified how the DCSF instructs the IMS AS to perform this, instead of R18 behaviour where the DCSF instructs the IMS AS to forward the received INVITE towards the terminating network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add an optional parameter "Session instruction" to the Nimsas_MediaControl_MediaInstruction service operation. The default action in IMS AS is "continue", this is used when the parameter is omitted in the request. The value "terminate" instructs the IMS AS to act as UAS for the IMS session and to generate the final response to the INVITE request. The DCSF sets the value to "terminate" in step 6 in figure AC.10.2.1-1. In step 11, the statement that the IMS AS acts based on the received Request URI is incorrect and is removed. Other changes: - "using PSI" is removed from the title, this gives an impression that there is another procedure where PSI is not used. - Subscription for standalone data channels needs to be verified by IMS AS in step 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCSF cannot instruct the IMS AS to terminate the SIP INVITE at the IMS AS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.3.2, AC.10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification: Description of additional media specification information needed for data channel media stream from application layer, which includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE:	DC Media Specification and AR Media Specification are optional depending on the scenario and media type.

Inputs, Conditional: None.

Inputs, Optional: Session Instruction.

Session Instruction indicates to the IMS AS how to continue the IMS session establishment which the event relates to. The parameter is applicable for the Event ID "SessionEstablishmentRequestEvent".  The possible values are:

- 	continue (default): instructs the IMS AS to continue the IMS session establishment towards the called party.

- 	terminate: instructs the IMS AS to act as UAS for the IMS session and to generate the response(s) to the INVITE request.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new media) and includes the media type according to SDP. Each entry may optionally contain DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media.

*************************** Next Change ****************************

### AC.10.2.1	Originating Standalone Bootstrap DC Setup

The standalone IMS data channel session setup with only bootstrap DC is triggered by a UE or the user of a UE for downloading application list and/or applications. After standalone bootstrap DC is established, the UE downloads the application list via the bootstrap DC, and may display the list of application information to the user for selection, downloading, and/or initiating standalone application DC. The UE terminates the standalone IMS data channel session when the session timer expired.

Figure AC.10.2.1-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific target URI, such as a specific PSI indicating standalone bootstrap DC. The initial SDP contains offers only for the bootstrap data channel establishment request with bootstrap DC stream ID(s).

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.1 apply with the following difference:

If the IMS AS determines that the UE has not subscribed for a standalone IMS data channel session establishes a standalone IMS data channel session, the IMS AS rejects the session establishment.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered a local bootstrap media, DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP, determines that UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which are used to receive UE request for application downloading from MF or MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set, Session Instruction) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF for originating side. The Session Instruction is set to terminate the IMS session at IMS AS. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	IMS AS sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The session with bootstrap data channel is established.

14.	The bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 sends application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), UE#1 sends SIP BYE to release the bootstrap DC.

*************************** End of Changes ****************************