3GPP TSG-WG SA2 Meeting #164	S2-2410505

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5797 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative Annex for description of usage of CAG ID for 5G Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the study phase of FS_5G_Femto, it is concluded that 5GS shall be enhanced to support deployment of 5G Femto. Therefore, this CR aims to add a new informative annex to describe how CAG work for Femto to show the difference with the PNI-NPN scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new annex to clarify the CAG provisioning of 5G Femto |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture the new objectives of 5G_Femto |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex X (informative):
CAG used in NR Femto

CAG concept is used in 5G Femto for access control. In order to ensure that the CAG ID is unique in the PLMN, the CAG owner or an authorized administrator provisions via AF a set of CAG ID defined in coordination of the PLMN’s of the UE at which are provisioned, for example the Operator may provide a list of  CAG IDs to be assigned toNR Femto to the owner of the NR Femto. How the list is provided to the CAG owener or to the authorized administrator is outside the scope of this specification.

When the AF performs the External Parameter Provisioning procedure to provision the CAG ID associated to the UE to the PLMN, the AF shall ensure that the CAG ID provising is valid for the PLMN, how this is performed is left to the AF implementation and outside the 3GPP scope.

The mechanism to trigger the External Parameter Provisioning is outside the scope of this specification, but in the following some example are described in order to clarify how the procedure specified may be used. The example are nt intended to limit scenairo not described in this Annex.

The AF may be triggered by the owner of the NR Femto by event not in the scope of this specification, for example an applicaton is running in the UE to enable the UE who is wishing to connects to a specific NR Femto to perform the request of authorisaton to the Owner of NR Femto. The Owner of NR Femto may provide the Femto access control information to the AF via the application. Alternative and addional procedures may be based on interaction via a phone call or web application, etc.

In case of AF deployed by 3rd party service provider, the AF may also store the relasionship between the CAG ID and the owner of 5G Femto, such as a Corporate, a commercial center, etc.. When the AF receives the request from the owner of 5G Femto, the AF can determine whether the CAG ID provided by the owner of 5G Femto is the corresponding CAG ID. If the CAG ID is the corresponding CAG ID of Femto owner, the AF triggers the External Parameter Provisioning procedure. Otherwise the AF rejects the request from the Femto owner.

In case of 5G Femto deployed by operator, the configuration of the binding of allowed CAG ID and the UE may be performed via OAM.

# * * * * End of changes * * * *