3GPP TSG-WG SA2 Meeting #165	S2-2410512

# Hyderabad, India, October 14 - 18, 2024	 	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0030 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to definition on Range and Direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ASN.1 data types on Relative2D-LocationWithUncertaintyEllipse and Relative3D-LocationWithUncertaintyEllipse have been defined in Clause 4.4.2 of TS 24.080. type LocationData “2dRelativeLocation” and “3dRelativeLocation” are defined in clause 6.1.6.2.54 and clause 6.1.6.2.55 of TS 29.572. rangeAndOrDirection, Relative2D-LocationWithUncertaintyEllipse and Relative3D-LocationWithUncertaintyEllipse are defined in clause 6.5 of TS 38.355. Several ENs are still left related to above definitions in clause 5.10, clause 5.11 and clause 5.12. Based on the above definitions, it is proposed to remove those ENs and add the references to the stage 3 definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the reference of TS 24.080 and TS 38.355. Remove the ENs under clause 5.10, clause 5.11 and clause 5.12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear on the alignments to corresponding stage 3 specifications in CT and RAN WGs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.10, 5.11, 5.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	GSM 01.04: "Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms".

[2]	GSM 04.07: "Digital cellular telecommunications system (Phase 2+); Mobile radio interface signalling layer 3 General aspects".

[3]	Military Standard WGS84 Metric MIL-STD-2401 (11 January 1994): "Military Standard Department of Defence World Geodetic System (WGS)".

[4]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[x]	3GPP TS 24.080: " Mobile radio interface layer 3; Supplementary services specification; Formats and coding".

[y]	3GPP TS 38.355: " NR; Sidelink Positioning Protocol (SLPP); Protocol Specification ".

# * * * * Next change * * * *

5.10	Range and Direction

The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d. The range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence.

Figure 3d: Description of a Range and Direction

# * * * * Next change * * * *

5.11	Relative 2D Location with uncertainty ellipse

The "relative 2D location with uncertainty ellipse" is characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. It describes formally a set of points which fall within or on the boundary of an ellipse cantered on a 2D point with semi-major axis of length r1 oriented at angle A (0 to 180o) measured clockwise from North and semi-minor axis of length r2. The confidence level with which the position of a target entity is included within this set of points is also included with this shape.

This shape is only applicable to 5GS.

Stage 3 definitions of Relative 2D Location with uncertainty ellipse are specified in Clause 4.4.2 of TS 24.080 [x], clause 6.1.6.2.54 of TS 29.572 [4] and clause 6.5 of TS 38.355 [y].

# * * * * Next change * * * *

5.12	Relative 3D Location with uncertainty ellipsoid

The "relative 3D location with uncertainty ellipsoid" is characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). It describes formally the set of points which fall within or on the surface of a general (three dimensional) ellipsoid centred on a 3D point whose semi-major, semi-minor and semi-mean axis are, respectively, r1, r2, r3 with r1  r2. The r3 axis is aligned vertically, while the r1 axis, which is the semi-major axis of the ellipse in a horizontal plane, is oriented at an angle A (0 to 180 degrees) measured clockwise from North. The confidence level with which the position of a target entity is included within the shape is also included.

This shape is only applicable to 5GS.

Stage 3 definitions of Relative 2D Location with uncertainty ellipse are specified in Clause 4.4.2 of TS 24.080 [x], clause 6.1.6.2.55 of TS 29.572 [4] and clause 6.5 of TS 38.355 [y].

# * * * * End of changes * * * *