3GPP TSG SA WG2 Meeting # 164	S2-2410517

# Hyderabad, India,  14 – 18 October, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | <> | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data collection for AI/ML positioning model training based on NG-RAN measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#1 conclusion states that MTLF or LMF may perform positioning model training and collect training data for this purpose. Considering the LMF has existing NRPPa interface with NG-RAN for measurement request/report, the LMF is the suitable node for executing data collection upon MTLF request or for its own model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new Clause 6.x for high-level procedure of training data collection for positioning model training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Training data collection for AI/ML positioning model training is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.18.1	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on UE measurements

The LMF needs to obtain input data for AI/ML based positioning model training and may also request the UE to provide the UE location. The AI/ML based positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 5.18.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train a AI/ML Model for UE Positioning. The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest").

Editor's note:	Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS.

3.	The LMF checks the user consent for data collection for model training.

Editor's note:	Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS.

Editor's note:	Further details on user consent for data collection for model training will be aligned with SA WG3.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the AI/ML based positioning model.

## 5.18.2	Data collection at LMF to train the AI/ML based positioning model to perform positioning based on NG-RAN measurements

Figure 5.18.2-1: Data collection at LMF to train the AI/ML based positioning model to perform positioning based on NG-RAN measurements

1a.	The positioning model training data collection at the LMF may be triggered by a training data collection request from the MTLF, for the purpose of the model training in the MTLF. The request shall indicate an Area of Interest, positioning methods or measurement inputs used by the target model. The MTLF may also provide a list of PRUs/UEs that may participate in training data collection.

1b.	The LMF may determine on its own to initiate training data collection, e.g. for the purpose of model training in the LMF. The LMF can derive the Area of Interest, positioning methods or measurement inputs based on target model information.

2.	If a list of PRUs/UEs is not provided in the training data collection request, the LMF may select a list of PRUs/UEs based on their availability in the target area and capabilities. The LMF may also select one or multiple NG-RAN nodes that can perform UL channel measurement.

3.	The LMF requests the NG-RAN node to perform measurements using NRPPa procedure. The LMF provides the list of PRUs/UEs and required measurement inputs.

4.	The NG-RAN node performs UL channel measurement. This may be repeated multiple times. The measurement result is reported to the LMF using NRPPa procedure.

5.	The LMF stores the NG-RAN’s measurement report and use it for its own model training, or sends it to the requesting MTLF.

* * * End of Changes * * *