3GPP TSG-WG SA2 Meeting #165	S2-2410528

# Hyderabad, India, Oct 14 – 18, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5113 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UPF event exposure during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a remaining editor’s note on whether or not the UPF notifies any remaining data to the consumer based on relocation reporting indication, The UPF behaviour on whether or not it reports any remaining data when the N4 session is released is unspecified in the Rel-18. Since the NF consumer anyways receives data from the UPF for subscribed events, UPF configuration should be an option to determine on whether or not to notify any remaining data to the consumer. Thus, relocation reporting indication should not be mandatory for the UPF to report any remaining data to the consumer upon UPF relocation and it should be still possible based on UPF implementation and operator configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified that the procedure in clause 4.15.4.5.6 applies only when ther is no SMF relocation. - Removed the editor’s notes - Replaced Steps 1-3 of clause 4.15.4.5.7 with a reference to clause 4.15.4.5.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remaining Editor’s note in the specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6, 4.15.4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation

This information flow is used in the scenario of UPF relocation, e.g. the source UPF is I-UPF which is relocated, or the source UPF is PSA UPF which is relocated in SSC mode 3 without SMF reallocation, or the source UPF is local PSA UPF which is relocated due to UE mobility. Case when SMF relocation occurs is not specified.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, e.g. discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated and the N4 session with the current UPF needs to be released.

5.	The SMF unsubscribes to the source UPF by invoking Nupf_EventExposure_Unsubscribe service operation. The SMF may include the relocation reporting indication from the UPF event consumer.

6.	The source UPF may optionally send the collected data to the UPF event consumer. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change; or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

7-8.	To continue the subscription of the UPF event consumer, after selection of target UPF and establishment of N4 session with it, the SMF re-subscribes to the target UPF based on the subscription from UPF event consumer in step 1. The target UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer when the subscribed event occurs.

# * * * * 2nd Change * * * *

##### 4.15.4.5.7	Information flow for subscription directly to UPF event exposure service during UPF relocation

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service during UPF relocation

1.	Steps 1-2 as described in Figure 4.15.4.5.5-1 are performed.

2.	If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE.  The consumer may include an indication to receive subscription termination notification.

3.	The UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

4.	When the N4 session corresponding to the UE is released, e.g. due to UPF relocation, the UPF determines that the subscription to the UPF event has been terminated.

5.	If step 4 took place the UPF sends the notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination notification to notify that the subscription to the UPF event has been terminated, and may include a cause code specifying the reason for subscription termination, e.g. N4 session release. The UPF may optionally report to the consumer any data that has been collected but not sent to the consumer yet.

# * * * * End of changes * * * *