3GPP TSG|WG-SA2 Meeting #165	S2-2410535

Hyderabad, India, October 14 – 18, 2024	(revision of S2-20xxxx)

Source:	Samsung

Title:	Discussion on DNN and S-NSSAI as data sub key to retrieve QoS policies for non-3GPP devices connecting behind UE or 5G-RG

Agenda Item:	19.8.2

Document for:	Information

# 1.	Background

For the KI#4, there was a proposal in the SA2#164 (endorsed tdoc S2-2409328) to consider DNN and S-NSSAI as data sub key for retrieving device identifier specific policies from UDR.

During the study it was discussed the content of these device identifier specific policies (i.e. device profiles) which will be provisioned by AF to UDR. However, these discussions were happening for KI#1, 2, 3 perspective and no more present in the scope of the WID, but the same concept was applicable for KI#4. One of the content was DNN and S-NSSAI but this should be subset of the subscription with which these devices will be connected to.

DNN is for data network identification and S-NSSAIs are for some specific services. Before implementing this feature, in the deployment already some set of DNNs and S-NSSAI would be present for the subscription.

# 2.	Discussion

Based on the description in background section, it is required to address whether a new set of DNNs and S-NSSAIs can be created to use as data sub key for retrieving device identifier specific policies from UDR.

As the DNN and S-NSSAI of the device profiles present in the UDR need to be subset of the subscription, there can not be any new DNN and S-NSSAI which is specific for non-3GPP devices and not present in the subscription of the UE/5G-RG behind which these devices are connected to.

Observation 1: New DNN and S-NSSAI can not be added to device profiles which are not present in the subscription.

The subscription can be updated by adding new set of DNNs and S-NSSAIs to accommodate or make it feasible for the AF to provision the device polices with newly added set of DNNs and S-NSSAIs. But as the DNNs are for data network identification and S-NSSAIs for some specific services, until unless some additional services needed from new data network, there are no use cases for adding new DNNs and S-NSSAIs.

Observation 2: There are no use cases to update the UE/5G-RG subscription with new DNNs and S-NSSAIs

For DNN and S-NSSAI as data sub key to work, there need to be a single PDU session for each non-3GPP devices in the system which is practically impossible and big restriction. For e.g. to get same IMS services , UE/5G-RG need to create multiple PDU session and as only 255 PDUs can be established per UE, if there are more services and devices are there in the system, these things will not work.

Observation 3: It is practically impossible for the system to have a single PDU session per non-3GPP devices, as it will lead to denial of same services for the devices.

# 3.	Proposal.

Proposal: By looking at the all three observations, it is proposed to not consider DNN and S-NSSAI as data sub key to retrieve device identifier specific polices from UDR.