3GPP TSG-WG2 Meeting #165	S2-2410551

10th – 18th October 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1263 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to General inference procedure for vertical federated learning to address ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TR conclusion, VFL inference was adopted in 23.288. In high level descriptions, it is clear that VFL server initiates VFL inference process using VFL model correlation ID, but there are several E.N.s relating o how to utilize VFL model correlation ID in the VFL inference procedure, which is crucial to make VFL inference procedure work properly. This CR addresses E.N’s as follows: - VFL server shall discover and select VFL clients based on VFL model correlation ID as VFL server initiates inference procedure using VFL model correlation ID. As VFL clients are SW based entities, there may be cases VFL clients are not available, in that case, VFL server may discover proper VFL clients and share local ML model reusing existing ML model provisioning procedure Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. Editor's note: Whether the VFL client needs to retrieval of stored local models is FFS. Editor's note: Whether the VFL model correlation is required in step 5 is FFS. - For VFL inference, samples to be used for VFL inference is missing in the service request information. (UE ID, target periods) Editor's note: Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients. - In basic principle, VFL clients communicate with VFL server. But, when AF is VFL server, VFL clients may communicate with other NF as it cannot communicate AF directly. But, this case is already noted in other E.N. relating to AF as VFL server. Therefore, the following E.N. does not need to be a separate E.N. Editor's note: Whether VFL client may also provide local intermediate inference results to other VFL client is FFS. - How VFL server compute VFL inference is implementation specific unless it requires communication with other entity. We do not think VFL server needs communication with other entity for VFL inference. Editor's note: Further details of the VFL server computiong the VFL inference is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several E.N.s are resolved to enable the inference procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL inference will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.2G.2	Procedures

#### 6.2G.2.1	General inference procedure for vertical federated learning

Figure 6.2G.2.1-1: General inference procedure for vertical federated learning

Editor's note	Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable.

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Inf ormation=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs, Analytics Reporting Parameters (including Anlytics target period, etc).

Editor's note:	Which service used between the consumer and the VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients.

When there is a VFL model correlation ID, which may be used for the requested VFL inference, VFL server discover VFL clients based on VFL model correlation ID. (i.e. VFL server discovers and selects VFL clients which has been involved VFL training which is represented by the VFL model correlation ID).

When there is not a VFL client available for VFL inference which has been involved for VFL training of the VFL model correlation ID, VFL server (i.e. AF) may send Nnef_VFLDiscovery Request to NEF to discover VFL clients for VFL inference by indicating Analytics ID, External UE IDs, filter information (e.g. Area of Interest, requested ML model), VFL client capability. In the discovery request, filter information (e.g., Area of Interest, requested ML model) is determined based on the stored ML model information of VFL model correlation ID. VFL server may provision the discovered VFL client with the trained local ML model of VFL model correlation ID using ML Model Provisioning procedure in subclause 6.2A.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

3.	If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and additional parameters such as requested UE IDs and Analytics target period.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and additional parameters such as requested UE IDs and Analytics target period. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

4.	Each VFL Client collects its local data associated to the requested UE IDs and Analytics target period by using the current mechanism if the VFL Client does not have local data available already.

5..	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

6.	Each VFL Client sends the intermediate local inference results to the VFL server.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

7.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

8.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

9.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * * End of Changes * * *