3GPP SA WG2 Meeting #165	S2-2410560

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	Discussions on 5G Femto terminology and common aspects with PNI-NPN

Document for:	Discussion

Agenda Item:	19.12.2

Work Item / Release:	5G_Femto / Rel-19

Abstract of the contribution: This paper proposes a discussion and way forward on the 5G Femto terminology used in various WGs and enabling features defined for 5G Femto to PNI-NPN.

# 1	Discussion

In S2-2409604/S3-243518, LS from SA3 on Clarification regarding definition of 5G NR femto ownership recommends using “Hosting Party” instead of “Femto owner” as follows:

As per TS 33.320, Hosting party: The party hosting the H(e)NB and having a contract with the PLMN operator.

SA3 considers that hosting party terminology should be inherited in 5G, and 5G NR Femto owner should be considered same as hosting party. Also, we recommend using hosting party as the terminology for normative work in 5G SA2 as well as SA3.

In TS 22.220, The party hosting the H(e)NB is defined as follows:

H(e)NB Hosting Party: A H(e)NB Hosting Party has a contractual relationship with the operator, related to the provision of access to the operator’s network via one or more H(e)NBs.

NOTE: 	A H(e)NB Hosting Party is likely to have the billing relationship with the operator. A H(e)NB Hosting Party will typically be the “lead” user in a household but could be e.g. the corporate IT manager in an enterprise context.

In TS 28.557, clause 4.5 defines CAG management aspects for a PNI-NPN scenario as follows:

-	Assignment and maintenance of CAG IDs.

-	Managing the actual list of UEs that are allowed on the CAG. The information contained on this list should be shared between the NPN-SP and the NPN-SC.

-	Access rights of individual CAG cells. The NPN-SC shall have the capability to configure access rights to CAG cells (e.g. allowed days / time slots for UEs provided to contractors of a company).

Where NPN-SP is defined in clause 4.2 of TS 28.557 as:

NPN Service Provider (NPN-SP): a Communication Service Provider (CSP) which provides communication services for non-public use, i.e. communication services offered over NPNs. An NPN-SP is the realization of the CSP role (see definition in TS 28.530 [2], clause 4.8) in NPN environments.

Observation 1: On the one hand, the definition used for CAG management aspects for a PNI-NPN scenario in 5G and for the H(e)NB Hosting Party in 4G has commonalities. On the other hand, as part of the conducted study on 5G Femto, CAG concept defined for PNI-NPN is re-used. In order to address SA3 recommendation, the term “CAG Hosting Party” can be defined similarly to “H(e)NB Hosting Party” and used in SA2 and SA3 specifications.

Proposal 1: Define “CAG Hosting Party” similarly to “H(e)NB Hosting Party” in TS 33.320 and TS 22.220, and use the term “CAG Hosting Party” instead of “5G Femto Owner” in the SA2 specifications.

As part of the normative work of WT#3 in 5G_Femto WID (SP-241327), it is enabled that an authorized owner or administrator can control/provision subscribers allowed to access 5G Femto cells. During the discussions in SA2#164, it was also proposed to enable the similar functionality for PNI-NPN scenarios.

Observation 2: Based on the definitions given in TS 28.557 on CAG management aspects, as noted above, NPN Service Provider has the capability and responsibility to manage the actual list of UEs that are allowed on the CAG. Therefore, the new information defined for 5G Femto CAG provisioning in Nnef_ParameterProvision Service can also be used by NPN Service Provider which can be considered as “CAG Hosting Party” based on the given Proposal 1.

Proposal 2: Enable the usage of the recently defined information on 5G Femto CAG provisioning as part of Nnef_ParameterProvision service for PNI-NPN scenarios.

On the terminology used in various WGs for Femto, there are differences. SA2 has been using the term 5G Femto or 5G NR Femto whereas RAN3 has used the term NR Femto.

Observation 3: In the past, the same term was used consistently in 3GPP specifications for Femto. In order to mitigate any confusion inside and outside 3GPP, we should use the same term in our specifications. The term “NR Femto” exists in both SA2 SID/WID and RAN3 WID and TR.

Proposal 3: The term “NR Femto” should be used consistently in all WGs and specifications.

# 2	Proposals

Proposal 1: Define “CAG Hosting Party” similarly to “H(e)NB Hosting Party” in TS 33.320 and TS 22.220, and use the term “CAG Hosting Party” instead of “5G Femto Owner” in the SA2 specifications.

Proposal 2: Enable the usage of the recently defined information on 5G Femto CAG provisioning as part of Nnef_ParameterProvision service for PNI-NPN scenarios.

Proposal 3: The term “NR Femto” should be used consistently in all WGs and specifications.