3GPP TSG SA WG2 Meeting #165	S2-2410577

# Hyderabad, India, 14 – 18 October, 2024                                          (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5813 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Session management for energy efficiency and energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It should be clarified how the energy related information is used in the 5G network to facilitate network operations for energy efficiency or energy saving. It is proposed in the CR the session management is enhanced to use the energy related information for optimization of UE data transmission in the UP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to clarify the session management which utilizes energy consumption information and the releated SM policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to use the collected energy information for optimizing user plane operations is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

## 5.6.X	Support for Energy Efficiency and Energy Saving

### 5.6.X.1	General

The amount of energy that are consumed to serve a UE in the network can be further considered in the session managent for the UE, based on the operator’s policy, network configuration, etc., to improve energy efficiency and energy saving in the 5G network.

The UDM stores the energy related configuration information for a UE in the subscription data of the UE and provides it to the SMF during the PDU session establishment.

In case of the dynamic policy control, the PCF may retrieve the energy related configuration information for the UE from the UDR and provides the SMF with energy related policy information as part of the SM policy to control the energy usage in the network during the policy association with the SMF. Based on the collected energy consumption information for the UE and the network energy state, the PCF may decide to provide the updated energy related policy information to the SMF to control the energy consumed to serve the UE in the network and improve the energy efficiency and energy saving in the network.

The SMF use the energy related configuration information from the UDM and the energy related policy information from the PCF to accept, modify and reject PDU sessions for the UE and to improve energy efficiency and energy saving in the network.

# * * * * End of changes * * * *