3GPP SA WG2 Meeting #165	S2-2410601

14 - 18 October, 2024, Hyderabad, India

Source:	Nokia

Title:	Discussions on options for I-SMF insertion for Local Offloading Management

Document for:	Discussion

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a discussion on the possible options and a way forward on I-SMF insertion for Local Offloading Management.

# 1	Discussion

In SA2#164 meeting, two options on I-SMF insertion for Local Offloading Management are proposed. These options can be summarized as follows:

- Option 1: AMF performs I-SMF insertion based on Local Offloading allowed indication as part of SMF Subscription data (S2-2407722). This option mimics HR-SBO procedures defined in clause 6.7.2 of TS 23.548.

- Option 2: SMF triggers I-SMF insertion based on Local Offloading allowed indication as part of SM Subscription data (S2-2408562). This option reuses the existing procedure on SMF triggered I-SMF selection and removal in clause 4.23.5.4 of TS 23.502.

As part TR 23.700-49 KI#1 definition and WT#1 in WID, the purpose is to trigger I-SMF insertion in order to enable I-SMF to select and configure EASDF.

In clause 5.34 of TS 23.501, I-SMF/V-SMF insertion/selection is described for the following cases:

“When the UE is outside of the SMF Service Area, or current SMF cannot serve the target DNAI for the traffic routing for local access to the DN, an I-SMF is inserted between the SMF and the AMF.”

“During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI.”

“According to the PCC rules related with AF influence traffic mechanism regarding DNAI(s), the SMF determines the target DNAI which is applicable to the current UE location and which can be based on the common DNAI (if applicable) as described in TS 23.548 [130]. If current (I-)SMF cannot serve the target DNAI or if the SMF can server the target DNAI and existing I-SMF is not needed, the SMF may send the target DNAI information to the AMF for triggering I-SMF (re)selection or removal, e.g. the AMF performs I-SMF (re)selection or removal based on the target DNAI and supported DNAI(s) of (I-)SMF.”

It is further noted that in clause 5.34.3 of TS 23.501:

“Compared to the SMF selection function defined in clause 6.3.2, the following parameters are not applicable for I-SMF/V-SMF selection:

-	Data Network Name (DNN).

-	Subscription information from UDM.

NOTE 1:	All SMF(s) and I-SMF are assumed to be able to control the UPF mapping between EPC bearers and 5GC QoS Flows.

If HR-SBO roaming is allowed for a PDU Session, the DNN is also considered for V-SMF selection.”

Observation 1: I-SMF insertion is triggered by AMF if the UE is outside of the SMF service area. I-SMF insertion is triggered by SMF if the target DNAI cannot be served by the anchor SMF. As part of SMF selection function defined in clause 6.3.2 of TS 23.501, DNN and Subscription information from UDM are not used by AMF/SCP for I-SMF/V-SMF insertion.

Observation 2: For HR-SBO case, namely V-SMF insertion, UE is outside of the H-SMF service area. Hence, AMF triggers V-SMF insertion. Therefore, during V-SMF selection, AMF should be aware of whether Session Breakout for HR Session in VPLMN is allowed per DNN, or per (S-NSSAI, subscribed DNN). That’s why, there is an exception that the SMF selection function can use DNN for V-SMF selection for only HR-SBO case. However, for the given KI#1 definition in TR 23.700-49 and WT#1 in WID, the I-SMF insertion for Local Offloading Management happens in the same PLMN.

Observation 3: The enhancement needed to trigger I-SMF insertion for Local Offloading Management is to enable I-SMF insertion while the UE is within SMF service area.

Proposal 1: For the purpose of enabling Local Offloading Management at I-SMF, define a new trigger for I-SMF insertion while the UE is within the SMF service area.

In clause 5 of TS 23.548, EASDF and Edge DNS Client (EDC) functionalities are described. As part of the given descriptions, EDC functionality is defined as:

“The Edge DNS Client (EDC) functionality is a 3GPP functionality in the UE that ensures that DNS requests from applications are sent to the DNS Server's (e.g. EASDF/DNS resolver) IP address received from the SMF in the ePCO.”

Observation 4: In order to ensure that the DNS request is sent to EASDF or DNS resolver that is provided by the operator, the UE should support EDC functionality. The UE capability to support EDC functionality is provided to the anchor SMF via PCO. In other words, the (anchor) SMF is the only 5GC entity that is aware of the UE capability and can be the decision point to use EASDF. It is not possible to have this capability information as part of the subscription data. Therefore, without knowing that UE has the capability to ensure that the DNS request is sent to the operator provided DNS Server (EASDF or DNS resolver):

(i) AMF cannot be an entity to decide I-SMF insertion for Local Offloading Management; and

(ii) AMF, on its own, cannot indicate I-SMF to select and configure an EASDF.

Otherwise, the I-SMF insertion will not serve the purpose of offloading the central SMF on EASDF selection/configuration.

Proposal 2: The entity that triggers I-SMF insertion for Local Offloading Management should be the anchor SMF.

Proposal 3: For the case I-SMF is inserted when the UE is outside of the SMF service area, the existing procedure defined for I-SMF insertion can be used. If the anchor SMF determines that (i) UE is allowed for Local Offloading Management (based on SM subscription data), (ii) UE capability supports EDC functionality and (iii) there is a need to offload Edge Computing related information management, the anchor SMF can trigger I-SMF insertion or I-SMF change if the selected I-SMF does not support Local Offloading Management.

Proposal 4: For I-SMF insertion when the UE is inside the SMF service area, either during PDU session establishment or after PDU session establishment, the anchor SMF can determine whether I-SMF insertion is needed, as described in Proposal 3.

# 2	Proposals

Proposal 1: For the purpose of enabling Local Offloading Management at I-SMF, define a new trigger for I-SMF insertion while the UE is within the SMF service area.

Proposal 2: The entity that triggers I-SMF insertion for Local Offloading Management should be the anchor SMF.

Proposal 3: For the case I-SMF is inserted when the UE is outside of the SMF service area, the existing procedure defined for I-SMF insertion can be used. If the anchor SMF determines that (i) UE is allowed for Local Offloading Management (based on SM subscription data), (ii) UE capability supports EDC functionality and (iii) there is a need to offload Edge Computing related information management, the anchor SMF can trigger I-SMF insertion or I-SMF change if the selected I-SMF does not support Local Offloading Management.

Proposal 4: For I-SMF insertion when the UE is inside the SMF service area, either during PDU session establishment or after PDU session establishment, the anchor SMF can determine whether I-SMF insertion is needed, as described in Proposal 3.