3GPP TSG SA WG2 Meeting #165	S2-2410618

# 18 – 24 October, 2024, Hyderabad, India                        (revision of S2-2410511)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1452 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authenitcation and Authorization of third party identities during IMS Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the TR 23.700-77 under KI#4, it was concluded how the IMS network is enhanced to support the usage of thrid party idnetities including authentication and authorization during IMS Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative description for authentication and authorization of third party identities and its usage in IMS Session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specififcation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

-

----

-	-

-

--

-	-

-

-	-

-

-

**** Next Change ****

## AA.2.1	HSS Services

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| imsUE Context | Registration | Request/Response | S-CSCF |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | Get | Request/Response | S-CSCF |
| imsParameter Provision | Update | Request/Response | AS |
|  | Create | Request/Response | AS |
|  | Delete | Request/Response | AS |

### AA.2.1.x	Nhss_ImsParameterProvision service

#### AA.2.1.x.1	Nhss_ImsParameterProvision_Update service operation

Service operation name: Nhss_ImsParameterProvision_Update

Description: This service operation is used by the AS to update the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

### AA.2.1.y	Nhss_ImsParameterProvision service

#### AA.2.1.y.1	Nhss_ImsParameterProvision_Create service operation

Service operation name: Nhss_ImsParameterProvision_Create

Description: This service operation is used by the AS to create the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

### AA.2.1.z	Nhss_ImsParameterProvision service

#### AA.2.1.z.1	Nhss_ImsParameterProvision_Delete service operation

Service operation name: Nhss_ImsParameterProvision_Delete

Description: This service operation is used by the AS to delete the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

Editor's note:	The definition of services using which AF/NEF will create to IMS AS for provisioning RCD information is FFS.

# AX.2	Procedure

Support for authentication and authorization of third party identities, additional functionalities in the S-CSCF, STI-AS, STI-VS, HSS as will be depicted below.

Figure AX.2-1: Authentication and Authorization of third party identity during an IMS session

The detailed procedure is described in Figure AX.2-1.

- The AF (e.g. third party like an Enterprise) will provision the third party identities/RCD information by invoking the service operation Nhss_ImsParameterProvision service for the target user (i.e for single IMPU or Wildcard IMPU).

Editor's note:	The provisioning of third party information (i.e. RCD information) by AF/NEF to HSS is FFS.

2.	UE1 is already registered.

3.	UE1 initiate an IMS session by sending INVITE to UE2.

4.   S-CSCF or IMS AS download the RCD information as part of the subscription information of the IMPU from HSS.

5.	S-CSCF or IMS AS checks the subscription information whether the user is authorized to use the third party identity service. If the parameter is enabled,

and if the RCD information is not present in subscription profile then S-CSCF or IMS AS contacts configured third party server to retrieve the third party identity/RCD information.

6.	 S-CSCF or IMS AS will invoke the Signing server (STI-AS) and sends the INVITE by adding the RCD information . This Signing server address, S-CSCF or IMS AS may get it from subscription information.

7.	Signing server after receiving the INVITE from S-CSCF or IMS AS, will  sign it and gives back the INVITE .

NOTE 1:	Adding the RCD information in specific headers by S-CSCF or IMS AS and singing the headers by STI-AS will be defined by CT1.

8.  S-CSCF forwards the INVITE to the terminating side.

9.	S-CSCF or IMS AS at terminating side will invoke the verification server based and forwards the INVITE request to STI-VS

10.	Verification server (STI-VS) will verify the signing and provides the validity status along with the INVITE to S-CSCF or IMS AS.

11.	S-CSCF at terminating side sends the INVITE towards UE2 based on the successful validation result. S-CSCF may reject the call for the unsuccessful result received from verification server.

12.	Successful call happens between UE1 & UE2.