3GPP TSG-WG SA2 Meeting #165	S2-2410622

# Hyderabad, India, 14th – 18th October, 2024		(revision of S2-2410313)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1427 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for UE-satellite-UE communicaitons |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Thales, CATT, CSCN, Honor, Tencent, China Telecom, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. - Understanding the architecture is crucial for identifying the affected interfaces and assessing the potential impacts or enhancements needed to support the introduced feature. In the context of UE-SAT-UE, several interfaces will involve satellite transport links (e.g feeder links, and/or inter-satellite links). Implementing this feature requires specific assumptions and requirements for these links. EN is resolved by replacing UE-satellite-UE with “IMS satellite media plane optimization” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an architecture to describe the interfaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-satellite-UE communication will not be activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.2.1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is supposed to be implemented together with CR 1428, after the clause of AX.2 Architecture and functional entities |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AX.2	Architecture and functional entities

AX.2.1	Architecture

AX.2.1.1		Support of IMS satellite media plane optimization with IMS-AGW deployed on satellite(s)

To support IMS satellite media plane optimization, as depicted in Figure AX.2.1.1-1, the IMS-AGW may be deployed on the satellite(s) that host the gNB and UPF (UL CL/BP and L-PSA) of the 5GC.

NOTE 1:	It is assumed that the satellite(s) can always connect to the ground with IP transport networks.

Figure AX.2.1.1-1: Reference architecture of IMS satellite media plane optimization

NOTE 2:		Iq interface is over satellite transport layer links (feeder link and optionally inter-satellite links), where the lower layer protocol is out of 3GPP scope;

NOTE 3:	For clarity, the connections within 5GC and IMS core are not fully depicted in the architecture diagrams. For more information on 5GC architectures refer to clause 4.2.3 in TS23.501 [93].

The above figure depicts a reference architecture for IMS satellite media plane optimization with following assumption:

-	For the IMS related PDU Session, the IP address allocated to the UE corresponds to a PSA UPF located on the ground. Thus, the IP address of the UE is not changed when serving satellite is changed.

-	UE(s) register onto IMS with this IP address and don't need to reregister when there is change of serving satellite.

-	The architecture deployment assumes, ISL(s) can be set-up within the same satellite constellation or across different constellations depending on satellite operator's deployments (SLA). The set of ISL(s) builds up an IP network which is out of 3GPP scope.

-	The routing between AGW-NTs across satellite via ISL is assumed to be based on IP routing and the AGW-NTs in satellite need to have non-conflicting IP address during resource allocation. It is up to the deployment to manage IP routing across ISL links and is out of 3GPP scope.

# * * * * End of changes * * * *