SA WG2 Meeting #165	S2-2410629

14 - 18 October, 2024, Hyderabad, India

# Title:	LS on Introduction of Extensions to IP Packet Filters for Differentiated QoS Handling for Multiplexed Media Flows

# Response to:	-

# Release:	Rel-19

# Work Item:	XRM_Ph2

Source:	SA WG2

To:	SA WG4

CC:	-

Contact person:	Paul Schliwa-Bertling

Paul.Schliwa-Bertling@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1	Overall description

SA2 has decided to extend IP Packet Filter Sets in Rel-19 in TS 23.501 to support the cases when multiple RTP streams, RTCP packets, and other protocols are multiplexed onto the same IP traffic flow (i.e., a single, non-wildcarded 5-tuple), as described by RFC 5761, RFC 5764, RFC 7983, RFC 8872, RFC 9143, and RFC 9443.

In those cases, the legacy (pre-Rel-19) IP Packet Filter Set information is not sufficient to identify separate RTP streams and RTCP packets, and to consistently identify PDU Sets related to those media flows.

RFC 9143 describes how to differentiate (S)RTP and (S)RTCP packets from other protocols in such multiplexed IP traffic flows.

The additional IP Packet Filter Set information fields defined by SA2 are:

-	RTP header Payload Type (PT)

-	Synchronization Source (SSRC)

SA2 would like to know if SA4 believes the chosen IP Packet Filter Set extension will suffice to handle needed differentiation for the intended, multiplexed IP traffic flows, as described above and if, additional information is needed, for example Media Identification (MID).

SA2 would also like to ask SA4 to guide SA2 on differentiated handling of RTCP packets that SA4 considers applicable.

# 2	Actions

To SA WG4

ACTION: 	SA2 ak SA4 to let SA2 know if the newly defined IP Packet Filter Set extensions will be sufficient to handle needed differentiation for the intended, multiplexed IP traffic flows.

# 3	Dates of next SA WG2 meetings

SA2#166:	18 - 22 November, 2024	Orlando, USA

SA2#166AH-e:	20 - 24 January, 2025	Electronic meeting

SA2#167:	17 - 21 February, 2025	Athens, Greece