3GPP TSG-SA2 Meeting #165	S2-2409928

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1235 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL procedures with AF as server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are open issues and missing documentation for VFL procedures with AF as server. See considerations in S2-2409927. |  |  |  |  |  |  |  |  |
| Summary of change: |  | VFL procedures for model training and inference with with AF as server are documented. The following related principles are applied: A trusted AF as VFL server directly accesses NRF, select NWDAFs as VFL clients, and communicates with them directly without an intermediate NEF NEF selects VFL clients based on NRF entries and requirements provided by AF as VFL server, provides temporary IDs for those VFL clients to the AF, and proxies subsequent communication based on those IDs The AF as VFL server may be triggered by internal logic or a request received from an 5GC NF to perform a VFL inference procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing documentation for VFL procedures with AF as server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 6.2H.2.3.1, new 6.2H.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 6.2H.2.3	Vertical Federated Learning (VFL) training procedures

##### 6.2H.2.3.1	Vertical Federated Learning (VFL) training procedures with AF as VFL server

Figure 6.2H.2.3.1-1: VFL model training with AF as VFL server

- The AF acting as VFL training server assigns a unique VFL Correlation ID.

Steps 2 to 7 are repeated until the MTLF acting as VFL training server determines that the training is complete.

2.	For each NWDAF VFL client, the AF acting as VFL training server sends a request to start a VFL training iteration. It includes the VFL Correlation ID. It also includes the intermediate results if step 2 is repeated. An untrusted AF identifies the VFL client using the temporary NWDAF ID assigned in the discovery procedure (see Clause 6.2H.2.1.1) and sends the request to the NEF. A trusted AF includes Analytics ID.

Editor's note:	Additional parameters, e.g. to indicate features and samples, are FFS. More clarifications about the intermediate results might be required; if loss information and gradient information is contained in intermediate results is FFS.

3a.	For an unstrusted AF: The NEF looks up information about VFL client stored in the discovery procedure (see Clause 6.2H.2.1.1). It forwards the request to the VFL client and includes the stored selection criteria. If the message in step 2a contains external identifiers, the NEF maps them to internal identifiers.

Editor's note:	It is ffs whether the NEF stores selection criteria received during discovery and includes them in subsequent messages or if the AF needs to provide related parameters.

4.	If intermediate results were received in step 2 or 3a, each NWDAF/MTLF acting as VFL client updates the own local VFL model based on the received intermediate results.
Each NWDAF/MTLF acting as VFL client collects input data, and calculates own intermediate results based on the input data and the own local VFL model.

Editor's note:	It is ffs whether the NWDAF as VFL client also uses the collected input data to update its model.

5a.	Each NWDAF/MTLF as VFL training client sends the intermediate results to the VFL server AF. For an untrusted AF, the NWDAF/MTLF sends the message via the NEF.

6a	For an unstrusted AF: For each NWDAF/MTLF as VFL training client, the NEF forwards the intermediate results to the AF

7.	The AF acting as VFL server updates the own model based on the received intermediate results. The AF may in addition collect own input data and also use those data to update the model. For each VFL client, it determines intermediate results based on the updated model and provides those intermediate results to the VFL clients in the request to start the next training iteration (step 2).

8.	Once the AF acting as VFL server determines that the trained model is sufficiently stable to terminate the training, it informs each VFL clients that the VFL training is completed. An untrusted AF sends the message via NEF.

Editor's note:	It is ffs whether intermedite results can be included in step 8..

9a.	For an unstrusted AF: The NEF looks up stored information about VFL client and selection criteria. It forwards the request to the VFL client and includes the stored selection criteria.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

Editor's note:	Terminology should be alligned related among VFL related descriptions, in particular related to intermediate results and VFL model correlation ID.

Editor's note:	The NEF procedures need to be confirmed.

10.	The AF acting as VFL server and the NWDAFs acting as VFL clients store their trained models along with the VFL correlation ID.

2nd change

#### 6.2H.2.4	Inference procedure for vertical federated learning

##### 6.2H.2.4.1	Inference procedure for vertical federated learning with AF as VFL server

Figure 6.2H.2.4.1-1: inference procedure for vertical federated learning with AF as VFL server.

1.	The inference procedure may be triggered by internal service logic of the AF acting as VFL server, or by a request or subscription for information from a 5GC entity, for instance for service experience information.

2.	For an unstrusted AF: The NEF forwards the request or subscription for information to the AF.

Editor's note:	Whether and how a 5GC NF can send a request to trigger the VFL server AF to perform inference is FFS. Whether the AF event exposure service can be reused is FFS

3.	AF acting as VFL server decides to initiate the VFL inference procedure with the VFL clients and for each client sends an inference request or subscription to the NEF including the Target of Analytics Reporting (e.g. UE ID(s)), possible analytics filters (see clause 6.1.3), and VFL correlation ID. An untrusted AF includes the temporary NWDAF ID obtained in the discovery procedure (see Clause 6.2H.2.1.1) and sends the request to the NEF. A trusted AF includes the Analytics ID and sends the request to the NWDAF.

Editor's note:	Additional parameters, e.g. to indicate features, are FFS.

4a.	For an unstrusted AF: The NEF looks up stored information about VFL client during in the discovery procedure (see Clause 6.2H.2.1.1). It forwards the request to the VFL client and includes the stored selection criteria. If the message in step 3a contains external identifiers, the NEF maps them to internal identifiers.

Editor's note:	It is ffs whether the NEF stores selection criteria received during discovery and includes them in subsequent messages or if the AF needs to provide related parameters.

5.	Each VFL Client collects its local data if the VFL Client does not have local data already available.

6.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results and generates intermediate local inference results using the VFL local model.

Editor's note:	Whether the VFL client needs to retrieve stored local models is FFS.

7.	Each VFL Client sends the intermediate local inference results to the NEF. The VFL Client may in addition include validity period, confidence, and Client Analytics Metadata Information (see clause 6.1.3).

8a.	For an unstrusted AF: For each VFL client, the NEF forwards the inference result to the VFL server.

If the VFL server used an inference subscription in step 3, step 5 to 8 may be repeated.

9.	The AF acting as VFL server may collect own local data and generate own intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results.

10.	If the inference was triggered by a request or subscription for information from a 5GC entity in step 1, the AF may report results derived by the inference process to the 5GC entity.

11a.	For an unstrusted AF: The NEF forwards the reported results to the 5GC

Editor's note:	The NEF procedures need to be confirmed.

End of changes